| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RS_MOB_MEAS-REP and MR Code-REP messages |  |
| Date Submitted | 2007-12-25 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan Gamini Senarath Nortel | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw Gamini Senarath [gamini@nortel.com] |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes combine on RS_MOB_MEAS-REP and MR Code-REP |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RS_MOB_MEAS-REP and MR Code-REP messages

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

This contribution proposes to replace the RS_MOB_MEASR-REP message by MR_Code-REP message since both messages are used to report measurement results from SSs and trigger unsolicited RNG-RSP in order to adjust the timing and the power level of SSs.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Spec changes

6.3.33 RS grouping

[Move the following subclause in line 40 of page 153 to subclause 6.3.33, and modified it as following.]

6.3.22.5.2 6.3.33.1 MS movement among access stations which share same BSID preamble/FCH/MAP

In this case, MS is not aware of the HO. Therefore, tThe stations that share the same BSID preamble/FCH/MAP shall perform measurement of MS signal quality to assist MS movement among stations.

The stations shall measure the signal quality (RSSI, CINR), Timing Adjustment (TA), power and Frequency Adjustment (FA) for each active MS served by these stations to support MS mobility among these stations. All RSs shall use RS_MOB_MEASR-REP MR_Code-REP to provide MR-BS with the selected report metrics (RSSI and/or CINR, TA, FA and power adjustment) for each active MS when needed.

Two modes of operation are described in 6.3.22.5.2.1 6.3.33.1.1 and 6.3.22.5.2.2 6.3.33.1.2. The mode of operation and related reporting parameters are configured in RS_Config-CMD in 6.3.2.3.70

An MR-BS may select a new designated RS based on the measurement results and use RNG-RSP to adjust the timing and the power level of the MS, in order to fulfill the handover procedure. An MR-BS may use the RS_Member_List_Update management message to notify the RSs of the changes regarding data forwarding status for the specified MSs.

6.3.22.5.2.1 6.3.33.1.1 Mode 1

For this mode of operation, only those RSs that are marked as designated RS shall automatically report the measurement results to MR-BS in an event-triggered or periodic way, for the basic CID of MSs provided in the MR_Member_List_Update.

For event-triggered reporting, the designated RS shall report its measurement results if at least one of the power, CINR, frequency or timing requirement for the specific MS is not satisfied. For periodic reporting, the designated RS shall send RS_MOB_MEASR-REP MR_Code-REP every REP_INT which is specified in RS_Config-CMD message and the MR-BS shall periodically allocate uplink resource for the designated RS to report the latest measurement result for each active MS.

In Mode 1, non-designated RSs shall report their measurement results only if MOB_RSSCN-REQ message is received. The MR-BS shall send MOB_RSSCN-REQ message to request all or part of RSs in the same RS group to report their measurement results for a specific MS. The MR-BS shall allocate uplink resource for the selected non-designated RSs to send their RS_MOB_MEASR-REPs MR_Code-REPs at the frame specified in MOB_RSSCN-REQ.

6.3.22.5.2.2 6.3.33.1.2 Mode 2

For this mode of operation an RS may report the measurement results using the RS_MOB_MEASR-REP MR_Code-REP to MR-BS in an event-triggered way as indicated below.

If the MR-BS provides the thresholds values for triggering, the RS shall send the measurement report to MR-BS in the following instances:

- When the measured RSSI/CINR crosses above RSSI/CINR_T_ADD[i] (i=0,...,max);

- When the measured RSSI/CINR crosses below the RSSI/CINR_T_DEL[i] (i=0,...,max) ;

- When the current measured TA exceeds TA_DIFF and the RS has the CID of the MS included in the RS_Member_List_Update message. If RS_Member_List_Update message is not provided, then an RS may send the measurement report whenever the current measured TA exceeds TA_DIFF.

If the MR-BS does not provide the thresholds values for adding/removing an MS, the RS uses its own threshold values to decide when to report the measurement to MR-BS.

[Deleted the following subclause in line 3 of page 59 as indicated:]

6.3.2.3.79 RS_MOB_MEAS-REP message

6.3.2.3.67 MR Code-REP message

[Modified the following text in line 3 of page 45 as indicated:]

This message is used by a transparent RS or a nontransparent RS with shared BSID to notify report the MR-BS that it has successfully the received CDMA ranging codes or measurement results from an SSs. This message shall be transmitted on the RS’s basic CID.

Table 183c—MR Code report (MR_Code-REP) message format

| Syntax | Size | Notes |
| --- | --- | --- |
| MR_Code-REP_Message_Format() { | - | - |
| Management Message Type = 72 | 8 bits | - |
| Report Type | 1 bit | 0 = report CDMA code, 1 = report UL traffic and/or Mode 1and Mode 2 handover measurement |
| If (Report Type == 0) | - | - |
| Size of Remaining CDMA codes | 7 bits | Amount of uplink bandwidth in bytes for CDMA codes remaining at the RS ready to be forwarded |
| Frame Number Index | 8 bits | LSBs of relevant frame number |
| } else { | - | - |
| UL Measurement type | 1 bit | Shall be set to 1 if UL measurement is for Mode 1 or Mode 2 handover measurements else set to zero. |
| Report CINR | 1 bit | Report CINR (0 = not report, 1 = report) |
| Report RSSI | 1 bit | Report RSSI (0 = not report, 1 = report) |
| reserved | 4 bit | Shall be zero |
| } | - | - |
| while(data remain) { | - | - |
| If (Report Type == 0) | - | - |
| Ranging Code | 8 bits | Indicates the CDMA Code sent by the SS. |
| Ranging Symbol | 8 bits | Indicates the OFDMA symbol used by the SS. |
| Ranging subchannel | 7 bits | Identifies the Ranging subchannel used by the SS. |
| Channel Measurement | 6 bits | The mean CINR as measured on CDMA ranging code. |
| } else { | - | - |
| MS basic CID | 16 bits | MS basic CID |
| If(Report CINR == 1) { | - | - |
| SS CINR mean | 8 bits | The mean CINR measurement. |
| } | - | - |
| If(Report RSSI == 1) { | - | - |
| SS RSSI mean | 8 bits | The mean RSSI measurement. |
| } | - | - |
| } | - | - |
| Include_TA | 1 bit | Include timing adjust (0 = absent, 1 = present) |
| Include_PLA | 1 bit | Include power level adjust (0 = absent, 1 = present) |
| Include_OFA | 1 bit | Include offset frequency adjust (0 = absent, 1 = present) |
| If(Include_TA == 1) { | - | - |
| Timing Adjust | 32 bits | Tx timing offset adjustment (signed 32-bit). |
| } | - | - |
| If(Include_PLA == 1) { | - | - |
| Power Level Adjust | 8 bits | Tx Power level adjustment (signed 8-bit, 0.25 dB units). |
| } | - | - |
| If(Include_OFA == 1) { | - | - |
| Offset Frequency Adjust | 32 bits | Tx frequency offset adjustment (signed 32-bit, Hz units). |
| } | - | - |
| } | - | - |
| } | - | - |

Size of Remaining CDMA codes

The required bandwidth for sending remaining CDMA codes received by the RS but not forwarded due to lack of bandwidth.

Channel Measurement

The mean CINR as measured on CDMA ranging code shall be quantized in 1 dB increments, ranging from a minimum of -10 dB (encoded 0x00) to a maximum of 53 dB (encoded 0x3F). (see 8.4.11.3)

SS CINR mean

MS CINR mean shall be interpreted as a single value from -16dB to 47.5 dB in units of 0.5 dB.

SS RSSI mean

The value shall be interpreted as an unsigned byte with units of 0.25dB, such that 0x00 is interpreted as -103.75 dBm, an RS shall be able to report values in the range -103.75dBm to -40 dBm.

Timing Adjust

The amount of time required to adjust MS/RS transmission so the bursts will arrive at the expected time instance at the access station. Units are PHY specific (see 10.3).

Power Level Adjust

Specifies the relative change in transmission power level that the MS/RS is to make in order that transmissions arrive at the access station at the desired power. When subchannelization is employed, the subscriber shall interpret the power offset adjustment as a required change to the transmitted power density.

Offset Frequency Adjust

Specifies the relative change in transmission frequency that the MS/RS is to make in order to better match the access station. (This is fine-frequency adjustment within a channel, not reassignment to a different channel.)