| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RS_Member_List_Update |  |
| Date Submitted | 2007-01-15 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes modification on RS_Member_List_Update |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RS_Member_List_Update

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In P802.16j/D2, the RS_Member_List_Update message could be sent via unicast or multicast with acknowledgements to ensure reliable delivery of the message. The unicast RS_Member_List_Update message could be ACK with MR_Generic-ACK or MR Acknowledgement Header. However, the ACK mechanism for multicast RS_Member_List_Update message is not defined. This contribution provides an ACK mechanism for multicast RS_Member_List_Update message.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Spec changes

[modified the following subclause in line 1 of page 68 as indicated.]

6.3.2.3.86 RS_Member_List_Update message

The superordinate station of the RS group may transmit RS_Member_List_Update message on the RS’s basic CID or the RS multicast management CID of the RS group, to update the group members with the details of the traffic burst they shall forward. This message may be transmitted whenever there is a change in the connection list of the RS group members. The RSs receiving the message may shall respond with MR_Generic-ACK or MR Acknowledgement Header for confirmation.

If the MR-BS did not receive all or some ACKs (MR_Generic-ACK or MR Acknowledgement Header) from RSs, the MR-BS shall retransmit the RS_Member_List_Update message, which contains the updated information of the RSs that the MR-BS fails to receive the ACK.

Table 183y—RS_Member_List_Update message format

| Syntax | Size | Note |
| --- | --- | --- |
| RS_Member_List_Update_message_Format { |  |  |
| Management Message Type = 93 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Frame Action Number | 4 bits | The 4-bits LSB of the frame number at the superordinate station when the configuration shall be applied by all RS group members |
| Configured_para_type | 41 bits | b0 = 1: RS sends acknowledgement if the message is received b0 = 0: RS does not send acknowledgment if the message is received b10 = 1: data forwarding on a per CID basis b10 = 0: RSs in the group shall forward all data b21 – b3: reserved |
| Reserved | 3 bits |  |
| If (b0 of Configured_para_type == 1 ) { |  |  |
| Transaction ID | 16 bits |  |
| } |  |  |
| If (b10 of Configured_para_type == 1) { |  |  |
| Number_RS | 5 bits | Number of RS group members involved with the CID list update |
| Reserved | 3 bits |  |
| for (i=0;i<Number_RS;i++) { |  |  |
| CID | 16 bits | Basic CID of the RS group member |
| If (Mode 1 is used) { |  |  |
| N_CID_Add_designated | 8 bits | Number of connections for which the RS is the designated RS. If Mode 2 is used this value is zero. |
| N_CID_Add_non_designated | 8 bits | Number of connections for which the RS is a non-designated RS. |
| for (j=0;j<N_CID_Add_designated;j++) { |  |  |
| CID | 16 bits | For this basic CID of MS the RS is designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a designated RS for this CID and shall forward data and measurements. |
| } |  |  |
| } |  |  |
| N_CID_Add_non_designated | 8 bits | Number of connections for which the RS is a non-designated RS. |
| for (j=0;j<N_CID_Add_non_designated;j++) { |  |  |
| CID | 16 bits | For this basic CID of the MS the RS is non-designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a non-designated RS for this CID and shall forward only traffic data. |
| } |  |  |
| N_CID_Remove | 8 bits | Number of CIDs to be removed from the forwarding list |
| for (j=0;j<N_CID_Remove;j++) { |  |  |
| CID | 16 bits | MS basic CID to be removed. |
| } |  |  |
| } |  |  |
| } |  |  |
| TLV Encoded Information | variable | TLV specific |
| } |  |  |