| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of TLVs for routing |  |
| Date Submitted | 2008-01-08 |  |
| Source(s) | Kanchei (Ken) Loa, Chun-Yen Hsu, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution clarifies routing TLVs |  |
| Purpose | Text proposal for 802.16j Draft Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of TLVs for Routing

Kanchei (Ken) Loa, Chun-Yen Hsu, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry

# Introduction

In subclause 11.23 of IEEE P802.16j/D2, several TLVs for routing are defined. However, some of them are redundant or not used at all, and could be removed.

We propose to remove the following TLVs.

Path-CID-Binding-Removal TLV

The Path-CID-Binding-Removal TLV could be replaced with the Path-CID-Binding-Update TLV. If the MR-BS intends to remove a binding between a CID and a path, it could send a DSD-REQ with the Path-CID-Binding-Update TLV along the path.

Path direction

IEEE 802.16j does not support asymmetric path. There is no need to specify/record the direction of a path here.

Number-of-RS TLV

The number of RSs attached in the TLV can be derived from the length of the respective TLV.

Existing path ID TLV

In IEEE P802.16j/D2, the Existing path ID TLV was used to indicate the reference path of a new path. In case the Existing path ID TLV is present, the Ordered list of RSs field records the difference between the old and new path. However, the usage for this TLV is not clear. Specifying the list of RSs along the path is a simple and robust way in creating a new path. Therefore the Existing path ID TLV could be removed.

Path-Info TLV

The Path-Info TLV is not referenced anywhere else in IEEE P802.16j/D2.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

# Spec changes :

[Modify Page 33, Line 35-43 as follows.]

The DSD-REQ message sent over relay link for the purpose of path management may contain the following TLVs:

Path ID (see 11.1.13.1)

Specification of the path to be completely removed

Path CID Binding Update  (see 11.1.13.3)

[Remove subclause 11.23]

[Insert new subclause 11.1.13]

11.1.13 Path management message encodings

The TLV encodings defined in this section are specific to the path management related MAC Management messages.

[Insert new subclause 11.1.13.1]

11.1.13.1 Path ID encoding

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 150 | 2 | ID of path | DSx-REQ, DSx-RSP, DSx-ACK |

[Insert new subclause 11.1.13.2]

11.23.2 Path Addition TLV

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Path Addition | 131 | Variable | Path ID (unsigned 16-bit) Ordered list of RSs (variable) | DSA-REQ, RNG-RSP |

Ordered List of RSs

The ordered list of RSs’ basic CIDs along the path in the downstream direction. The upstream direction list can be obtained by reverse this ordered list.

[Insert new subclause 11.1.13.3]

11.1.13.3 Path CID Binding Update TLV

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Path CID Binding Update | 130 | Variable | Path ID (unsigned 16-bit) List of CIDs (variable) | DSA-REQ, DSD-REQ, RNG-RSP |

List of CIDs

A list of CIDs involved in the binding update operation.