| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RCD TLV |  |
| Date Submitted | 2008-01-08 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan Su Chang Chae, Junhong Hui, Young-il Kim ETRI [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw schae@etri.re.kr |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes modifications on RCD TLV |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RCD TLV

Kanchei (Ken) Loa Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee
Institute for Information Industry

Su Chang Chae, Junhong Hui, Young-il Kim
ETRI

Introduction

Comment # 894 (IEEE C802.16j-07/526r3) has been accepted in Session #52. However the corresponding TLVs are not included in P802.16j/D2. This contribution is proposed to add these TLVs.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Specification Changes

6.3.2.3.65 R-link channel descriptor (RCD) message

[Insert the following text:]

The RCD message may include the following TLVs for Direct Relay Zone configuration:

Direct Relay Zone configuration(11.23.7)

[Insert the following subclause:]

11.23.7 Direct Relay Zone configurations

This field is used by an MR-BS to inform the usage of the direct relay zone to all RSs.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| DL Direct Relay Zone configuration | TBA | 4 | Number of direct relay zones (unsigned 1-bit) For(j=0;j<Number of direct relay zone;j++){ Symbol offset for Direct receiving zone j(unsigned 8-bits) No. OFDMA symbols for Direct receiving zone j(unsigned 6-bits) Symbol offset for Direct transmitting zone j(unsigned 8-bits) No. OFDMA symbols for Direct transmitting zone j(unsigned 6-bits) Reserved(unsigned 3-bits) } | RCD |
| UL Direct Relay Zone configuration | TBA | 4 | Number of direct relay zones (unsigned 1-bit) For(j=0;j<Number of direct relay zone;j++){ Symbol offset for Direct receiving zone j(unsigned 8-bits) No. OFDMA symbols for Direct receiving zone j(unsigned 6-bits) Symbol offset for Direct transmitting zone j(unsigned 8-bits) No. OFDMA symbols for Direct transmitting zone j(unsigned 6-bits) Reserved(unsigned 3-bits) } | RCD |

Number of direct relay zones

This indicates the number of direct relay zones which includes Direct Receiving and Direct Transmitting zone respectively.

Symbol offset for Direct receiving zone

The OFDMA symbol offset in which direct receiving zone starts for downlink/uplink respectively.

No. of OFDMA symbols for Direct receiving zone

The number of OFDMA symbols which RS should demodulate and modulate in the direct relay zone for downlink/uplink respectively.

Symbol offset for Direct Transmitting zone

The OFDMA symbol offset in which Direct Relay for transmitting starts for downlink or uplink respectively.

No. OFDMA symbols for Direct transmitting zone

The number of OFDMA symbols which RS should demodulate and modulate in the direct relay zone for downlink/uplink respectively.