| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RS_Path-REQ messages |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes to replace “RS_Path-REQ message” by the “RNG-RSP message with status abort and Preamble Indexes TLV” |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RS_Path-REQ messages

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

This contribution proposes to optimize the RS network entry by replacing the RS_Path-REQ message with the RNG_RSP message and skip the access station selection phase if there is no change for the access station.

In P802.16j/D2, the “RS path selection request (RS_Path-REQ) message” is used by a MR-BS to indicate the access station the RS is supposed to attach to. Since this message is only used for RS network entry, which occurs infrequency, and could be easily replaced by “ranging response (RNG-RSP) message” that has been used for similar purpose during the MS network entry. We proposes to replace the “RS_Path-REQ message” by the “RNG-RSP message with status abort and Preamble Indexes TLV” to indicate the access station the RS shall attach to. (see Figure 1)

(a) Original Scenario	(b) Proposed Scenario

Figure 1 Access Station Changed

In addition, if there is no change for the access station, the MR-BS can send an “RS_Config-CMD message” immediately to signal the ending of the access station selection phase and the starting of the RS operational configuration phase. (see Figure 2). There is no need to go through RS_Path-REQ/ME_Generic-ACK iterations.

(a) Original Scenario	(b) Proposed Scenario

Figure 2 Access Station Unchanged

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Spec changes

[Delete the following subclause in line 40 of page 66 as indicated:]

6.3.2.3.84 RS path selection request (RS_Path-REQ) message

6.3.2.3.6 Ranging response (RNG-RSP) message

[Modified the following text in line 7 of page 27 as indicated:]

The following parameter may be included in the RNG-RSP message with abort status during RS initial ranging phase or during access station selection phase for RS neighbor access stations reselection:

Preamble Indexes

8-bit preamble indexes of candidate neighbor access stations

6.3.9.16.1 RS neighbor station measurement report

[Replace the following figure in line 1 of page 101 as indicated:]

Figure 102a—Handling RCD first reception Perform neighbor measurement report at a RS

6.3.9.16.2 Access station selection

[Modified the following text in line 28 of page 101 as indicated:]

The access station selection procedure may be performed in either phase a1) or phase l). The access station selection procedure performed in phase l) (prior to "RS operation parameter configuration") is an optional procedure as indicated by the RS network entry optimization TLV in the RNG-RSP message. This operation, if required, happens after neighbor station measurement report and before RS operation parameter configuration. During this operation, the MR-BS shall determine the path (i.e. access station) of this RS based on the reported neighbour station measurements and other information such as path loading. If the current access station is changed, tThe MR-BS shall send the RS_Path-REQ RNG-RSP message which contains abort status and Preamble Indexes TLV to the RS to indicate the preamble index of a newlythe selected access station. The RS shall respond with the MR_Generic-ACK message. If the access station indicated in the RS_Path-REQ message is not the access station the RS currently attaches to, and the RS shall perform network re-entry as described in 6.3.9.

[Replace the following figure in line 41 of page 101 as indicated:]

Figure 102c— Handling RNG-RSP RS_PATH-REQ first reception at a RS

[Replace the following figure in line 23 of page 102 as indicated:]

Figure 102e—Perform path access station selection at a MR-BS

[Delete the following figure in line 1 of page 102 as indicated:]

Figure 102d—Handling RS_PATH-REQ retransmission at a RS

10.1 Global values

[Modified the following Table 583 in line 9 of page 229 as indicated:]

Table 583—Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| RS | T62 | Wait for RNG-RSP (abort) or RS_Config-CMD after sending RS_NBR-MEAS-REP to MR-BS | TBD | TBD | TBD |