| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Burst-based Forwarding in RS Group |  |
| Date Submitted | 2008-01-13 |  |
| Source(s) | Kanchei (Ken) Loa, Hua-Chiang Yin, Yung-Ting Lee, Yi-Hsueh Tsai, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution modifies the relaying support for combined ranging and initial topology discovery in order to consistent with the network entry procedure. |  |
| Purpose | Text proposal for 802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Burst-based Forwarding in RS Group

Kanchei (Ken) Loa, Hua-Chiang Yin, Yung-Ting Lee, Yi-Hsueh Tsai, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

The RS group has a superordinate station (non-transparent RS or MR-BS) that is the superordinate station of all RSs in the group. All the RSs in the RS group shall either transmit the same preamble, FCH and MAPs (non-transparent RS) or they all do not transmit any preamble, FCH or MAPs (transparent RS).

In p802.16j/D2, the burst-based forwarding scheme has been defined to accommodate frequent changes of the forwarding path due to the MS movement or the CDMA contention-based ranging within the RS group with the transparent RS. In an RS group with non-transparent RSs, the forwarding path also changes frequently due to the MS movement or the CDMA contention-based ranging, which should require a similar solution. Therefore, we proposed to apply the burst-based forwarding scheme to both transparent and non-transparent RS in the RS group

Our study shows that the burst-based forwarding scheme defined in D2 for the transparent RS could also be applied to the non-transparent RS in the RS group with the following minor modifications in the relaying scheme by re-using existing DL_Burst_Transmit_IE and the UL_Burst_Receive_IE already defined for the transparent RS

- The DL_Burst_Transmit_IE and the UL_Burst_Receive_IE for the non-transparent RS to relay data in the access zone are included in the RS_Access-MAP sent in the relay zone

- The data burst for relaying via the non-transparent RS is described by the R-MAP sent in relay zone with RS primary management CID.

In order to elaborate the burst-based forwarding scheme in the RS group, examples of the RS group with transparent RS and non-transparent RS are given in the following.

Table 1: Example of burst-based forwarding scheme with DL_Burst_Transmit_IE in RS group where all RSs do not transmit any preamble, FCH and MAPs.

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | DL Access Zone (superordinate station:Tx, MS :Rx, RS1:Rx, RS2:Rx) | DL-MAP_IE1() | MAP IEs for MSs receiving from superordinate station |
|  |  |  |  |
|  |  | DL-MAP_IEi() |  |
|  |  | DL-MAP_IEi+1 () | Data burst for RS1 itself (with RS1 basic CID) |
|  |  | DL-MAP_IEi+2() | Data burst for RS1 relaying (with RS1 primary CID) |
|  |  | DL-MAP_IEi+3 () | Data burst for RS2 itself (with RS2 basic CID) |
|  |  | DL-MAP_IEi+4() | Data burst for RS2 relaying (with RS2 primary CID) |
|  |  | STC_Zone_IE | Indicate zone switch |
|  | 1st DL Trans-parent Zone (RS1:Tx,RS2:Tx MS:Rx) | DL-MAP_IE with DL_Burst_Transmit_IE (RS1, Nr = m) | RS1 is assigned to transmit the following m legacy DL-MAP IEs for MSs |
|  |  | DL-MAP_IEj+1() | MAP IEs for MSs receiving from RS1 |
|  |  |  |  |
|  |  | DL-MAP_IEj+m() |  |
|  |  | DL-MAP_IE with DL_Burst_Transmit_IE (RS2, Nr = n) | RS2 is assigned to transmit the following n legacy DL-MAP IEs |
|  |  | DL-MAP_IEk+1() | MAP IEs for MSs receiving from RS2 |
|  |  |  |  |
|  |  | DL-MAP_IE k+n() |  |

Table 2: Example of burst-based forwarding scheme with UL_Burst_Receive_IE in RS group where all RSs do not transmit any preamble, FCH and MAPs.

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | UL Access Zone (MS:Tx, RS1:Rx, RS2:Rx) | UL-MAP_IE1() | MAP IEs for MSs transmitting to superordinate station |
|  |  |  |  |
|  |  | UL-MAP_IEi() |  |
|  |  | UL-MAP_IE with UL Burst Receive IE (RS1, m) | RS1 is assigned to receive following m UL-MAP IEs which are transmitted in DL Access Zone |
|  |  | UL-MAP_IEj+1() | MAP IEs for MS transmitting to RS1 |
|  |  |  |  |
|  |  | UL-MAP_IEj+m() |  |
|  |  | UL-MAP_IEj with UL Burst Receive IE (RS2, n) | RS2 is assigned to receive following n UL-MAP IEs which are transmitted in DL Access Zone |
|  |  | UL-MAP_IEk+1() | MAP IEs for MS transmitting to RS2 |
|  |  |  |  |
|  |  | UL-MAP_IEk+n () |  |

Table 3: Example of burst-based forwarding scheme with DL_Burst_Transmit_IE in RS group where all RSs   transmit the preamble, FCH and MAPs.

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | DL Access Zone at frame # k (superordinate station :Tx, MS :Rx, RS1:Tx, RS2:Tx) | DL-MAP_IE1() | MAP IEs for MSs receiving data from the superordinate station in RS group |
|  |  |  |  |
|  |  | DL-MAP_IEi() |  |
|  |  | DL-MAP_IEi+1() | RS1 is assigned to transmit data to MSs via the DL-MAP IEs in RS_Access-MAP received at frame # (k-1) |
|  |  |  |  |
|  |  | DL-MAP_IEj() |  |
|  |  | DL-MAP_IEj+1() | RS2 is assigned to transmit data to MSs via the DL-MAP IEs in RS_Access-MAP received at frame # (k-1) |
|  |  |  |  |
|  |  | DL-MAP_IEk() |  |
|  | DL Relay Zone at frame #k (superordinate station :Tx,, RS1:Rx, RS2:Rx) | DL-MAP_IE1 () | Data burst is for RS1 itself with RS1 basic CID via R-MAP. The data burst includes the RS_Access-MAP with DL_Burst_Transmit_IE (RS1, Nr = m) and m legacy DL-MAP IEs. |
|  |  | DL-MAP_IE2() | Data burst for RS1 relaying with RS1 primary CID via R-MAP |
|  |  | DL-MAP_IE3 () | Data burst is for RS2 itself with RS2 basic CID via R-MAP. The data burst includes the RS_Access-MAP with DL_Burst_Transmit_IE (RS2, Nr = n) and n legacy DL-MAP IEs. |
|  |  | DL-MAP_IE4() | Data burst is for RS2 relaying with RS2 primary CID via R-MAP |

Table 4: Example of burst-based forwarding scheme with UL_Burst_Transmit_IE in RS group where all RSs   transmit the preamble, FCH and MAPs.

| MR Network Topology | Zone | MAP-IEs used to describe the zone(s) | Notes |
| --- | --- | --- | --- |
|  | UL Access Zone at frame # k (superordinate station :Rx, MS :Tx, RS1:Rx, RS2:Rx) | UL-MAP_IE1() | MAP IEs for MSs transmitting data to the superordinate station in RS group |
|  |  |  |  |
|  |  | UL-MAP_IEi() |  |
|  |  | UL-MAP_IEi+1() | RS1 is assigned to receive data from MSs via the UL_Burst_Transmit_IE and UL-MAP IEs in RS_Access-MAP received at frame # (k-1) |
|  |  |  |  |
|  |  | UL-MAP_IEj() |  |
|  |  | UL-MAP_IEj+1() | RS2 is assigned to receive data to MSs via the UL_Burst_Transmit_IE and UL-MAP IEs in RS_Access-MAP received at frame # (k-1) |
|  |  |  |  |
|  |  | UL-MAP_IEk() |  |

In addition, in an RS group with non-transparent RSs, the burst-based forwarding could also be used to supplement the MPDU-based forwarding and RS_Member_List_Update messages. The combined scheme provides an efficient and complete solution. When CID routing tables of the RS group is converged, MPDU-based forwarding is the forwarding scheme that minimizes the overhead. However, When the CID routing table is required to be modified due to the RS group member updates, the MS movement or the CDMA contention-based ranging; the superordinate station unicast/multicast the RS_Member_List_Update message to related RSs and requires the acknowledgement from each RS to ensure the routing consistency within the RS group. The convergence of the CID routing table update could be time-consuming. So, the burst-based forwarding scheme could be used to mitigate the data forwarding outage due to the CID routing table convergence.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft document P802.16j/D2 are listed below.

Text Proposal

6.3.3.8.2 Transmission using station CID

[Change the following text in line 23 of page 80 as indicated]

The burst-based forwarding scheme, which is used with centralized, performs forwarding based on the burst described in the MAP IEs and utilizes forwarding rules encoded in MAPs. Data bursts that are scheduled to be relayed by the receiving RS shall be sent with the RS primary management CID. For transparent RS and non-transparent RS in an RS group, the DL_Burst_Transmit_IE and UL_Burst_Receive_IE, as described in 8.4.5.3.28 and 8.4.5.4.29, respectively, shall be used. The DL_Burst_Transmit_IE and the UL_Burst_Receive_IE for transparent RS are described in DL-MAP and UL-MAP, respectively. Whereas the DL_Burst_Transmit_IE and the UL_Burst_Receive_IE for non-transparent RS are described in RS_Access-MAP. The DL_Burst_Transmit_IE describes the DL data relaying information and the UL_Burst Receive_IE describes UL data relaying information. For DL MAP IEs following the DL_Burst_Transmit_IE, the RS shall forward the data in allocations defined by these IEs, where the forwarded data is received in the DL burst with the RS primary management CID. For UL MAP IEs following the UL_Burst_Receive_IE, the RS shall receive the data in allocations defined by these IEs and forward to its superordinated station in the next available allocation, defined by legacy UL-MAP IE, in UL relay zone.