| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | .16j distributed authentication MS SA management |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Sheng Sun; Guo-Qiang Wang; Hang Zhang; Peiying Zhu; Wen Tong; Mo-han Fong 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a |  |
| Abstract |  |  |
| Purpose | To incorporate the proposed text into the P802.16j/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# .16j Distributed authentication SA Management

Sheng Sun; Guo-qiang Wang; Hang Zhang;

Peiying Zhu; Wen Tong; Mo-han Fong

Nortel

# Introduction

The proposal is aimed to reply to comments that strengthen the distributed authentication with the description of the SA management and the scheme to protect the MS’s PKM messages over the Relay Link.

Within the distributed authentication, after the AR RS issues the PKMv2 REQ which requests the identity from the MS, the MS starts off the authentication process by issuing the MS’s PKM  RSP messages towards the AR RS which will eventually be relayed to MR BS. The MS’s PKM RSP messages shall be protected with  Messages Authentication Code (HMAC/CMAC) calculated from the AR RS’s HMAC/CMAC_KEY_UL in order for the MR BS to verify the relayed MS’s PKM messages is from an authenticated AR RS.

The  Security Association (SA) that contains the security pertaining to one or more MSs will be created on MS, AR RS and the MR BS in order to support secure communications across the IEEE 802.16 network. The same three types of SAs are defined with accordance to the SA definition in sec 7.2.1.1in IEEE 802.16e-2005: Primary, Static, and Dynamic. Each MS establishes a primary security association during the MS initialization process. Static SAs are provisioned within the BS. Dynamic SAs are established and eliminated, on the fly, in response to the initiation and termination of specific service flows. SAs are identified using SAIDs. Each MS shall establish an exclusive primary SAs with AR RS and the MR BS. The scheme for identifying the primary SAIDs is as the following table

| Location | SA Identification |
| --- | --- |
| MS | SAID (MS Basic CID) |
| AR-RS | SAID (Access RS Basic CID) , SAID (MS Basic CID) |
| MR-BS | SAID (Access RS Basic CID) , SAID (MS Basic CID) |

Using the PKM protocol, an MS requests from its BS an SA’s keying material through the AR RS. The BS shall ensure that each client MS only has access to the SAs it is authorized to access.

# 2. Proposed text changes

++++++++++++ start text proposal ++++++++++++++++++++++++++++++++++

[Insert the followings after the end of section  7.1.7 ]

Within the distributed authentication, after the AR RS issues the PKMv2 REQ which requests the identity from the MS, the MS starts off the authentication process by issuing the MS’s PKM  RSP messages towards the AR RS which will eventually be relayed to MR BS. The MS’s PKM RSP messages shall be protected with  Messages Authentication Code (HMAC/CMAC) calculated from the AR RS’s HMAC/CMAC_KEY_UL in order for the MR BS to verify the relayed MS’s PKM messages is from an authenticated AR RS.

The  Security Association (SA) that contains the security pertaining to one or more MSs will be created on MS, AR RS and the MR BS in order to support secure communications across the IEEE 802.16 network. The same three types of SAs are defined with accordance to the SA definition in sec 7.2.1.1 of IEEE 802.16e-2005: Primary, Static, and Dynamic. Each MS establishes a primary security association during the MS initialization process. Static SAs are provisioned within the BS. Dynamic SAs are established and eliminated, on the fly, in response to the initiation and termination of specific service flows. SAs are identified using SAIDs. Each MS shall establish an exclusive primary SAs with AR RS and the MR BS. The scheme for identifying the primary SAIDs is as the following table.

| Location | SA Identification |
| --- | --- |
| MS | SAID (MS Basic CID) |
| AR-RS | SAID (Access RS Basic CID) , SAID (MS Basic CID) |
| MR-BS | SAID (Access RS Basic CID) , SAID (MS Basic CID) |

Using the PKM protocol, an MS requests from its BS an SA’s keying material through the AR RS. The BS shall ensure that each client MS only has access to the SAs it is authorized to access.

++++++++++++++++++++ End of text proposal +++++++++++++++++++++