| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Optimizing MS HO |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Yunbao Zeng, Ting Li, Liangliang Zhang, Shulan Feng Hisilicon Technologies Harbour Building, No.8, Dongbeiwang West Road, HaiDian District, Beijing, China | Voice: 86-10-82829055 Fax: 86-10-82829075 mailto:zengyunbao@huawei.com |
| Re: | This contribution is a response to “IEEE 802.16 Working Group Letter Ballot Recirc #28a: anouncement”. |  |
| Abstract | This contribution proposes a method for optimizing MS HO in MR network. |  |
| Purpose | Text proposal for 802.16j Draft Document 2.0. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Optimizing MS HO

Yunbao Zeng, Ting Li, Liangliang Zhang, Shulan Feng

Hisilicon Technologies

# Introduction

As defined in 802.16Rev2/D1, Serving BS criteria for recommendation of target BS may include factors such as expected MS performance at potential target BS, BS and network loading conditions, and MS QoS requirements. This means that the MS performance such as MS UL CINR obtained by a potential target BS during MS association procedure is a factor for the Serving BS to determine whether to recommend the potential target BS for the MS through an MOB_BSHO-REQ/RSP message.

In MR networks, when MS’s potential target station is a non-transparent RS, MS performance at the RS such as UL CINR during MS association procedure is also a factor for recommendation of target station by the MS’s Serving MR-BS. However, such information is not obtained by an MS’s Serving MR-BS in the current Draft, and the lack of this may cause inaccurate of the recommended target station(s) determined by the MS’s Serving MR-BS.

# Proposal

During MS association procedure, a non-transparent RS may learned information such as MS UL PHY quality. The RS may notify its Serving MR-BS of such information.

When the RS’s Serving MR-BS is the MS’s neighbor MR-BS, the RS’s Serving MR-BS may forward the information obtained from the RS to the MS’s Serving MR-BS through backbone.

At the HO decision stage, when obtaining information such as MS UL PHY quality learned by a non-transparent RS, the MS’s Serving MR-BS may take such information as a factor to determine whether to recommend the RS as the MS's target station.

# Proposed Text

[Insert the following sentence at the end of the paragraph in subclause 6.3.22.1.4.2:]

Additionally, the RNG-REQ message the RS forwards to its Serving MR-BS may include an Association Info TLV (see 11.5), in which information may be used by the MS’s serving MR-BS to determine whether to recommend the RS as the MS's target station at the HO decision stage.

[Insert the following sentence at the end of the paragraph in subclause 6.3.22.1.4.3:]

Additionally, the RNG-REQ message with the TLV parameters (Serving BS ID, MS MAC address) the RS forwards to its Serving MR-BS may include an Association Info TLV, in which information may be used by the MS’s serving MR-BS to determine whether to recommend the RS as the MS's target station at the HO decision stage.

[Insert the following text after the first paragraph in subclause 6.3.22.5.1:]

At the HO decision stage, if the MS’s Serving MR-BS has obtained information of MS UL PHY quality learned by an associated RS, it may take such information as a factor to determine whether to recommend the RS as the MS's target station.

[Insert the following text at the end of 6.3.2.3.5:]

The following parameter may be included in the RNG-REQ message which an RS forwards to its Serving MR-BS when an MS is attempting to perform association at the RS:

Association Info TLV

[Insert the following row in Table 622:]

| Name | Type | Length | Value | PHY Scope |
| --- | --- | --- | --- | --- |
| Association Info | TBD | 2 | Bit#0~Bit#7:MS CINR mean Bit#8~Bit#15:MS RSSI mean | OFDMA |