| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RS operational mode |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Yunbao Zeng, Liangliang Zhang, Ti Li, Shulan Feng Hisilicon Technologies Harbour Building, No.8, Dongbeiwang West Road, HaiDian District, Beijing, China | Voice: 86-10-82829055 Fax: 86-10-82829075 mailto:zengyunbao@huawei.com |
| Re: | This contribution is a response to “IEEE 802.16 Working Group Letter Ballot Recirc #28a: anouncement”. |  |
| Abstract | This contribution proposes a method for optimizing the configuration of RS operational mode |  |
| Purpose | Text proposal for 802.16j Draft Document 2.0. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Comments on RS operational mode

Yunbao Zeng, Liangliang Zhang, Ting Li, Shulan Feng

Hisilicon Technologies

# 1 Introduction

A non-transparent RS transmits DL frame-start preamble, FCH, MAP message(s) and channel descriptor (DCD/UCD) messages. But a transparent RS doesn’t transmit those ones.

During the Negotiate basic capabilities process, the response of MR-BS is the subset of RS capabilities present in the SBC-REQ message. The MR-BS responds to the RS and indicates whether these capabilities may be used.

As defined in 802.16j draft2, after registration, transparent RS and non-transparent RS work in a different way. But before registration, there is not any exact definition for RS which works as a transparent RS or a non transparent RS. One bit indication (bit#0 access zone preamble transmission support) in SBC-REQ/RSP is used to indicate RS whether can support access zone preamble transmission. It just means RS have an ability to send a preamble, but it does not mean that RS has to send a preamble. RS may be able to support both transparent and non-transparent mode.

Therefore it is not defined very clearly that whether RS works as a transparent RS or a non transparent RS during the network initial process.

# 2 Proposal

If a RS can support access zone preamble transmission, BS indicates the RS to work as transparent or non-transparent mode by using one bit in REG-REQ/RSP message during network initial process.

When BS indicates RS to work as transparent RS, so that bit#17 is set 0.Otherwise RS works as non transparent RS. REG-REQ/RSP TLV is showed as follow:

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 49 | 3 | Bit #0: Centralized scheduling mode support Bit #1: Distributed scheduling mode support Bit #2: NBR-ADV generating support Bit #3: Tunnel packet mode support Bit #4: Tunnel burst mode support Bit #5: RS mobility support Bit #6: Subordinate RS network entry support Bit #7: Location support Bit #8: Multicast management support Bit #9: DL Flow control Bit #10: MRS mode Bit #11: RS centralized security support Bit #12: RS distributed security support Bit #13: Embedded path management support Bit #14: Explicit path management support Bit #15: Burst-based forwarding support Bit #16: Local CID allocation support Bit #17:0=Transparent RS,1=Non-transparent RS Bit #18-#23: Reserved | REG-REQ REG-RSP |

# 3 Proposed Text Changes

11.7.8.10 MR MAC feature support

[Change the table as indicated:]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 49 | 3 | Bit #0: Centralized scheduling mode support Bit #1: Distributed scheduling mode support Bit #2: NBR-ADV generating support Bit #3: Tunnel packet mode support Bit #4: Tunnel burst mode support Bit #5: RS mobility support Bit #6: Subordinate RS network entry support Bit #7: Location support Bit #8: Multicast management support Bit #9: DL Flow control Bit #10: MRS mode Bit #11: RS centralized security support Bit #12: RS distributed security support Bit #13: Embedded path management support Bit #14: Explicit path management support Bit #15: Burst-based forwarding support Bit #16: Local CID allocation support Bit #17: 0=Transparent RS,1=Non-transparent RS Bit #18-#23: Reserved | REG-REQ REG-RSP |