| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A more efficient DL-MAP IE for MAPs |  |
| Date Submitted | 2008-01-13 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Tsung-Yu Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes the compressed DL-MAP IE |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A more efficient DL-MAP IE for MAPs

Kanchei (Ken) Loa, Tsung-Yu Tsai, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In the current definition of the allocation of downlink data in WirelessMAN-OFDMA, each allocation (i.e. a burst) is indicated as a two-dimensional rectangle located in a table with time axis (OFDMA symbol) and frequency axis (sub-channel). For description of such a 2-D allocation, the current DL-MAP IE adopts totally four parameters, that is, a “OFDMA symbol offset” and a “OFDMA sub-channel offset” to locate the coordinate of the left-top point of the allocated rectangular, then, “number of OFDMA symbols” and “number of OFDMA sub-channels” to indicate its length and width respectively. This approach is quite intuitive, but seems not be efficient as well, since some parameters may not be necessary and bring additional MAP overhead.

In this contribution, we propose a more efficient DL-MAP IE with only two parameters: “number of OFDMA symbols” and “number of sub-channels”. Theoretically speaking, the new DL-MAP IE can save up to 30% overhead per IE. This improvement will become significant in several prevalent scenarios such as environment dominant by services with low data rate and small packet like VoIP where large amount of bursts may be aggregated in the same frame, or environment supporting multi-hop relay with non-transparent relay stations where R-MAP transmitted in a frame, and the impact of MAP overhead becomes more crucial.

Proposed Reduced DL-MAP IE

Table 1 shows the DL-MAP IE in the current draft document. Table 380a is our proposed new DL-MAP IE. Briefly speaking, our proposed new IE has two main differences with that of the conventional DL-MAP IE in current draft document. Instead of using four parameters to describe a burst, our new IE removes two parameters and only uses “number of OFDMA symbols” and “number of sub-channels” to represent a burst, thus our new IE has much less overhead.

Table 1 DL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| DL-MAP_IE() { |  |  |
| DIUC | 4 bits |  |
| if (DIUC == 14 { |  |  |
| Extended-2 DIUC dependent IE | variable |  |
| } else if (DIUC == 15) { |  |  |
| Extended DIUC dependent IE | variable | See subclauses following 8.4.5.3.1 |
| } else { |  |  |
| if (INC_CID == 1) { |  | The DL-MAP starts with INC_CID =0. INC_CID is toggled between 0 and 1 by the CID-SWITCH_IE() (8.4.5.3.7) |
| N_CID | 8 bits | Number of CIDs assigned for this IE |
| for (n=0; n< N_CID; n++) { |  |  |
| If ( included in SUB-DL-UL-MAP) { |  |  |
| RCID_IE() | variable | For SUB-DL-UL-MAP, reduced CID format is used |
| } else { |  |  |
| CID | 16 bits | Represents the assignment of the IE to a broadcast, multicast, or unicast address. |
| } |  |  |
| } |  |  |
| } |  |  |
| OFDMA Symbol offset | 8 bits |  |
| if (Permutation = 0b11 and (AMC type is 2x3 or 1x6)) { |  |  |
| Subchannel offset | 8 bits |  |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| No. OFDMA triple symbol | 5 bits | Number of OFDMA symbols is given in multiples of 3 symbols |
| No. Subchannels | 6 bits |  |
| } else { |  |  |
| Subchannel offset | 6 bits |  |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| No. OFDMA Symbols | 7 bits |  |
| No. Subchannels | 6 bits |  |
| } |  |  |
| Repetition Coding Indication | 2 bits | 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| } |  |  |
| } |  |  |

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Spec changes

8.4.5.3 DL-MAP IE format

[Insert the following paragraphs at end of 8.4.5.3 DL-MAP:]

Table 380a Reduced DL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Compressed_DL-MAP_IE() { | - | - |
| DIUC | 4 bits |  |
| if (DIUC == 14 { | - | - |
| Extended-2 DIUC dependent IE | variable |  |
| } else if (DIUC == 15) { | - | - |
| Extended DIUC dependent IE | variable | See subclauses following 8.4.5.3.1 |
| } else { | - |  |
| if (INC_CID == 1) { | - | The DL-MAP starts with INC_CID =0. INC_CID is toggled between 0 and 1 by the CID-SWITCH_IE() (8.4.5.3.7) |
| N_CID | 4 bits | Number of CIDs assigned for this IE |
| for (n=0; n< N_CID; n++) { | - | - |
| RCID_IE() | variable | For R-MAP, RS_Access-MAP and RS_Relay-MAP, reduced CID format is used |
| } | - | - |
| } | - | - |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| if (Permutation = 0b11 and (AMC type is 2x3 or 1x6)) { | - | - |
| No. OFDMA triple symbol | 5 bits | Number of OFDMA symbols is given in multiples of 3 symbols |
| } else { | - | - |
| No. OFDMA Symbols | 7 bits |  |
| } | - | - |
| No. Subchannels | 6 bits |  |
| Repetition Coding Indication | 2 bits | 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| } | - | - |
| } | - | - |

[Modify the following subclause 8.4.5.10 as following indicated:]

8.4.5.10 R-MAP message

Table 496a—R-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID 11 0b10 = RCID 7 0b11 = RCID 3 |
| DL-MAP IE format | 1 bit | 0b0 = Normal DL-MAP IE 0b1 = Compressed DL-MAP IE |
| Length | 11 bits | Length of R-MAP in bytes |
| DL_IE count | 6 bits | Number of DL_IE in the burst |
| UL_IE count | 6 bits | Number of UL_IE in the burst |
| for(i=0;i<DL_IE count; i++){ |  |  |
| If(DL-MAP IE format == 0b0) { |  |  |
| DL_-MAP_IE() | variable |  |
| } else { |  |  |
| Compressed_DL-MAP_IE() | variable |  |
| } |  |  |
| } |  |  |