| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | State machine for RS Network Entry |  |
| Date Submitted | 2008-10-13 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan [add other co-authors here] | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes state machine for RS network entry |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

State machine for RS Network Entry

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In P80216Rev2/D2, there are two network entry state machines defined for the BS side and the SS side respectively (Figure 85 and Figure 86). However, in P802.16j/D2, there is no “Network entry state machine” to describe error path for the RS and the MR-BS. We proposed to add the “Network entry state machine” for the transparent RS, the non-transparent RS and the MR-BS.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D2 are listed below.

Spec changes

[Insert the following subclause as indicated:]

6.3.9.16.4 Network entry state machine

Figure xxx describe the network entry process on the transparent RS side. Figure yyy describe the network entry process on the non-transparent RS side. Figure zzz describes the network entry process on the MR-BS side.

Figure xxx—Ranging procedure in network entry state machine at a transparent RS

Figure yyy—Ranging procedure in network entry state machine at a non-transparent RS

Figure zzz—Ranging procedure in network entry state machine at a MR-BS

10.1 Global values

[Modified the following Table 583 in line 9 of page 229 by including the following entries as indicated:]

Table 583—Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MR-RS | T63 | Wait for ACK after sending RS_Config-CMD to RS | TBD | TBD | TBD |
| MR-RS | T64 | Wait for RS_NBR-MEAS-REP after sending REG-RSP to RS | TBD | TBD | TBD |