| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggestion on RS Broadcast Message Relaying in MR Network |  |
| Date Submitted | 2008-01-23 |  |
| Source(s) | Yuqin Chen, Hongyun Qu ZTE Corp Mary Chion ZTE USA Inc Kanchei(Ken) Loa, Yung-Ting Lee Institute for Information Industry | E-mail: chen.yuqin@zte.com.cn loa@iii.org.tw |
| Re: | IEEE 802.16-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement |  |
| Abstract | This contribution proposes a method to provide sufficient RS broadcast message relaying scheme. |  |
| Purpose | Discuss and adopt proposed text in TG16j |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Remedy on Periodical RS Broadcast Message Relaying in Non-transparent Mode with Centralized Scheduling

Yuqin Chen, Hongyun Qu, Mary Chion,Yang Liu

ZTE

# Introduction

As defined in current draft 16j/D2, before RS transmits DCD/UCD message in non-transparent mode with centralized scheduling, RS shall request MR-BS for downlink bandwidth. However, this scheme bears several drawbacks. First, after RS requesting bandwidth, MR-BS has no information which message is going to forward on this bandwidth. Therefore, if the DCD/UCD change count changes, how does MR-BS know the exact time when to adopt the new DCD/UCD change count in DL-MAP and UL-MAP. Second, he periodic bandwidth requesting procedure brings additional overheads. Third, upon receiving the bandwidth request, it is not guaranteed that there is available bandwidth for this purpose, which causes latency.

Since MR-BS is aware the periodical interval for RS to broadcast these messages, we propose a scheme that MR-BS periodically allocates unsolicited downlink bandwidth to RS for relaying. Moreover, to enable RS broadcast fragmental DCD/UCD message, when some RS_BW-Alloc_IE is lost, RS requests bandwidth by RS BR header.

# Proposed Solutions

Under centralized scheduling, upon receiving the DCD/UCD message with RS primary CID, the RS shall acknowledge the reception of DCD or UCD messages over primary management connection by sending an acknowledgement header(See 6.3.2.1.2.2.2.3). The transaction ID of the ACK header shall be set to the Configuration Change Count of DCD or UCD message.

After receiving the DCD/UCD messages, RS shall save them and periodically broadcast them over relay or access links to RS or MS. To let MR-BS have the exact time when RS broadcasts the DCD/UCD message, and to reduce the overhead of requesting bandwidth every time and make sure the bandwidth be allocated in time, MR-BS periodically assigns unsolicited downlink bandwidth to RS for broadcast message relaying, and the bandwidth allocation is done by “RS_BW-Alloc_IE”. If the access link channel is not enough, MR-BS shall allocate less bandwidth to RS, then DCD/UCD shall be sent out on fragmentable broadcast CID. If the timer to receive the RS_BW-Alloc_IE expires, RS shall request bandwidth through RS BR header.

# Proposed Text

[To modify section 6.3.28.1 on Page 154.]

6.3.28.1   RS broadcast message relaying

[To change the fourth paragraph as follows]

Under centralized scheduling, as shown in Figure 157d, the RS should request bandwidth on the access link to broadcast the DCD/UCD message with fragmentable broadcast CID ,after receiving Acknowledgement header for DCD/UCD reception from RS, MR-BS shall periodically allocate unsolicited bandwidth to RS to broadcast the DCD/UCD message over relay link and access link using RS_BW-Alloc_IE. To enable RS send DCD/UCD message with fragmentable broadcast CID, if some RS_BW-Alloc_IE is lost, RS shall request bandwidth by RS BR header after the timer Txx expires.

[Modify Figure 157d as follows]

[To modify section 8.4.5.10.1.2 on page 211 as follows]

8.4.5.10.1.2 RS bandwidth allocation IE (RS_BW-ALLOC_IE)

This IE is transmitted to a RS from MR-BS. This IE provides the allocation to RS for transmission of messages composed by the RS over access link and relay link to MS and RS.

[To modify Table 496e on page 211 as follows. ]

Table 496e-RS_BW-ALLOC IE format

| Name | Length | Description |
| --- | --- | --- |
| RS_BW-ALLOC_IE{ | - | - |
| Type | 5bits | RS_BW-ALLOC_IE = 0x01 |
| Length | 4bits | variable |
| RCID_IE() | 4,8,12,16bits | RS basic CID in RCID_IE format (see 8.4.5.3.20.1) |
| Type | 1bit 2bits | 0b00:Response for RS BR header 0b01:RS broadcasting RNG-RSP 0b10: unsolicited DL bandwidth allocation to RS by MR-BS 0b11:reserved |
| If(Type==0x00) { | - | - |
| TID | 4bits | Transaction ID |
| }else if(Type==0x01) | - | - |
| Frame Number Index | 4bits | LSBs of relevant frame number |
| Number of rejected SS | 4bits | Number of rejected SS (i.e. RNG-RSP message with status “Abort”) |
| INC_RNG_SUC | 1bit | Include bandwidth for RNG-RSP message with status “success” (0b1:no, 0b1:yes) |
| INC_DFO | 1bit | Include bandwidth for RNG-RSP message containing downlink frequency override (0b1:no, 0b1:yes) |
| } |  |  |
| DL-MAP IE index | 8bits | RS shall transmit message on the burst described by the k-th DL MAP IE within the DL-MAP message broadcasted by the RS at the next available frame after receiving this IE, where k is the DL-MAP IE index. |
| If(Type==0b10) { | - | - |
| Message Type | 3bits | 0b000: DCD 0b001: UCD 0b010-0b111:reserved |
| DL-MAP IE index k | 8bits | RS shall transmit message on the burst described by the k-th DL MAP IE within the DL MAP message broadcasted by the RS at the next available frame after receiving this IE, where k is the DL-MAP IE index. |
| R-DL-MAP IE index k | 8bits | RS shall transmit message on the burst described by the k-th R-DL MAP IE within the R-DL-MAP message broadcasted by the RS at the next available frame after receiving this IE, where k is the R-DL-MAP IE index. |
| } |  |  |
| } |  |  |

[To add a Timer in 10.1 on page 229 as follows]

| System | Name | Time Reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| RS | Txx | Time the RS waits for unsolicited RS_BW-Alloc_IE from MR-BS | TBD | TBD | TBD |

# Reference

- “Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Multihop Relay Specification”, IEEE 802.16j/D2, December 2007

.