| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ACK Notification for RS in UL HARQ |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | LIU Yang, QU hongyun, CHEN Yuqin ZTE Corporation Mary Chion ZTE USA 706/4, Liantang Pengji Industrial Park, Shenzhen, P.R.China 518004 Youngbin Chang Samsung Elecronics | E-mail: [liu.yang8@zte.com.cn] Email: [yb.chang@samsung.com] |
| Re: | IEEE 802.16-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement |  |
| Abstract | This contribution provides a design to notify ACK for RS in UL HARQ. The scenarios of initial transmission and retransmission are unified. |  |
| Purpose | Revise the text to the section 8.4.5.10.1.7 “HARQ ACK region allocation IE” in IEEE 802.16j/D2. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# ACK Notification for RS in UL HARQ

LIU Yang, QU hongyun, CHEN Yuqin  Mary Chion,   Youngbin Chang

ZTE Corporation,         Samsung Elecronics

Introduction

This contribution provides how to notify ACK for RS in UL HARQ. The scenarios of initial transmission and retransmission are all included.

In current IEEE 802.16j draft [1], for end to end UL HARQ, in the initial data transmission of multhop relay network, to determine the frame where the ACK region lies, following statements are found. “In the case of a non-transparent RS, the RS that receives HARQ UL sub burst, from MS or sub-ordinate RS for relaying to MR-BS at frame i shall transmit the ACK/NAK signal through the ACK Channel in the CKCH region along with the UL MS HARQ sub-burst at frame (i+k). The RS shall transmit the ACK/NAK signal according to the order of UL HARQ sub-burst in the UL-MAP. The frame offset k is defined by the "HARQ ACK Delay for UL Burst for MR" field in the UCD message.”

However, this design is not applicable to the retransmission. Since no sub burst will be received in the RS where retransmission begins, the RS may not know when to send the receiving status report.

This contribution proposes proposed a MAC design to unify the ACK notification in both transmission and retransmission scenarios.

# Specified text changes

[Replace the last paragraph in P.217 and the first paragraph in P.218 with following text]

8.4.5.10.1.7 HARQ ACK region allocation IE

In the case of a non-transparent RS, the RS that receives HARQ UL sub burst, from MS or sub-ordinate RS

for relaying to MR-BS at frame i shall transmit the ACK/NAK signal through the ACK Channel in the

ACKCH region along with the UL MS HARQ sub-burst at frame (i+k). The RS shall transmit the ACK/

NAK signal according to the order of UL HARQ sub-burst in the UL-MAP. The frame offset k is defined by

the "HARQ ACK Delay for UL Burst for MR" field in the UCD message.

In case of a non-transparent RS, the RS that received the HARQ UL MAP IE shall transmit ACK/NAK to the superordinate station in the frame when the RS sends the corresponding HARQ sub-bursts indicated by HARQ UL MAP IE which may includes retransmission burst starting from its own RS. The UL HARQ procedure follows the description in section 6.3.17.5.1.

The ACK/NAK for UL HARQ transmission is allocated by HARQ ACK region allocation IE and the order of ACKs in UL ACKCH shall follow the order of UL HARQ sub-burst in the HARQ UL MAP IE. [Edited]

# Reference

[1] IEEE P802.16j/D2, Draft IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems Multihop Relay Specification, Dec., 2007.

[2] IEEE C802.16j-07/524r2, “Corrections to HARQ ACK/NAK Scheduling”