| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay MAC PDU Construction |  |
| Date Submitted | 2008-01-13 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 Canada Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab 201 Broadway Cambridge, MA 02139 USA Toshiyuki Kuze Mitsubishi Electric Corp 5-1-1 Ofuna Kamakura, Kanagawa 2478501, Japan | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com Voice: +617-621-7557 E-mail : tao@merl.com Voice: +81-467-41-2885 E-mail : Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | IEEE 802.16j-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a |  |
| Abstract |  |  |
| Purpose | To incorporate the proposed text into the P802.16j/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Relay MAC PDU Construction

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze

Mitsubishi Electric Corp

# 1. Instruction

In D2, the description of relay MAC PDU construction is missing. This contribution is addressing this topic.

2. Proposal

One of functionality of R-MAC function is to relay MPDUs and enable packing/repacking and segmenting/re-segmenting of MPDUs to fit the time-variant available resource of each link over a path. In order to reduce packing overhead, tunnel data unit (TDU) is defined as a unit which includes a number of MPDUs of a logical tunnel. R-MAC PDU then is constructed by encapsulating a number of TDUs and/or fragments of TDUs.

The tunnel data units are created by the ingress station (MR-BS or access RS) of a tunnel. A tunnel data unit is logically partitioned into chunks whose length is specified by the Tunnel_Data_Unit_Chunk_Size TLV in RCD message. The chunk sequence number CSN is assigned to a chunk by the ingress station and shall remain unchanged during the forwarding over a tunnel. Therefore the order of tunnel data units is able to be kept between the ingress station and the egress station of a tunnel. If segmentation of a TDU is required, the length of the fragment (excepting the last fragment) should be the integer multiple of chunk size.

During data forwarding, the intermediate RS may re-pack or re-segment the received tunnel data units when it constructs its R-MAC PDU. The intermediate RS may send a fragment of a tunnel data unit without waiting until the entire tunnel data unit is received.

The egress station reassembles tunnel data unit created by the ingress station. The egress station may re-order the received tunnel data units if the received tunnel data unit flow is found to be out-of-order based on the chunk sequence numbers assigned by the ingress station. The egress station separates MPDUs carried by tunnel data unit by reading generic MAC headers of MPDUs

The following Figure shows one example of R-MAC PDU construction and process by ingress RS, intermediate RS and egress RS of a tunnel.

Figure. R-MAC PDU construction and process.

3. Proposed text change

[Add the following section 6.3.2.2.8.3 and 6.3.2.2.8.4  as indicated]

6.3.2.2.8.3 Fragmentation subheader (FSH)

The fragmentation subheader format is specified in Table XXX.

Table XXX – Fragmentation subheader format.

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| fragment subheader() { |  |  |
| FC | 2 | 00 = no fragmentation of MPDU 01 = last fragmentation of MPDU 10 = first fragment of MPDU 11 = middle fragment of MPDU |
| CSN | 14 | Chunk sequence number |
| } |  |  |

The fragmentation subheader contains a CSN which is the chunk sequence number of the first chunk in the sequence of chunks following the subheader.

6.3.2.2.8.4 Packing subheader (PSH)

The packing subheader format is specified in Table XXX.

Table XXX – Packing subheader format.

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| packing subheader() { |  |  |
| FC | 2 | 00 = no fragmentation of MPDU 01 = last fragmentation of MPDU 10 = first fragment of MPDU 11 = Middle fragment of MPDU |
| CSN | 14 | Chunk sequence number |
| Length | 12 |  |
| Reserved | 4 |  |
| ) |  |  |

The packing subheader contains a CSN which is the chunk sequence number of the first chunk in the sequence of chunks following the subheader.

[Add the following section 6.3.3.8.3 as indicated]

6.3.3.8.3 R-MAC PDU construction in relay network

One of functionality of R-MAC function is to relay MPDUs and enable packing/repacking and segmenting/re-segmenting of MPDUs to fit the time-variant available resource of each link over a path. In order to reduce packing overhead, tunnel data unit (TDU) is defined as a unit which includes a number of MPDUs. R-MAC PDU then is constructed by encapsulating a number of TDUs and/or fragments of TDUs.

The tunnel data units are created by the ingress station (MR-BS or access RS) of a tunnel. A tunnel data unit is logically partitioned into chunks whose length is specified by the Tunnel_Data_Unit_Chunk_Size TLV in RCD message. The chunk sequence number CSN is assigned to a chunk by the ingress station and shall remain unchanged during the forwarding over a tunnel. Therefore the order of tunnel data units is able to be kept between the ingress station and the egress station of a tunnel. If segmentation of a TDU is required, the length of the fragment (excepting the last fragment) should be the integer multiple of chunk size.

During data forwarding, the intermediate RS may re-pack or re-segment the received tunnel data units when it constructs its R-MAC PDU. The intermediate RS may send a fragment of a tunnel data unit without waiting until the entire tunnel data unit is received.

The egress station reassembles tunnel data unit created by the ingress station. The egress station may re-order the received tunnel data units if the received tunnel data unit flow is found to be out-of-order based on the chunk sequence numbers assigned by the ingress station. The egress station separates MPDUs carried by tunnel data unit by reading generic MAC headers of MPDUs

Figure XXX shows one example of R-MAC PDU construction and process by ingress RS, intermediate RS and egress RS of a tunnel.

Figure xxx. R-MAC construction and process by ingress RS, intermediate RS and egress RS of a tunnel.

[Add the following in 19 of page 38]

Tunnel_Data_Unit_Chunk_Size (see 11.25.7)

[Add section 11.25.7]

11.25.7 Tunnel_Data_Unit_Chunk_Size

This TLV indicates the chunk size (in bytes) of tunnel data unit.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Tunnel_Data_Unit_Chunk_Size | 18 | 1 | 1-256 | RCD |