| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Construction of MAC PDU of Access RS in distributed security scenario |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/059: IEEE 802.16 Working Group Letter Ballot Recirc #28a |  |
| Abstract |  |  |
| Purpose | To incorporate the proposed text into the P802.16j/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Construction of MAC PDU of Access RS in Distributed Security Scenario

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden

Nortel

# 1. Introduction

In current baseline document, both centralized and distributed security modes are described. In the distributed security mode, the security materials of a MS are accessible and kept by the access RS of a MS. Such an access RS implements MS traffic en/decryption and message authentication functions and MS SDUs are visible to access RS. Therefore, in DL, all SDUs to MSs attached to an access RS can be viewed as SDUs targeting to the access RS. In UL, all received MAC SDUs from MSs by an access RS can be viewed as the SDUs originated from this RS.

We propose a solution that

- MS SDUs from the same or different service flows are encapsulated into a single MPDU of the RS

- An extended subheader is used to carry CIDs and related information which are extracted from MS MAC PDUs

- each RS MAC PDU is encrypted using the security materials of the corresponding access RS

- such a MAC PDU is encapsulated into a R-MAC PDU for forwarding purpose

The main reason of introducing the method is to utilize the benefit of distributed security (RS is able to process MS traffic at SDUs level) to minimize the encryption overhead.

Since each encrypted MPDU has the overhead related to encryption (12 bytes for AEC CCM) and CRC (4 bytes). When N MS SDUs can be forwarded together using R-MAC PDU, the encryption and CRC overhead in first method would be N times of that of second method. This overhead issue becomes severer for short SDUs (VoIP) case.

.

2. Proposed text change

[Modify last row of Table 13b as indicated]

| ES type | Name | ES body size | Description |
| --- | --- | --- | --- |
| 6 | CID extended subheader | variable | 6.3.2.2.7.9 |
| 7-127 | Reserved | - | - |

[Modify last row of Table 13c as indicated]

| ES type | Name | ES body size | Description |
| --- | --- | --- | --- |
| 5 | CID extended subheader | variable | 6.3.2.2.7.9 |
| 6-127 | Reserved | - | - |

[Add the section 6.3.2.2.7.9 as indicated]

++++++++++++++++++++++ Start Text ++++++++++++++++++++++++++++++++

6.3.2.2.7.9 CID extended subheader

This extended subheader is used to RS MAC PDU to carry information of CID of SDUs, and related information, which are extracted from MS PDU header, sub-header and extended subheader.

| Name | Size | Description |
| --- | --- | --- |
| Number of SDUs | 8 bits | Number of SDUs encapsulated in the payload of the MAC PDU |
| For (i = 0; i<=Number of SDUs; i++) { |  |  |
| CID | 16 bits |  |
| ARQ_flag | 1 bit |  |
| ExSH_flag | 1 bit |  |
| If (ARQ_flag == 1) { |  |  |
| ARQ feedback payload } | x bits | ARQ feedback payload associated with the CID which need to be sent over access link |
| if (ExSH_flag == 1) { |  |  |
| Extended subheader} | variable bits | Extended subheader associated with the CID which needs to be sent over access link |
| padding | variable | Padding bit to ensure octet aligned |
| } |  |  |

+++++++++++++ End text ++++++++++++++++++++++++++++++++++++++++++++++++