| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On RS parameters configuration for initialization |  |
| Date Submitted | 2008-01-09 |  |
| Source(s) | Adrian Boariu, Shashikant Maheshwari, Haihong Zheng, Yousuf Saifullah, Peter Wang Nokia Siemens Networks | E-mail: adrian.boariu@nsn.com |
|  | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | This is in response for call for comments P802.16j/D2 |  |
| Abstract | Some clarifications for section 6.3.9.16.3 |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On RS parameters configuration for initialization

Adrian Boariu, et al.

NSN

# Introduction

Some clarifications are provided for the section 6.3.9.16.3

# Specification changes

[Change the section 6.3.9.16.3 as following]

During this phase, MR-BS shall determine the RS operation parameters and send an RS_Config-CMD message to configure these parameters (see 6.3.2.3.70), and start a T63 timer. The message shall contain RS mode, and may contain some parameters for proper RS operation like, preamble index, R-amble index, the allocated management CID for RS operating in local CID allocation mode, RS Frame Offset etc. The RS shall respond with the MR_Generic-ACK message to the MR-BS, and stop T62 timer. After receiving the MR_Generic-ACK message from the RS, the MR-BS and the RS shall complete the RS network entry and enter operational state, and stop T63 timer. The RS applies the configuration at the indicated Frame Number Action. Once T63 timer expired without receiving MR_Generic-ACK message from RS, the MR-BS shall retransmit the RS_Config-CMD message to the transparent RS and the non-transparent RS in the access zone and the relay zone, respectively.

A non-transparent RS should keep its synchronization by listening to the R-amble transmitted by its superordinate station as provided in RCD message (see 6.3.2.3.65). A transparent RS can keep its synchronization based on the preamble transmission of its superordinate station.

In the operational state, the non-transparent RS shall start transmitting its own frame start preamble. An RS may transmit the R-amble as per the instruction received in the RCD message (see section 8.4.6.1.1.4).

The frame number used by the RS for its transmission in any frame shall take into account the RS Frame Offset TLV. If the RS Frame Offset is not provided, then the RS shall use the same frame number as its superordinate station when it transmits in a particular frame, i.e. the RS shall consider that the RS Frame Offset is zero.

For additional configuration parameters and their usage see section 6.3.2.3.70.

[Replace Figure 102f and the caption as following indicated]

Figure 102f—Handling RS_Config-CMD first reception at a RS

[Replace Figure 102g and the caption as following indicated]

Figure 102g—Handling RS_Config-CMD retransmission at a RS

[Replace Figure 102i and the caption as following indicated]

Figure 102i—Configure operation parameters to a RS at a MR-BS

[Replace Figure 102j and the caption as following indicated]

Figure 102j—R-amble synchronization at a nontransparent RS

[Delete Figure 102h and the caption]

10.1 Global values

[Modified the following Table 583 in line 9 of page 229 by including the following entries as indicated:]

Table 583—Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| RS | T62 | Wait for RNG-RSP (abort) or RS_Config-CMD after sending RS_NBR-MEAS-REP to MR-BS | TBD | TBD | TBD |
| RS | T63 | Wait for ACK after sending RS_Config-CMD to RS | TBD | TBD | TBD |

[Change the section 6.3.9.16.4 as following]

Delete the heading on p.105 line 38 which is the same as previous section 6.3.9.16.4

Change on p.105 line 40, … is used by the MR-BS to …

Change on p.105 line 54,  … sec 6.3.27.1.