| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Tunnel Support in Relay System |  |
| Date Submitted | 2008-01-24 |  |
| Source(s) | Haihong Zheng, Yousuf Saifullah, Shashikant Maheshwari, Adrian Boariu Nokia Siemens Networks 6000 Connection Drive, Irving, TX 75019 USA | Voice: 972-894-5000 Email: Haihong.Zheng@nsn.com |
| Re: | IEEE 802.16j-06/027: “Call for Technical Proposals regarding IEEE Project P802.16j” |  |
| Abstract | This proposal clarifies the QoS aspects in relay network system. |  |
| Purpose | Discuss and adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification on Tunnel Support in Relay System

Haihong Zheng, et al.

Nokia Siemens Networks

# 1. Introduction

Tunnels are introduced in relay system. However, some aspects of tunnel operation are missing in the current specification.

After tunnel is established, individual SS connections are mapped into tunnels. The MR-BS decides which individual downlink SS connection shall get into which tunnel. However, the access RS doesn’t have the same knowledge for uplink SS connections. The MR-BS needs to inform the access RS of the mapping of SS individual connection to tunnel connection when admitting the SS service flow. This is not defined in the current standard. This contribution proposes text change to address this issue.

# 2. Specific Text Change

[Change section 6.3.2.3.14 as following:]

[Add following text in line 29 on page 31]

This DSC-REQ sent over relay link for the purpose of admission control may contain the following TLV if the admission control is applied to a tunnel, and meanwhile the included individual MS CIDs are added into or removed from the tunnel.

CIDs Added into Tunnel (see 11.13.39)

The CIDs to be added into the specified tunnel

CIDs Removed from Tunnel (see 11.13.40)

The CIDs to be removed from the specified tunnel

[Change section 6.3.14.9.3.1 as following:]

[Change line 47-50 on page 126 as following]

- If the service flow maps to an existing tunnel and the service flow parameter for the tunnel is changed, the MR-BS shall send a DSC-REQ to all the RS on the path to obtain admission control decision. The CID in the service flow parameter should be the tunnel CID.  If the tunnel is for the uplink direction, the individual SS CIDs to be added into the tunnel shall be included in the CID-Added-into-Tunnel TLV in the DSC-REQ. Only the access RS processes the CID-Added-Into-Tunnel TLV and use such information to map the individual uplink SS CID into the tunnel. Other intermediate RSs ignore it and just simply forward.

[Change section 6.3.14.9.4.1 as following:]

[Change line 7-10 on page 128 as following]

- If the service flow is mapped to a tunnel and the service flow parameters for the tunnel are changed, the MR-BS shall send a DSC-REQ to all the RS on the path to obtain admission control decision. The CID in the service flow parameter should be the tunnel CID. If the tunnel is for the uplink direction, the individual SS CIDs to be added into or removed from the tunnel shall be included in the CID-Added-into-Tunnel TLV or CID-Removed-from-Tunnel TLV in the DSC-REQ. Only the access RS processes the CID-Added-Into-Tunnel and CID-Removed-from-Tunnel TLV and uses such information to map or remove the individual uplink SS CID into from from the tunnel. Other intermediate RSs ignore them and just simply forward.

[Insert new subclause 11.13.39]

11.13.39 CIDs-Added-Into-Tunnel TLV

This field contains a compound attribute whose attributes identify the CIDs to be added into a tunnel and the number of CIDs included as listed in Table xx.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | Variable | Compound | DSA-REQ, DSC-REQ |

Table xxx – CIDs-Mapped-Into-Tunnel sub-attributes

| Attribute | Content |
| --- | --- |
| Number of CIDs | The number of CIDs to be added into a tunnel |
| List of CIDs | A list of CIDs to be added into a tunnel |

[Insert new subclause 11.13.40]

11.13.40 CIDs-Removed-from-Tunnel TLV

This field contains a compound attribute whose attributes identify the CIDs to be removed from a tunnel and the number of CIDs included as listed in Table xx.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | Variable | Compound | DSC-REQ |

Table xxx – CIDs-Mapped-Into-Tunnel sub-attributes

| Attribute | Content |
| --- | --- |
| Number of CIDs | The number of CIDs to be removed from a tunnel |
| List of CIDs | A list of CIDs to be removed from a tunnel |