| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fragmentation and packing of relay MAC PDU in MR networks |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Wei-Peng Chen Fujitsu Laboratories of America Inc. Masato Okuda Fujitsu Laboratories Ltd Shashikant Maheshwari, Yousuf Saifullah, Haihong Zheng NSN | Voice: +1 408 530 4622 E-mail: wei-peng.chen@us.fujitsu.com Voice: +81 44 754 2811 E-mail: okuda@jp.fujitsu.com E-mail: shashi.maheshwari@nsn.com |
| Re: | IEEE 802.16 WG Letter Ballot #28a |  |
| Abstract | Propose to use two extended subheaders to achieve fragmentation and packing of relay MAC PDU. |  |
| Purpose | This is a supplement to a comment entered into commentary. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Fragmentation and Packing of Relay MAC PDU in MR Networks

Wei-Peng Chen, Masato Okuda

Fujitsu Laboratories of America Inc., Fujitsu Laboratories Ltd.

# Problem Description

The packet tunnel mode defined in P802.16j/D2 enables that multiple MAC PDUs from connections that traverse the same tunnel can be concatenated into a tunnel packet for transmission. The ingress station of the tunnel encapsulates MAC PDUs into a relay MAC PDU and transmits the entire relay MAC PDU to next hop station. The next hop station simply forwards the entire relay MAC PDU to its next hop station until reaching the egress station of the tunnel. Finally the egress station removes the relay MAC header and decapsulate individual MDPs from the relay MAC PDU payload.

Scheduling of relay MAC PDUs within a tunnel across more than one hop (i.e. in >2-hops MR networks) creates more challenges compared to scheduling of MAC PDUs over one hop. The reasons include:

- We need more resource to transmit a relay MAC PDU which is usually longer than regular MAC PDUs. If relay MAC PDUs must remain intact within the tunnel, more constraints are imposed on the scheduling algorithm, which could lead to inflexibility and inefficiency of system.

- The channel condition fluctuates quickly and RSs may not be able to provide feedbacks in time to MR-BS. If channel condition degrades quickly and the granted resource based on previous bandwidth request is insufficient to transmit the whole relay MAC PDU, intermediate RS might be forced to drop the relay MAC PDU.

Therefore, it is desirable to enable fragmentation and/or packing function at relay MAC PDU level. Both scheduling flexibility and system performance could be increased with the small cost of additional overheads.  In P802.16j/D2, flags of FSH and PSH exist in the relay MAC header, which implies fragmentation and packing of relay MAC PDU are supported. However, no details of the operations of packing and fragmentation exist in P802.16j/D2. There are several problems showing that we cannot simply apply the same fragmentation and packing rules of MAC PDU on those of relay MAC PDU:

- The definition of FSH and PSH in 802.16e-2005 includes one bit, Type bit Extended Type, which does not exists in the definition of relay MAC header. Therefore, FSH and PSH cannot be reused on relay MAC PDU.

- Both FSH and PSH in 802.16e-2005 rely on two bits, FC, to identify the order of fragments belonging to one MAC PDU. Nevertheless, in the case of multi-hops transmission (>2 hops MR networks), it is insufficient to use the information of these two bits to reconstruct the relay MAC PDU from fragments because intermediate RSs do not guarantee the ordered delivery of the middle fragments of relay MAC PDU.

In this contribution, we propose to modify the definition of FSH and PSH in the relay MAC header to realize the fragmentation and packing function on relay MAC PDU. The same concept of IP fragmentation [1] is applied to avoid out-of-order problem mentioned above. Besides, we use the form of extended subheader to express both FSH and PSH. That is, extended-FSH (E-FSH) and extended-PSH (E-PSH) are used to provide  realize the fragmentation and packing function on relay MAC PDU.

# Proposed Solution

IP fragmentation [1] uses the following information to express one fragment:

- The total length of the fragment

- The more fragments (MF) flag is set for all fragments except the last one

- The fragment offset (FO) is set based on the offset of the fragment in the original data payload.

- The identification field is to specify which data gram to which the fragment belongs.

In 802.16, the total length information exists in the length field of relay MAC header to express the length information of a fragment. The MF flag only contains partial information of FC bits in 802.16; however, fragment offset provides much more information of the location of the fragment such that it is much easier to divide and reconstruct relay MAC PDU.

A simply example is provided to illustrate the fragmentation operation. Suppose the length of the concatenated MAC PDUs before encapsulation into a relay MAC PDU at ingress station is 2000 bytes. When the first intermediate RS (IRS) receives the relay MAC PDU, it divides the relay MAC PDU payload into two relay fragments, with 600 and 1400 bytes, respectively. Then two relay MAC PDUs are constructed at this IRS and E-FSH is inserted after relay MAC header and before relay fragment. The values of fragment offset in these two E-FSHs are 0 and 600, respectively.

Note that it is not necessary to enable fragmentation of a relay MAC PDU at ingress station because we can use SDU-level fragment (i.e. 16e SDU fragment) to achieve the same goal of achieving small granularity in scheduling.  Therefore, this contribution is applicable to the case when an intermediate RS exists, i.e. > 2 hops MR networks.

In conclusion, the proposed scheme is applicable to the configuration listed below:

- Non-transparent RS, (2) Distributed and centralized security mode, (3) distributed scheduling mode (4) Packet tunnel mode, and (5)  2 hops MR networks

# Text Proposal

6.3.2.2.8.3 Fragmentation subheader (FSH) in relay MAC header

FSH in relay MAC header indicates the location of a fragment of a relay MAC PDU originated from an ingress station of the thunnel. The format of fragmentation subheader in relay MAC header is specified in Table 37c.

Table 37c – Format of fragmentation subheader in relay MAC header.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| fragment subheader() { | - | - |
| More Fragment (MF) flag | 1 | MF is set for all fragments of the same TDU except the last one |
| Tunnel Data Unit Sequence Number (TSN) | 7 | The sequence number of the TDU to which the fragment belongs. |
| Fragment Offset (FO) | 12 | The offset of the fragment of a TDU compared to the beginning of the TDU |
| Reserved | 4 | Shall be set to zero |
| } | - | - |

[Insert new subclause 6.3.2.2.8.4:]

6.3.2.2.8.4 Packing subheader (PSH) in relay MAC header

PSH in relay MAC header indicates the location and the length of a fragment of a relay MAC PDU originated from an ingress station of the thunnel. The format of packing subheader in relay MAC header is specified in Table 37d.

Table 37d – Format of packing subheader in relay MAC header.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Packing subheader { | - | - |
| More Fragment (MF) flag | 1 | MF is set for all fragments of the same TDU except the last one |
| Tunnel Data Unit Sequence Number (TSN) | 7 | The sequence number of the TDU to which the fragment belongs. |
| Fragment Offset (FO) | 12 | The offset of the fragment of a TDU compared to the beginning of the TDU |
| Fragment Length | 12 | The length of the fragment |
| } | - | - |

[Insert new subclause 6.3.3.8.1.1:]

6.3.3.8.1.1  Fragmentation and packing of relay MAC PDU

In tunnel packet mode, the MAC PDUs encapsulated in a relay MAC PDU from the ingress station, called as Tunnel data unit (TDU), may be fragmented or packed. TDU is defined as a set of concatenated MPDUs to be encapsulated in a relay MAC PDU at the ingress station. TDU shall not include the extended subheaders and subheaders in a relay MAC PDU. When the function of fragmentation and packing of a relay MAC PDU are enabled, a fragmentaion subheader (FSH) specified in 6.3.2.2.8.3 is inserted in the relay MAC PDUs. An TDU sequence number (TSN in Table 37c and 37d) is assigned to each TDU. During the fragmentation or packing processes, the value of TSN remains the same for all the fragments from the same TDU.

A TDU may be fragmented at the ingress station or an intermediate RS before being encapsulated in the relay MAC PDU. When only one fragmented TDU is encapsulated in a relay MAC PDUs, a FSH in inserted before the fragmented TDU. The length of the fragmented TDU can be interpreted from the length field specified in the relay MAC header. More Fragment flag is used to indicate whether the fragment is the last fragment from the TDU. Fragment Offset is used to specify the location of the fragment relative to the beginning of the TDU. When more than one fragmented TDUs are encapsulated in a relay MAC PDU, a PSH specified in 6.3.2.2.8.4 is placed before each fragmented TDU. The fragmented TDUs packed in the same relay MAC PDU may be originated from the same or different TDU (i.e. with the same or different TDU sequence number). The length field in the PSH represents the length of the fragment.

When an intermediate RS receives multiple fragments with the same TDU sequence number from its previous hop station, it may may re-pack or re-fragment the received TDU before forwarding it to its next hop station. The intermediate RS may send a fragment of a tunnel data unit without waiting until the entire TDU is received. Alternately, an intermediate RS may reassemble the fragments with thre same TDU sequence number and contiguous offsets into one larger fragment before forwards them to the next hop station. All the fragmentation subheaders except that of the first one are removed and the values of the fields in the fragmentation subheader are updated accordingly.

The egress station reassembles the fragments with the same TDU sequence number and then retrieves individual MAC PDUs from the assembled TDU.

# Reference

[1] RFC 791, “INTERNET PROTOCOL - DARPA INTERNET PROGRAM PROTOCOL SPECIFICATION”