| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of UL ACK transmission corresponding to Retx. HARQ burst |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Youngbin Chang, Jung Je Son Samsung Electronics Rakesh Taori Samsung Advanced Institute of Technology | E-mail: yb.chang@samsung.com Voice: +82 31 279 5519 E-mail: rakesh.taori@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments on the IEEE 802.16j D2 |  |
| Abstract | In this document we propose clarification text to explain UL ACK for HARQ retransmission bursts |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j/D2 document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification of UL ACK transmission corresponding to Retx. HARQ burst

Youngbin Chang, Jung Je Son, Rakesh Taori,

# 1. Introduction

1. The current text of procedure regarding UL encoded ACK for retransmission HARQ bursts is complex to understand and not to appropriate to describe detail procedure in general sub-clause of Non-transparent RS.

Therefore, this contribution proposed that the detail procedure of UL encoded ACK for retransmission HARQ bursts is moved to another sub-clause and modified to be understandable.

# 2. Proposed Text Changes

[Change the text from line 7 to 34 on page 132, in 6.3.17.4.1 into proposed text below:]

When a transmission of an HARQ burst fails, MR-BS shall schedule retransmission of HARQ burst only on the failed link. From the link where RS didn’t transmit erroneous data, MR-BS shall schedule retransmission of HARQ burst as new HARQ data transmission. The procedure of reporting encoded ACK of retransmission bursts from subordinate station to MR-BS is described in 6.3.17.4.1.2.

[Delete the Figure 157a and Figure 157b:]

[Insert new subclause 6.3.17.4.1.2 after subclause 6.3.17.4.1.1 as below:]

6.3.17.4.1.2. HARQ Error report for retransmission bursts

To know whether an RS/MS received retransmission burst correctly, the MR-BS shall schedule additional UL ACKCH region along the path from the RS on the failed link to the MR-BS. This UL ACK region for retransmission bursts are allocated by HARQ ACKCH region allocation for data IE in 8.4.5.10.1.7.

When RS reports the encoded ACK/NAK, UL ACKCH region is divided two regions when ‘retx_region_present’ field is set to 1. The first ACKCH sub-region is for ACKs of initial transmission HARQ bursts related to DL HARQ MAP received from superodinate RS. The second ACKCH sub-region is for ACKs of retransmission HARQ bursts received from subordinated RS. The beginning slot of first sub-region is corresponding to the start of HARQ ACKCH region, and the second sub-region indicated by ‘Retx_ACHCH_offset’ field is appended at the end slot of first sub-region.

The order of RS UL ACK corresponding to each HARQ retransmission burst in second ACKCH sub-region follows the order RCID of the retransmitted bursts have been appeared in the RS HARQ DL IE or HARQ_DL-MAP IE. When an RS reports the encoded ACK/NAK of retransmission bursts to superodinate station, the RS shall locate the encoded ACK/NAK of retransmitted bursts scheduled by RS own after ACKs of received from subordinate station. The encoded ACK/NAKs in the second ACKCH region are stacked from the most far- away hop subordinate station to the least far away hop subordinate station.

The RS which retransmitted HARQ burst on the failed link shall transmit encoded ACK/NAK to superordinate station after ACK_frame_delay frames from receiving the DL HARQ MAP. Intermediate RS along the path to MR-BS when receive the encoded ACK/NAK signals included the ACKs in the second ACKCH sub-region from subordinate station forwards to superordinate RS when ACKs of in the first ACKCH sub-region are forwarded to superodinated RS.

Figures 157a and 157b show examples of the initial transmission and retransmission of an HARQ burst.

Figure 157a—Example of Initial transmission of HARQ burst

Figure 157b—Example of initial transmission and retransmission of HARQ burst