| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of UL ACK transmission corresponding to Retx. HARQ burst |  |
| Date Submitted | 2008-01-17 |  |
| Source(s) | Youngbin Chang, Jung Je Son Samsung Electronics Rakesh Taori Samsung Advanced Institute of Technology | E-mail: yb.chang@samsung.com Voice: +82 31 279 5519 E-mail: rakesh.taori@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments on the IEEE 802.16j/D2 |  |
| Abstract | In this document, we propose clarification text to explain UL ACK for HARQ retransmission bursts |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j/D2 document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of UL ACK transmission corresponding to Retx. HARQ burst

Youngbin Chang, Jung Je Son, Rakesh Taori

# 1. Introduction

The current text of procedure regarding UL encoded ACK for retransmission HARQ bursts is complex to understand and not to appropriate to describe detail procedure in general sub-clause of Non-transparent RS.

Therefore, this contribution proposed that the detail procedure of UL encoded ACK for retransmission HARQ bursts is moved to another sub-clause and modified to be understandable.

# 2. Proposed Text Changes

[Change the text from line 7 to 34 on page 132, in 6.3.17.4.1 into proposed text below:]

When a transmission of an HARQ burst fails on any link along a path, the ACK/NAK from the failed link to the MR-BS is transmitted on the UL ACKCH as scheduled. Subsequently, the MR-BS shall schedule retransmission of the HARQ burst on the failed link and the subsequent links. The ACK/NAK scheduling corresponding to the retransmission of an HARQ burst is described in 6.3.17.4.1.2

[Delete the Figure 157a and Figure 157b:]

[Insert new subclause 6.3.17.4.1.2 after subclause 6.3.17.4.1.1 as below:]

6.3.17.4.1.2. ACK/NAK scheduling for HARQ retransmission burst

The MR-BS shall schedule additional UL ACKCH region for relaying ACK/NAK, corresponding to the retransmission burst, from the subordinate stations. This additional UL ACK region is allocated when ‘retx_region_present’ is set to 1 within HARQ ACKCH region allocation for data IE in 8.4.5.10.1.7, and the UL ACKCH region is divided into two regions. The first ACKCH sub-region is for the ACK/NAKs corresponding to the HARQ bursts related to DL HARQ MAP received from the superodinate RS. The second ACKCH sub-region is for relaying the ACK/NAKs corresponding to the retransmission HARQ bursts received from the subordinated RSs. The start of the first sub-region corresponds to the start of the HARQ ACKCH region, and the start of the second sub-region is indicated by the ‘Retx_ACHCH_offset’ field.

The ACK/NAKs in the second sub-region are stacked starting with ACK/NAK of the subordinate RS that is furthest away to the nearest subordinate RS. For relaying ACK/NAK of the subordinate RSs that are same hop count away, the stacking shall be done in the order in which the RCID appears in HARQ DL MAP.

Initial and retransmitted HARQ bursts that are transmitted to a subordinate station in frame N shall be acknowledged to the superordinate station in frame N + ACK_frame_delay. All the ACK/NAK received from subordinate stations in the same ACKCH region shall be forwarded to the superordinate station in the same frame.

Figures 157a and 157b show examples of the initial transmission and retransmission of an HARQ burst.

Figure 157a—Example of Initial transmission of HARQ burst

Figure 157b—Example of initial transmission and retransmission of HARQ burst