| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On using load information in multi-hop case |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Jungmin So, Mi-Sun Do, Ki-Sun Oh Samsung Advanced Institute of Technology | Voice: +82 31 280 9625 E-mail: jungmin.so@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments and contributions on the IEEE 802.16j baseline draft (802.16j/D2). |  |
| Abstract | [Description of document contents.] |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j baseline draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On using load information in multi-hop case

Jungmin So, Mi-Sun Do and Ki-Sun Oh

Samsung Advanced Institute of Technology

Introduction to the problem

In the current TGj baseline draft (802.16j-D2), traffic load is not considered in procedures such as network entry, path selection and load balancing. In multi-hop networks, even for a single MR-BS, multiple paths can exist for an MS to reach the MR-BS. Use of load information can help MSs in selecting access station, and also help MR-BS to perform load balancing to improve performance.

In 802.16Rev2-D2, new fields are added to DCD and UCD messages, which are “Available DL Radio Resources” and “Available UL Radio Resources.” They indicate how much resource is available at BS for DL and UL. This information can be used by MS in selecting an access station which is less loaded. When it comes to a multi-hop scenario, not only load of the access station is important, but also the load of the stations that are in the path to the MR-BS (including the MR-BS itself) become important. Thus, it will be useful to share load information among MR-BS and RSs.

For example, consider the following scenario in figure 1, where an MS1 is about to select its access station. MS1 can either select RS1 and RS2, which are connected to BS1 and BS2, respectively. Suppose that currently the resource at RS1 and RS2 are fully available. Also, radio resource at BS1 is lightly loaded, whereas radio resource at BS2 is heavily loaded. It is beneficial for the MS to choose RS1 as its access station and thus choose to join BS1 cell, since the cell is less loaded.

Figure 1. A scenario where MS1 can select either RS1 or RS2 as its access station.Mntry, path selection and load balancing. es such as netwokr 0000000000000000000000000000000000000000000000000000000000000000000

In current baseline draft, if RS1 composes a DCD, it only knows the resource availability of itself, but does not know the resource situation at super-ordinate stations.

An MR-BS can also use the load information in a useful way. It could make suggestions to MS on target BSs or RSs based on the load information, or it can start handovers with the purpose of load balancing to improve cell performance.

# Suggested Remedy

The suggestions in this contribution apply only to the case of distributed scheduling. In order to share load information among MR-BS and RSs, we suggest that RSs report their load information to MR-BS, and the MR-BS provides its load information to RSs.

For MR-BS to provide propagate load information to RSs, we propose to add a TLV in the R-link Channel Descriptor (RCD) message. In order for RSs to report their load information to the MR-BS, the RSs need to a way to signal MR-BS on its load situation. Since the Extended MAC signaling header type II is currently full, this must be done using a separate message, unless we further extend the header to add more types of signaling.

In the “Propose Text” section, we propose the new field in the R-link Channel Descriptor (RCD) message, and leave the way of signaling RSs’ load information to BS for a discussion.

# Proposed Text

Table 7a – Relay MAC PDU header

Insert a new subclause 11.25.7

11.25.7 Load Metric

This field is used by an MR-BS to inform load metric to RS.