| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On using load information in multi-hop case |  |
| Date Submitted | 2008-01-14 |  |
| Source(s) | Jungmin So, Mi-Sun Do, Ki-Sun Oh Samsung Advanced Institute of Technology | Voice: +82 31 280 9625 E-mail: jungmin.so@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments and contributions on the IEEE 802.16j baseline draft (802.16j/D2). |  |
| Abstract | [Description of document contents.] |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j baseline draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On using load information in multi-hop case

Jungmin So, Mi-Sun Do and Ki-Sun Oh

Samsung Advanced Institute of Technology

# Introduction to the problem

In this contribution, we only consider the distributed scheduling case.

In the current TGj baseline draft (802.16j-D2), traffic load is not considered in procedures such as network entry, path selection and load balancing. In multi-hop networks, even for a single MR-BS, multiple paths can exist for an MS to reach the MR-BS. Use of load information can help MSs in selecting access station, and also help MR-BS to perform load balancing in order to improve performance.

In 802.16Rev2-D2, new fields are added to DCD and UCD messages, which are “Available DL Radio Resources” and “Available UL Radio Resources.” They indicate how much resource is available at BS for DL and UL. This information can be used by MSs in selecting an access station which is less loaded. When it comes to a multi-hop network, RSs can also use this field to advertise its resource availability to MSs that are within its reach.

However, in a multi-hop scenario, resource availability of a “path” may be more important than the resource availability of only the access station. In other words, to consider resource availability in selecting path, all nodes on the path from the access RS to the MR-BS must be considered.

For example, consider the following scenario in figure 1, where an MS1 is about to select its access station. MS1 can either select RS1 and RS2, which are connected to BS1 and BS2, respectively. Suppose that currently the resource at RS1 and RS2 are fully available. Also, radio resource at BS1 is lightly loaded, whereas radio resource at BS2 is heavily loaded. It is beneficial for the MS to choose RS1 as its access station and thus choose to join BS1 cell, since the cell is less loaded. If RS1 and RS2 only advertise their own resource availability, MS cannot know that the path via RS1 is better in terms of resource availability.

Figure 1. A scenario where MS1 can select either RS1 or RS2 as its access station.Mntry, path selection and load balancing. es such as netwokr 0000000000000000000000000000000000000000000000000000000000000000000

In the current baseline draft, RS1 (or RS2) can only advertise its own resource availability using DCD and UCD, since it does not know the resource availability at the BS (or superordinate RSs).

# Suggested Remedy

Note again that the suggestion here only apply to the case of distributed scheduling.

If an RS has information on resource availability of superordinate stations, it can advertise a more meaningful value in the “available DL radio resources” or “available UL radio resources” fields. (An RS may also use other means to advertise the path load.)

So we need a way for superordinate stations to indicate their resource availability for subordinate RSs to know. Since 802.16Rev2-D2 has included “available DL radio resources” and “available UL radio resources” in DCD and UCD messages, it nicely fits to add fields on DL and UL radio resources in the R-link Channel Descriptor (RCD) message.

Since an RS only receives RCD from its direct superordinate station, it cannot separately receive this information from each of its superordinate stations along the path to the MR-BS. However, as the resource availability information is passed down from MR-BS to an RS, it can be processed by the intermediate RSs so that it contains meaningful information on the resource availability of the path from the MR-BS to the RS receiving the RCD message.

How to compute, process, and interpret the value in the suggested fields is out of scope of this specification.

# Proposed Text

Insert a new subclause 11.25.7

11.25.7. Available radio resources

This field is used by an MR-BS to inform load metric to RS.