| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The timing of RS transmission allocation start time |  |
| Date Submitted | 2008-01-15 |  |
| Source(s) | Junhong Hui, Young-il, Kim, Suchang Chae ETRI | E-mail: junhonghui@etri.re.kr |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | A proposal for RS timing of transmission allocation start time |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The timing of relay station transmission start time

Junhong Hui, Young-il Kim, Suchang Cha,

ETRI

# Introduction

Relay UL allocation start time indicates the effective start time of the uplink allocation defined by the RMAP on R-link. If the effective start time is defined as 0, the uplink allocation defined by the RMAP is effective in the current frame; if the value is set to N, the uplink allocation defined by the R-MAP in frame i is effective in frame i + N.

As defined in the D2 section 11.25.1, the UL relay allocation start time aiming at the timing of transmission in UL relay link after receiving data from the UL access link for non-transparent RS (Because in the definition of relay UL allocation start time, it specifically mentions about the RMAP on R-link) is transmitted from MR-BS to configure one or all RSs.

But the same problem also remains for other link connection conditions, like the UL relay allocation start time with the timing of transmission in UL relay link after receiving data from the UL relay link for non-transparent RS in a more than 2 hop MMR system. Similarly, in the DL relay allocation start time with the timing of transmission in DL relay link after receiving data from the DL relay link for non-transparent RS in a more than 2 hop MMR system or in the DL relay allocation start time with the timing of transmission in DL access link after receiving data from the DL relay link in both transparent RS and non-transparent RS assisted system.

As above, we need to signal such timing information from MR-BS to the RSs both in DL and UL in consideration of the following situations:

- In the transparent RS system, MR-BS need to inform the RS when to transmit the data received in the access zone in the current frame in order to match the DL-MAP allocation which is going to be received by the MS directly at that time. For example, if direct relay zone is used, the received data will be transmitted in the same frame. While in other cases, it may not be possible for the RS to finish the whole processing including decoding, demodulation, upper layer processing, encoding and modulation, within one frame, such a delay of allocation start time for the RS access link transmission should be informed to the RS. Such a timing value for the relay transmission start time should not only consider the decoding delay but also the other processing time like upper layer control messages and etc.

- In the Non-transparent RS system, in case of a large frame size like 20 ms, there might be multiple relay zones in one frame. MR-BS might want to let RS receive data in the earlier relay zone and transmit such data in another later relay zone within the same frame in order to reduce the delay.

- If we consider the multi-hop case, it may be required to utilize the adaptive modulation and coding, this will cause a big and frequent change of such timing information in the relay link, RS or MRBS station may need to transmit such timing information in a timely manner by some R-link specific IE to the receiving RS.

If this timing information needs not to be changed frequently, a field for generic channel description in RCD-TLV encoding could be adopted instead of the R-link specific IE.

- For distributed scheduling of more than 2 hop cases, to make RS configure and schedule its own access zone and relay zone according to the possible changing of the traffic ratio between access zone and relay zone in order to utilize the bandwidth in a more efficient way, such timing information for both the RS access link and relay link may be further granulated into symbol unit and should be informed to the receiving RS.

Based on these considerations, we propose to modify the draft as the text proposal hereafter.

# Specification changes

Option1:

[Insert this field into the section 11.25 Generic channel description as following]

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Relay UL allocation start time of received access zone data for operational RS | 8 |  | Indicates the effective start time of the uplink allocation of received access zone data | RCD |
| Relay DL allocation start time for access zone for operational RS | 9 |  | Indicates the effective start time of the downlink allocation for access zone | RCD |
| Relay DL allocation start time for relay zone for operational RS | 10 |  | Indicates the effective start time of the downlink allocation for relay zone | RCD |

[Modify this field in the section 11.25 Generic channel description as following]

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Relay UL allocation start time of received relay zone data for operational RS | 7 |  | Indicates the effective start time of the uplink allocation defined by the R-MAP on R-link of received relay zone data | RCD |

Relay UL allocation start time of received relay zone data

Relay UL allocation start time of received relay zone data indicates the effective start time of the uplink allocation defined by the R-MAP on R-link of received relay zone data. If the effective start time is defined as 0, the uplink allocation defined by the R-MAP of received relay zone data is effective in the current frame; if the value is set to N, the uplink allocation defined by the R-MAP of received relay zone data in frame i is effective in frame i + N.

Relay UL allocation start time of received access zone data

Relay UL allocation start time of received access zone data indicates the effective start time of the uplink allocation of received relay zone data. If the effective start time is defined as 0, the uplink allocation of received access zone data is effective in the current frame; if the value is set to N, the uplink allocation of received access zone data in frame i is effective in frame i + N.

Relay DL allocation start time for access zone

Relay DL allocation start time for access zone indicates the effective start time of the downlink allocation for access zone. If the effective start time is defined as 0, the downlink allocation for access zone is effective in the current frame; if the value is set to N, the downlink allocation for access zone in frame i is effective in frame i + N.

Relay DL allocation start time for relay zone

Relay DL allocation start time for relay zone indicates the effective start time of the downlink allocation for relay zone. If the effective start time is defined as 0, the downlink allocation for relay zone is effective in the current frame; if the value is set to N, the downlink allocation for relay zone in frame i is effective in frame i + N.

Option 2:

[Modify Table 496c in 8.4.5.10.1]

Table 496c — R-link specific IE types

| Type | Usage |
| --- | --- |
| 09-1F | Reserved |
| 09 | Relay station allocation start time IE |
| 10-1F | Reserved |

[Insert following text before 8.4.6]

8.4.5.10.1.10 RS allocation start time message

Transmitted from MRBS or RS to the operational RS, the RS allocation start time IE indicates the timing information of relay station transmission allocation start time related to the downlink and uplink received data.

Table XX— RS allocation start time IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| RS allocation start time IE format(){ |  | RS allocation start time IE = 0x09 |
| Type | 5 bits |  |
| Length | 8 bits | variable |
| If (occur==1) { |  | If exists |
| Relay UL allocation start time of received relay zone data | 4 bits | variable |
| } |  |  |
| If (occur==1) { |  | If exists |
| Relay UL allocation start time of received access zone data | 4 bits | variable |
| } |  |  |
| If (occur==1) { |  | If exists |
| Relay DL allocation start time for access zone | 4 bits | variable |
| } |  |  |
| If (occur==1) { |  | If exists |
| Relay DL allocation start time for relay zone | 4 bits | Variable |
| } |  |  |
| } |  |  |

Relay UL allocation start time of received relay zone data

Relay UL allocation start time of received relay zone data indicates the effective start time of the uplink allocation of received relay zone data. If the effective start time is defined as 0, the uplink allocation defined by of received relay zone data is effective in the current frame; if the value is set to N, the uplink allocation of received relay zone data in frame i is effective in frame i + N.

Relay UL allocation start time of received access zone data

Relay UL allocation start time of received access zone data indicates the effective start time of the uplink allocation of received relay zone data. If the effective start time is defined as 0, the uplink allocation of received access zone data is effective in the current frame; if the value is set to N, the uplink allocation of received access zone data in frame i is effective in frame i + N.

Relay DL allocation start time for access zone

Relay DL allocation start time for access zone indicates the effective start time of the downlink allocation for access zone. If the effective start time is defined as 0, the downlink allocation for access zone is effective in the current frame; if the value is set to N, the downlink allocation for access zone in frame i is effective in frame i + N.

Relay DL allocation start time for relay zone

Relay DL allocation start time for relay zone indicates the effective start time of the downlink allocation for relay zone. If the effective start time is defined as 0, the downlink allocation for relay zone is effective in the current frame; if the value is set to N, the downlink allocation for relay zone in frame i is effective in frame i + N.

[modify this field in the section 11.25 Generic channel description as following]

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Relay UL allocation start time for operational RS | 7 |  | Indicates the effective start time of the uplink allocation defined by the R-MAP on R-link | RCD |

Relay UL allocation start time

Relay UL allocation start time indicates the effective start time of the uplink allocation defined by the R-MAP on R-link. If the effective start time is defined as 0, the uplink allocation defined by the R-MAP is effective in the current frame; if the value is set to N, the uplink allocation defined by the R-MAP in frame i is effective in frame i + N.