| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications on RS_Member_List_Update Message |  |
| Date Submitted | 2008-01-15 |  |
| Source(s) | Junhong Hui, Young-il, Kim, Suchang Chae ETRI | E-mail: junhonghui@etri.re.kr |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | Modifications on the RS_Member_List_Update Message. |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications on RS_Member_List_Update Message

Junhong Hui, Young-il Kim, Suchang Chae,

ETRI

# Introduction

Some modifications of RS_Member_List_Update Message are proposed in order to reduce the unnecessary overhead.

# Specification changes

[Modify the following subclause in 6.3.2.3.86 as following]

This message may be transmitted when the group parent decides to update the connection list for RS group members. whenever there is a change in the connection list of the RS group members . After receiving this message, RS shall respond with MR_Generic-ACK message or MR Acknowledgement Header for confirmation such a receipt .

Table 183y—RS_Member_List_Update message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_Member_List_Update_Message_Form at { |  |  |
| Management Message Type = 93 | 8 bits |  |
| Frame Action Number | 4 bits | The 4-bits LSB of the frame number at the superordinate station when the configuration shall be applied by all RS group members |
| Configured_para_type | 4 bits | b0 = 1: RS sends acknowledgement if the message is received b0 = 0: RS does not send acknowledgment if the message is received b1 = 1: data forwarding on a per MS CID basis b1 = 0: RSs in the group shall forward all data b2-b3: reserved |
| If (b0 of Configured_para_type == 1 ) { |  |  |
| Transaction ID | 16 bits |  |
| } |  |  |
| If (b1 of Configured_para_type == 1 ) { |  |  |
| if ( unicast message selected) go to point A |  |  |
| if ( multicast message selected) { |  |  |
| Number_RS | 5 bits | Number of RS group members involved with the CID list update |
| Reserved | 3 bits |  |
| for (i=0;i<Number_RS;i++) { |  |  |
| CID | 16 bits | Basic CID of the RS group member |
| A: |  |  |
| if ( mode 1 is selected) { |  |  |
| N_CID_Add_designated | 8 bits | Number of connections for which the RS is the designated RS. If Mode 2 is used this value is zero. |
| N_CID_Add_non_designated | 8 bits | Number of connections for which the RS is a non-designated RS. |
| for (j=0;j<N_CID_Add_designated;j++) { |  |  |
| CID | 16 bits | For this basic CID of MS the RS is designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a designated RS for this CID and shall forward data and measurements. |
| } |  |  |
| for(j=0;j<N_CID_Add_non_designated; j++){ |  |  |
| CID | 16 bits | For this basic CID of the MS the RS is non-designated. Irrespective of the current RS assignment for a given CID, the indicated RS would be a non-designated RS for this CID and shall forward only traffic data. |
| } |  |  |
| N_CID_ | 8 bits | Number of MS connection information need to update |
| For (j=0;j<N_CID; j++) { |  |  |
| CID | 16 bits | Basic CID of MS |
| } |  |  |
| } |  |  |
| if ( mode 2 is selected) { |  |  |
| N_CID | 8 bits |  |
| For (j=0;j<N_CID; j++) { |  |  |
| CID | 16 bits | Basic CID of MS |
| ADD_Remove configuration | 1bit | b0=1: Add the connection of MS to the RS b0=0: Delete the connection of MS from the RS |
| } |  |  |
| } |  |  |
| If ( multicast message selected) } |  |  |
| } |  |  |
| } |  |  |
| TLV Encoded Information | variable | TLV specific |
| } |  |  |