| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Access Messages for Direct Signaling in the AAS Relay Zone. |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Dale Branlund, Matt Volpe, Will Sun BRN Phoenix Inc. 2500 Augustine Drive Santa Clara, CA, USA 95054 John Norin, Robert Popoli The DIRECTV Group, Inc. 2250 East Imperial Hwy El Segundo, CA 90245 | Voice: +1-408-572-9703 Fax: +1-408-351-4911 dbranlund@brnphoenix.com Voice: +1-310-964-0717 Fax: +1-310-535-5422 john.norin@directv.com |
| Re: | Working Group Letter Ballot Recirc #28a, Technical Comments and Contributions regarding IEEE Project P802.16j; Draft Amendment P802.16j/D2. |  |
| Abstract | This contribution describes the Direct Signaling messages transmitted in the access channel of the AAS Relay Zone to accomplish ranging and bandwidth request and grant. |  |
| Purpose | This document provides the necessary access messaging to properly accomplish ranging and bandwidth request/grant within the AAS Relay Zone for Direct Signaling mode of operation. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Access Messages for Direct Signaling in the AAS Relay Zone

# Dale Branlund, Will Sun, Matt Volpe, BRN Phoenix, Santa Clara, CA, USA;
John Norin, Robert Popoli, The DIRECTV Group, Inc., El Segundo, CA, USA

This document describes the access messages required to support bandwidth request/grant, ranging, frequency, and power adjustment in Direct Signaling mode of operation within the AAS Relay Zone.

# Background

Direct Signaling operation within the relay zone provides a bandwidth request mechanism that can scale with an M-fold increase in the number of users afforded by muti-user beamforming.

Section 8.4.4.7.2.3 of Draft P802.16j/D2 describes the AAS Relay Zone access channel but does not detail the access messaging required for managing channel descriptor request, initial bandwidth request/grant, codeword assignment and range/frequency/power adjustment.

# Proposed Solution

The proposed solution is to describe the Reverse Link bandwidth request/grant mechanism and the required Direct Signaling Access Messages to accomplish it within the AAS Relay Zone.

# Detailed Solution

## Reverse Link Payload (RLP) Message

The RLP is used to request bandwidth for ranging via the RLP purpose, request bandwidth for transport via the RLP purpose, transport CID, and queue depth, or to

request the compact or full channel descriptor (as described in 8.4.4.7.2.3.3) via the RLP purpose.

The following table defines the AAS_RLP Message structure:

.

Table 1: Structure of AAS_RLP()

| RLP Component | Bits | Reps | Slot | Comment |
| --- | --- | --- | --- | --- |
| AAS_RLP Message { |  |  |  |  |
| CID | 16 | 1 |  | Transport CID |
| Partition Exploratory Map | 24 | 1 |  | Sized for 24 partitions (i.e 10 MHz) |
| Probe Codeword index | 2 | 1 |  | Probe Codeword index |
| Queue Depth | 2 | 1 |  | Sized for 4 levels |
| RLA Purpose | 4 | 1 |  | value 0=Transport BW Request 1=Ranging BW Request 2=Compact Chan Description Req 3=Full Chan Description Req |
| Reserve | 40 | 1 |  | Reserved for Future use |
| CRC | 8 | 1 |  | CRC-8 |
| } |  |  |  |  |
| Total RLP Bits | 96 |  | 2 |  |

- CID - Transport CID for which a bandwidth request is being made

- PEM - Partition Exploratory Map. One bit per partition - sized for 24 partitions (i.e 10 MHz)

- Probe Codeword Index – partition probe codeword

- Qdepth - Qdepth level of the transport CID that is requesting bandwidth. Four levels can be described.

- RLA Purpose

- The pupose for which the asynchronous access is being requested, including:

    - 0x0 = Transport Bandwidth Request

    - 0x1 = Ranging Bandwidth Request

    - 0x2 = Compact Channel Description Request as described in section 8.4.4.7.2.3.3

    - 0x3 = Full Channel Description Request as described in section 8.4.4.7.2.3.3

- CRC - CRC-8 covers both slots

## Forward Link Payload (FLP) Message

The FLP provides code word assignments during initial ranging and provides bandwidth grant,  codeword assignment, supportable MCS for UL/DL, and range time/frequency/power adjustment during the bandwidth grant exchange.

The following table defines the AAS_FLP Message structure:

Table 2: Structure of AAS_FLP()

| FLP Component | Bits | Reps | Slot | Comment |
| --- | --- | --- | --- | --- |
| AAS_FLP Message { |  |  |  |  |
| PB_Ctl_0 | 7 | 1 |  | Partition/Burst control described in Table2 |
| PB_Ctl_1 | 7 | 1 |  | Partition/Burst control described in Table2 |
| PB_Ctl_2 | 7 | 1 |  | Partition/Burst control described in Table2 |
| DIUC | 3 | 1 |  | DL MCS level |
| PB_Ctl_3 | 7 | 1 |  | Partition/Burst control described in Table2 |
| PB_Ctl_4 | 7 | 1 |  | Partition/Burst control described in Table2 |
| PB_Ctl_5 | 7 | 1 |  | Partition/Burst control described in Table2 |
| Codeword Type | 1 | 1 |  | 0=access, 1=transport |
| UIUC Offset | 2 | 1 |  | UL MCS offset from the DIUC index |
| Reserved | 6 | 1 |  |  |
| UL Range Adjust | 2 | 1 |  |  |
| UL Frequency Adjust | 2 | 1 |  |  |
| UL Access PwrCtrlt | 2 | 1 |  | 2 bits/partition |
| UL PwrCtl | 12 | 1 |  | 2 bits/partition |
| Assigned Codeword Index | 16 | 1 |  |  |
| CRC | 8 | 1 |  |  |
| } |  |  |  |  |
| Total FLP Bits | 96 |  | 2 |  |

- PB_Ctl_x (7 bit encode) – The partition/burst control encode is described in Table 3 below:

Table 3: Partition/Burst Control encode

| 5 bit Value | 2 bit encode value | Description |
| --- | --- | --- |
| 0-23 | 0-3 (burst index) | Probe, Open or Maintain (p,o,m) the partition indexed by the 5 bit value in the burst indexed by the 2 bit encode value. |
| 24 | 0-3 (partition index) | Close Partition in the 4 partition range 0-3 indexed by the 2 bit encode value |
| 25 | 0-3 (partition index) | Close Partition in the 4 partition range 4-7 indexed by the 2 bit encode value |
| 26 | 0-3 (partition index) | Close Partition in the 4 partition range 8-11 indexed by the 2 bit encode value |
| 27 | 0-3 (partition index) | Close Partition in the 4 partition range 12-15 indexed by the 2 bit encode value |
| 28 | 0-3 (partition index) | Close Partition in the 4 partition range 16-19 indexed by the 2 bit encode value |
| 29 | 0-3 (partition index) | Close Partition in the 4 partition range 20-23 indexed by the 2 bit encode value |
| 30 | 0-3 (burst index) | Close all partitions in burst using 2 bit encode as burst number |
| 31 | 0-3 | 0: Fast power control for MCS increase (affects all partitions in current burst) 1-3: Reserved |

- DIUC (3 bit encode) – DL MCS, one of eight defined MCS levels

- Codeword Type - Describes the response type (assigning RLAor RLT). 0 = Access codeword 1=Transport codeword

- UIUC Offset (2 bit encode) – UL Differential MCS equals the number of MCS levels below the DL MCS

- UL Time (Range) Adjust (2 bit encode) - A step size of dt  = 0.714 us is proposed.  0.714 us corresponds to 4 samples at a 5.6 MHz sample rate.  Four samples corresponds to a “PS”.  The range adjustment encode is described in Table 4 below:

Table 4: UL Range (Time) Adjustment Encoding

| Encoded bits | Time Delay @ BS | SS UL Adjustment |
| --- | --- | --- |
| 00 | te < -t | Delay |
| 01 | -t ≤ te < 0 | Delay |
| 10 | 0 ≤ te < t | Advance |
| 11 | t ≤ te | Advance |

- UL Frequency Adjust (2 bit encode) – The frequency adjustment encode is described in Table 5 below:

Table 5: UL Frequency Adjustment Encoding

| Encoded bits | Frequency Error @ BS | SS UL Adjustment |
| --- | --- | --- |
| 00 | fe < -df | increase frequency |
| 01 | -df ≤ fe < 0 | increase frequency |
| 10 | 0 ≤ fe < df | decrease frequency |
| 11 | df ≤ fe | decrease frequency |

- UL Power Adjust (Traffic Ps) (2 bit encode/partition) – 4 levels of adjustment per partition

- UL Power Adjust (Access P) (2 bit encode) – 4 levels of adjustment

## Forward Link Initiation (FLI) Signaling

### BS Initiated Traffic and Paging

The base station can initiate traffic or page the SS using the FLI mechanism. The FLI is a unique code provided to the SS registered with the base station.  SS continuously search for the FLI using a correlation process using the assigned code as the paradigm.  One FLI allocation is included in the MAP in the last slot of symbols 2 and 3.  The FLI(i) is the paging mechanism informing the ith user to respond on both the access partition and available traffic partitions.  FLIs start BS initiated traffic. 200 FLIs per second are available.  SS(i) responds to the FLI on the access and traffic partitions with an RLA training sequence and an RLP.

### FLI Codeword Transmission

The Map is constructed to fit into 15 PUSC slots available in symbols 2 and 3.  4 slots are dedicated to the FCH, either 8 or 10 slots are reserved for the Map, and 0 or 2 slots are allocated for broadcast messages such as the DCD/UCD message or the Neighbor Advertisement message.  The remaining slot is allocated to the Forward Link Initiation (FLI) paging signal.

There are 48 sub-carriers allocated to the FLI.  The 48 sub-carriers are split into two groups of 24.  If the 48 were numbered sequentially, the first group would be the even sub-carriers and the second group would be the odd subcarriers..  Each group contains a column of a 24 × 24 Hadamard matrix and is BPSK modulated.  BPSK has two values.  Two values times 24 different columns is 48 total combinations per group.  48 combinations in the first group times 48 combinations in the second group is 2304 total.  The BS can ring up 2304 SS per 5 MHz channel.

The signal processing for the transmitter is shown in Figure 1.  b represents the FLI codeword and ranges from 0 to 2303.  It is then broken down into two groups of 24.  i0 selects one of 24 Hadamard columns for the first group and ranges from 0 to 23.  k0 selects the BPSK modulation for the first group.  i1 selects one of 24 Hadamard columns for the second group.  k1 selects the BPSK modulation for the second group.  The Hadamard matrix is based on the Matlab function: H = hadamard(24).  The first group of 24 sub-carriers is multiplexed with the second group of 24 sub-carriers.  If the indices to the sub-carriers of a slot were numbered from 0 to 47, the even numbered sub-carriers come from the first group of 24 and the odd number sub-carriers come from the second group of 24.  The first sub-carrier of the slot is H(0, i0) k0, the second, is H(0, i1) k1, the third is H(1, i0) k0, the fourth is H(1, i1) k1, etc., where zero-based indexing is assumed when accessing the Hadamard matrix, H(n, i).

Find i0

k0

i1

k1

Select Hadamard Column

vector, length = 24

24 even subcarriers

b

Select Hadamard Column

vector, length = 24

24 odd subcarriers

Multi-plex:

even, odd, even, odd, …

48 subcarriers

Figure 1.  Transmitter Signal Processing

g0=b÷48	(1)

g1=b−48g0	(2)

k0=−1if g0<24+1otherwise	(3)

k1=−1if g1<24+1otherwise	(4)

i0=g0−12(1+k0)	(5)

i1=g1−12(1+k1)	(6)

## Proposed Text Changes

Insert new subclause 8.4.4.7.2.3.4:

8.4.4.7.2.3.4 AAS Relay Zone Access Channel Messaging

Insert new subclause 8.4.4.7.2.3.4.1:

8.4.4.7.2.3.4.1 Reverse Link Payload Message

The RLP is used to request bandwidth for ranging via the RLP purpose, request bandwidth for transport via the RLP purpose, transport CID, and queue depth, or to

request the compact or full channel descriptor (as described in 8.4.4.7.2.3.3) via the RLP purpose.

Insert  Table 270a (.16e)/Table 386a (Rev2) as indicated:

Table xxx—Structure of AAS_RLP()

| Syntax | Size | Notes |
| --- | --- | --- |
| AAS_RLP() { |  |  |
| CID | 16 bits | Transport CID |
| PEM | 24 bits | Partition Exploratory Map |
| Probe Codeword Index | 2 bits | Partition Probe codeword |
| Qdepth | 2 bits | Qdepth level 0b00 = level1 0b01 = level2 0b10 = level3 0b11 = level4 |
| ##### RLA Purpose | 4 bits | 0x0 = Transport Bandwidth Request 0x1 = Ranging Bandwidth Request 0x2 = Compact Channel Description Request 0x3 = Full Channel Description Request 0x4-0xF = Reserved |
| Reserved | 40 bits | Shall be set to zero |
| CRC | 8 bits | CRC-8 |
| } |  |  |

Insert the following parameter descriptions following Table 270a (.16e)/Table 386a (Rev2)  as indicated:

### CID

Transport CID for which a bandwidth request is being made

PEM

Partition Exploratory Map. One bit per partition - sized for 24 partitions (i.e 10 MHz)

Partition Probe Codeword Index

Partition Probe Codeword

#### Qdepth

Qdepth level of the transport CID that is requesting bandwidth. Four levels can be described.

#### RLA Purpose

The pupose for which the asynchronous access is being requested, including:

0x0 = Transport Bandwidth Request

0x1 = Ranging Bandwidth Request

0x2 = Compact Channel Description Request as described in section 8.4.4.7.2.3.3

0x3 = Full Channel Description Request as described in section 8.4.4.7.2.3.3

#### CRC

CRC-8 covers both slots

Insert new subclause 8.4.4.7.2.3.4.2:

8.4.4.7.2.3.4.2 Forward Link Payload Message

The FLP provides code word assignments during initial ranging and provides bandwidth grant,  codeword assignment, supportable MCS for UL/DL, and range time/frequency/power adjustment during the bandwidth grant exchange.

Insert  Table 270b (.16e)/Table 386b (Rev2) as indicated:

Table xxx—Structure of AAS_FLP()

| Syntax | Size | Notes |
| --- | --- | --- |
| AAS_FLP() { |  |  |
| PB_Ctl_0 | 7 bits | partition control structures as described in Table xxx |
| PB_Ctl_1 | 7 bits | partition control structures as described in Table xxx |
| PB_Ctl_2 | 7 bits | partition control structures as described in Table xxx |
| DIUC | 3 bits | DL MCS |
| PB_Ctl_3 | 7 bits | partition control structures as described in Table xxx |
| PB_Ctl_4 | 7 bits | partition control structures as described in Table xxx |
| PB_Ctl_5 | 7 bits | partition control structures as described in Table xxx |
| Codeword Type | 1 bit | 0 = Access codeword 1=Transport codeword |
| UIUC offset | 2 bits | UL MCS differential offset from the DIUC index |
| Reserved | 6 bits | Shall be set to zero |
| UL Range Adjust | 2 bits | Encoded step adjustments |
| UL Frequency Adjust | 2 bits | Encoded step adjustments |
| UL Access Power Adjust | 2 bits | Encoded step adjustments |
| UL Partition Power Adjust | 12 bits | 2 bits encoded adjustment per described partition |
| Assigned Codeword Index | 16 bits |  |
| CRC | 8 bits | CRC-8 |
| } |  |  |

Insert the following parameter descriptions following Table 270b (.16e)/Table 386b (Rev2)  as indicated:

#### PB_Ctl_x

Partition control structures with 7 bits of control per partition as described by table xxxx below

### DIUC

### DIUC

#### Codeword Type

Describes the response type (assigning RLAor RLT). 0 = Access codeword 1=Transport codeword

### UIUC Offset

Offset from the DIUC

#### UL Range Adjust

Range Adjustment encode.

#### UL Frequency Adjust

Frequency Adjustment encode.

#### UL Access Power Adjust

Power adjustment for the access channel

#### UL Partition Power Adjust

Power adjustment for bearer partitions (2 bit encode per partition described in partition control structures)

#### Assigned Codeword Index

#### Assigned codeword (may be RLA or RLT)

#### CRC

CRC-8 covers both slots

Insert new subclause 8.4.4.7.2.3.5:

8.4.4.7.2.3.5 AAS Relay Zone Forward Link Initiation (FLI) Signaling

Insert new subclause 8.4.4.7.2.3.5.1:

8.4.4.7.2.3.5.1 BS Initiated Traffic and Paging

The base station can initiate traffic or page the SS using the FLI mechanism. The FLI is a unique code provided to the SS registered with the base station.  SS continuously search for the FLI using a correlation process using the assigned code as the paradigm.  One FLI allocation is included in the MAP in the last slot of symbols 2 and 3.  The FLI(i) is the paging mechanism informing the ith user to respond on both the access partition and available traffic partitions.  FLIs start BS initiated traffic. 200 FLIs per second are available.  SS(i) responds to the FLI on the access and traffic partitions with an RLA training sequence and an RLP.

Insert new subclause 8.4.4.7.2.3.5.2:

8.4.4.7.2.3.5.2 FLI Codeword Transmission

The Map is constructed to fit into 15 PUSC slots available in symbols 2 and 3.  4 slots are dedicated to the FCH, either 8 or 10 slots are reserved for the Map, and 0 or 2 slots are allocated for broadcast messages such as the DCD/UCD message or the Neighbor Advertisement message.  The remaining slot is allocated to the Forward Link Initiation (FLI) paging signal.

There are 48 sub-carriers allocated to the FLI.  The 48 sub-carriers are split into two groups of 24.  If the 48 were numbered sequentially, the first group would be the even sub-carriers and the second group would be the odd subcarriers..  Each group contains a column of a 24 × 24 Hadamard matrix and is BPSK modulated.  BPSK has two values.  Two values times 24 different columns is 48 total combinations per group.  48 combinations in the first group times 48 combinations in the second group is 2304 total.  The BS can ring up 2304 SS per 5 MHz channel.

The signal processing for the transmitter is shown in Figure 1.  b represents the FLI codeword and ranges from 0 to 2303.  It is then broken down into two groups of 24.  i0 selects one of 24 Hadamard columns for the first group and ranges from 0 to 23.  k0 selects the BPSK modulation for the first group.  i1 selects one of 24 Hadamard columns for the second group.  k1 selects the BPSK modulation for the second group.  The Hadamard matrix is based on the Matlab function: H = hadamard(24).  The first group of 24 sub-carriers is multiplexed with the second group of 24 sub-carriers.  If the indices to the sub-carriers of a slot were numbered from 0 to 47, the even numbered sub-carriers come from the first group of 24 and the odd number sub-carriers come from the second group of 24.  The first sub-carrier of the slot is H(0, i0) k0, the second, is H(0, i1) k1, the third is H(1, i0) k0, the fourth is H(1, i1) k1, etc., where zero-based indexing is assumed when accessing the Hadamard matrix, H(n, i).

Insert Figure xxxa (.16e)/Figure xxxa (Rev2) as indicated:

Find i0

k0

i1

k1

Select Hadamard Column

vector, length = 24

24 even subcarriers

b

Select Hadamard Column

vector, length = 24

24 odd subcarriers

Multi-plex:

even, odd, even, odd, …

48 subcarriers

Figure 2.  Transmitter Signal Processing

g0=b÷48	(1)

g1=b−48g0	(2)

k0=−1if g0<24+1otherwise	(3)

k1=−1if g1<24+1otherwise	(4)

i0=g0−12(1+k0)	(5)

i1=g1−12(1+k1)	(6)