| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Private Map Message and CQICH Direct Signaling in the AAS Relay Zone. |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Dale Branlund, Matt Volpe, Will Sun BRN Phoenix Inc. 2500 Augustine Drive Santa Clara, CA, USA 95054 John Norin, Robert Popoli The DIRECTV Group, Inc. 2250 East Imperial Hwy El Segundo, CA 90245 | Voice: +1-408-572-9703 Fax: +1-408-351-4911 dbranlund@brnphoenix.com Voice: +1-310-964-0717 Fax: +1-310-535-5422 john.norin@directv.com |
| Re: | Working Group Letter Ballot Recirc #28a, Technical Comments and Contributions regarding IEEE Project P802.16j; Draft Amendment P802.16j/D2. |  |
| Abstract | This contribution describes the Direct Signaling Private Map message transmitted in the AAS Relay Zone to accomplish bandwidth grant, physical link adjustment and fast HARQ feedback. |  |
| Purpose | This document provides the necessary messaging to properly accomplish bandwidth grant and HARQ control within the AAS Relay Zone for Direct Signaling mode of operation. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Private Map Message and CQICH Direct Signaling in the AAS Relay Zone

# Dale Branlund, Will Sun, Matt Volpe, BRN Phoenix, Santa Clara, CA, USA;
John Norin, Robert Popoli, The DIRECTV Group, Inc., El Segundo, CA, USA

This document provides the necessary messaging to properly accomplish bandwidth grant and HARQ control within the AAS Relay Zone for Direct Signaling mode of operation.

# Background

Direct Signaling operation within the relay zone provides a bandwidth request mechanism that can scale with an M-fold increase in the number of users afforded by muti-user beamforming.

Section 8.4.4.7.2.3 of Draft Amendment P802.16j/D2 describes the AAS Relay Zone access channel but does not detail the access messaging required for managing channel descriptor request, initial bandwidth request/grant, codeword assignment, range/frequency/power adjustment.

# Proposed Solution

The proposed solution is to describe the bandwidth grant and link adjustment mechanism and the required Direct Signaling Access Messages to accomplish it within the AAS Relay Zone.

# Detailed Solution

## Private MAP (PMAP)

The PMAP is used to grant bandwidth and provide HARQ control signaling on a burst basis.  The following table defines the AAS_PMAP Message structure:

.

Table 1: AAS_PMAP Message Structure

| RLP Component | Bits | Reps | Slot | Comment |
| --- | --- | --- | --- | --- |
| AAS_PMAP_Message { |  |  |  |  |
| PB_Ctl_0_Partition_Id | 5 | 1 |  | Partition id for control_0 structure |
| PB_Ctl_0_Burst_Id | 2 | 1 |  | Burst id for control_0 structure |
| UL HARQ Ack/Nack | 1 | 1 |  | 1=Ack, 0=Nack |
| PB_Ctl_1_Partition_Id | 5 | 1 |  | Partition id for control_1 structure |
| PB_Ctl_1_Burst_Id | 2 | 1 |  | Burst id for control_1 structure |
| DL HARQ re-tx indication | 1 | 1 |  | 0=new frame, 1=frame n-2 retransmission |
| PB_Ctl_2_Partition_Id | 5 | 1 |  | Partition id for control_2 structure |
| DIUC | 3 | 1 |  | DL MCS level |
| PB_Ctl_2_Burst_Id | 2 | 1 |  | Burst id for control_2 structure |
| UIUC Offset | 2 | 1 |  | UL MCS offset from the DIUC index |
| UL Range Adjust | 2 | 1 |  | Range Adjustment encode |
| UL Frequency Adjust | 2 | 1 |  | Frequency Adjustment encode |
| UL Partition Power Adjust | 8 | 1 |  | 1 bit per partition (CVSD) |
| CRC | 8 | 1 |  | CRC-8 |
| } |  |  |  |  |
| Total RLP Bits | 96 |  | 2 |  |

- Partition/Burst Control (7 bit encode) – The partition/burst control encode is described in Table 2 below:

Table 2: Partition/Burst Control encode

| 5 bit value | 2 bit encode value | Description |
| --- | --- | --- |
| 0-23 | 0-3 (burst index) | Probe, Open or Maintain (p,o,m) the partition indexed by the 5 bit value in the burst indexed by the 2 bit encode value. |
| 24 | 0-3 (partition index) | Close Partition in the 4 partition range 0-3 indexed by the 2 bit encode value |
| 25 | 0-3 (partition index) | Close Partition in the 4 partition range 4-7 indexed by the 2 bit encode value |
| 26 | 0-3 (partition index) | Close Partition in the 4 partition range 8-11 indexed by the 2 bit encode value |
| 27 | 0-3 (partition index) | Close Partition in the 4 partition range 12-15 indexed by the 2 bit encode value |
| 28 | 0-3 (partition index) | Close Partition in the 4 partition range 16-19 indexed by the 2 bit encode value |
| 29 | 0-3 (partition index) | Close Partition in the 4 partition range 20-23 indexed by the 2 bit encode value |
| 30 | 0-3 (burst index) | Close all partitions in burst using 2 bit encode as burst number |
| 31 | 0-3 | 0: Fast power control for MCS increase (affects all partitions in current burst) 1-3: Reserved |

A special encode for value 31 is used when there is an UL MCS increase. In this case there must be power increase step sizes of approximately 1.8 dB to ensure we can increase power over 3dB within two frames.

- UL Time (Range) Adjust (2 bit encode) - There will be a configurable delta (dt) for the time adjustment. A step size of dt  = 0.714 us is proposed.  0.714 us corresponds to 4 samples at a 5.6 MHz sample rate.  In 802.16, 4 samples corresponds to a “PS”.  The frequency adjustment encode is described in Table 3  below:

Table 3: UL Range (Time) Adjustment Encoding

| Encoded bits | Time Delay @ BS | SS UL Adjustment |
| --- | --- | --- |
| 00 | te < -t | Delay |
| 01 | -t ≤ te < 0 | Delay |
| 10 | 0 ≤ te < t | Advance |
| 11 | t ≤ te | Advance |

- UL Frequency Adjust (2 bit encode) – There will be a configurable delta (df) for the frequency adjustment. The frequency adjustment encode is described in Table 4 below:

Table 4: UL Frequency Adjustment Encoding

| Encoded bits | Frequency Error @ BS | SS UL Adjustment |
| --- | --- | --- |
| 00 | fe < -df | increase frequency |
| 01 | -df ≤ fe < 0 | increase frequency |
| 10 | 0 ≤ fe < df | decrease frequency |
| 11 | df ≤ fe | decrease frequency |

- UL Power Adjust (8 bit encode) – The power adjust is a 1-bit CVSD encode per available partition in a burst.

## CQICH Signaling

The CQICH is always allocated as the last symbol of a frame for any partition that is allocated to a subscriber. There is no need for an allocation IE or control IE to allocate  resources, or manage configuration. The configuration is implicit according to the sections below.

### CQICH mapper and demodulator

Select Hadamard Column

K-4 bits

K bits

4

MSB bits

vector, length = 16

11

10

00

01

1

0

LSB

16 subcarriers

Simplex (K=4),

BPSK (K=5), or

QPSK(K=6) Mapper

b4b3b2b1b0

Toggl-ing

Figure 1. Block diagram of CQICH mapper

We will use K = 5 bits.  The constellation is BPSK. The constellation amplitude is unity; no boosting.  b4 is the MSB.

The toggling vector is applied per partition, which is described as:

in which the k-th element( from the leftmost) corresponds to the k-th bit (from the lowest subcarrier) of the coded CQICH sequence.

### Subscriber headroom report

This field describes the difference between the maximum permissible power and the current SS UL transmit power.  The current UL transmit power is for all of the partitions and is the greater one of the two antennas.

#### For a burst with only one partition

b3

b2

b1

b0

b4

MSB

LSB

R0

H

P01

P00

R1

Figure 2. CQICH bit allocation of 1 partition sub-burst

Note: is defined as power control for each partition, and subscript denotes partition index, and subscript denotes power control bit index; H is defined as HARQ acknowledge bit; and  is defined as headroom report bits.

Table 5. For a burst with only 1 partition

| R1 R0 | Description |
| --- | --- |
| 00 | There is no headroom and this partition is the most expensive partition (MEP) among all open partitions |
| 01 | There is no headroom and this partition is not the MEP among all open partitions taking into account partitions in other bursts. |
| 10 | There is enough headroom to open 1 additional partition or increase the MCS |
| 11 | There is enough headroom to open 2 partitions or increase the MCS |

If multiple bursts are open, only one partition in one of the bursts will be indicated as the MEP if the SS is out of headroom.

In a multiple bursts case, the recommendation for each burst can be implemented simultaneously by the BS. For example, if burst 0 says OK to add 2 partitions, and burst 1 says OK to add 1 partition, the BS may choose to add 0 to 3 partitions.

H is the DL HARQ bit; 1=ACK, 0=NACK.

#### For a burst with two partitions

R0

H

P01

P00

R1

R2

H

P11

P10

C1

First partition of the burst

Second partition of the burst

Figure 3. CQICH bit allocation for 2 partition sub-burst

Table 6. For a burst with 2 to 4 partitions

| R2R1R0 | Description |
| --- | --- |
| 000 | There is no headroom, and 1st partition in the burst is the MEP |
| 001 | There is no headroom, and 2nd partition in the burst is the MEP |
| 010 | There is no headroom, and 3rd partition in the burst is the MEP |
| 011 | There is no headroom, and 4th partition in the burst is the MEP |
| 100 | Not enough headroom to increase MCS nor open new partition, and no partition in the burst is the MEP |
| 101 | Enough headroom to open 1 partition |
| 110 | Enough headroom to open 2 partitions |
| 111 | Enough headroom to increase MCS of the burst |

Bit C1 is used for even parity check across the CQICH bits, except the HARQ bit of the 2nd partition, of the 2 partitions, such as:

At the base station, if the parity check fails or either of the HARQ bits on two partitions are equal to NACK, we assume a NACK was transmitted.  During transmission, the HARQ bit of the second partition is identical to the HARQ bit of the first partition.

When there are two partitions are open, the two partitions can be placed in either one burst or two bursts. Here we recommend using one burst with two partitions. By doing this, we can reduce the PMAP overhead, and add parity check and HARQ bit repetition.

#### For a burst with 3 partitions

R0

H

P01

P00

C0

R1

H

P11

P10

C1

R2

H

P21

P20

C2

Figure 4. CQICH bit allocation for 3 partition sub-burst

Bit Ci is used for even parity check across the CQICH bits on each partition, such as:

At base station, we assume that a HARQ NACK was transmitted if any of the following conditions are true:

- No partition passes parity check.

- At least one partition passes parity check, and the number of partitions with a NACK is more than or equal to the number of partition with an ACK.

This rule also applies to a burst with more than 3 partitions.

#### For a burst with 4 partitions

R0

H

P01

P00

C0

R1

H

P11

P10

C1

R2

H

P21

P20

C2

1

H

P31

P30

C3

Figure 5. CQICH bit allocation for a 4 partition sub-burst.

The parity check bit for the first 3 partitions are defined as in .

The parity check bits for the 4th partition is defined as:

#### For a burst with more than 4 partitions

R0

H

P01

P00

C0

R1

H

P11

P10

C1

R2

H

P21

P20

C2

R3

H

P31

P30

C3

1

H

Pm1

Pm0

Cm

Figure 6. CQICH bit allocation for a sub-burst with more than 4 partitions (m=5, 6, 7, 8)

The parity check bit for the first 4 partitions are defined as in .

The parity check bit for other partitions are defined as in .

Table 7. For a burst with 5 to 8 partitions

| R3R2R1R0 | Description |
| --- | --- |
| 0000 | There is no headroom, and 1st partition of the burst is the MEP |
| 0001 | There is no headroom, and 2nd partition of the burst is the MEP |
| 0010 | There is no headroom, and 3rd partition of the burst is the MEP |
| 0011 | There is no headroom, and 4th partition of the burst is the MEP |
| 0100 | There is no headroom, and 5th partition of the burst is the MEP |
| 0101 | There is no headroom, and 6th partition of the burst is the MEP |
| 0110 | There is no headroom, and 7th partition of the burst is the MEP |
| 0111 | There is no headroom, and 8th partition of the burst is the MEP |
| 1000 | There is no enough headroom to increase MCS nor open a new partition. |
| 1001 | Enough headroom to open 1 partition |
| 1010 | Enough headroom to open 2 partitions |
| 1011 | Enough headroom to increase MCS of the burst |
| 11xx | Not defined |

# Proposed Text Changes

Insert the following subclause:

8.4.4.7.2.3.4 AAS Relay Zone Messaging

8.4.4.7.2.3.4.1 Private MAP (PMAP) Message

The FLP provides code word assignments during initial ranging and provides bandwidth grant,  codeword assignment, supportable MCS for UL/DL, and range(time)/frequency/power adjustment during the bandwidth grant exchange.

Insert  Table 2xx (.16e)/Table 3xx (Rev2) as indicated:

| Syntax | Size | Notes |
| --- | --- | --- |
| AAS_PMAP() { |  |  |
| PB_Ctl_0_Partition_Id | 5 bits | Partition id for control_0 structure |
| PB_Ctl_0_Burst_Id | 2 bits | Burst id for control_0 structure |
| UL HARQ Ack/Nack | 1 bit | 1=Ack, 0=Nack |
| PB_Ctl_1_Partition_Id | 5 bits | Partition id for control_1 structure |
| PB_Ctl_1_Burst_Id | 2 bits | Burst id for control_1 structure |
| DL HARQ re-tx indication | 1 bit | 0=new frame, 1=frame n-2 retransmission |
| PB_Ctl_2_Partition_Id | 5 bits | Partition id for control_2 structure |
| DIUC | 3 bits | DL MCS level |
| PB_Ctl_2_Burst_Id | 2 bits | Burst id for control_2 structure |
| UIUC Offset | 2 bits | UL MCS offset from the DIUC index |
| UL Range Adjust | 2 bits | Range Adjustment encode |
| UL Frequency Adjust | 2 bits | Frequency Adjustment encode |
| UL Partition Power Adjust | 8 bits | 1 bit per partition (CVSD) |
| CRC | 8 bits | CRC-8 |
| } |  |  |

Insert the following parameter descriptions following Table 2xx (.16e)/Table 3xx (Rev2)  as indicated:

#### Partition_Ctl_0_Partition_Id

5 bit Partition Id encode portion for the first of three partition control structures (Partition_Ctl_0)

#### Partition_Ctl_0_Burst_Id

2 bit Burst Id encode portion for the first of three partition control structures (Partition_Ctl_0)

### UL HARQ Ack/Nack

### Indicates whether frame n-2 UL was received successfully or not (1=Ack, 0=Nack)

#### Partition_Ctl_1_Partition_Id

5 bit Partition Id encode portion for the second of three partition control structures (Partition_Ctl_1)

#### Partition_Ctl_1_Burst_Id

2 bit Burst Id encode portion for the second of three partition control structures (Partition_Ctl_1)

### DL HARQ re-tx indication

Indicates whether this DL burst is a retransmission or not. (0=new frame, 1=frame n-2 retransmission)

#### Partition_Ctl_2_Partition_Id

5 bit Partition Id encode portion for the third of three partition control structures (Partition_Ctl_2)

### DIUC

### DIUC

#### Partition_Ctl_2_Burst_Id

2 bit Burst Id encode portion for the third of three partition control structures (Partition_Ctl_2)

### UIUC Offset

Offset from the DIUC

#### UL Range Adjust

Range Adjustment.

#### UL Frequency Adjust

Frequency adjustment

#### UL Partition Power Adjust

Power adjustment for the bearer partitions (up to 6 described in the partition grant control)