| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of Burst-based Forwarding in RS Group |  |
| Date Submitted | 2008-01-21 |  |
| Source(s) | Kanchei (Ken) Loa, Hua-Chiang Yin, Yung-Ting Lee, Yi-Hsueh Tsai, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan | Voice: +886-2-27399616 Fax: +886-2-23782328 loa@iii.org.tw |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | Clarify the benefits of burst-based forwarding in RS group |  |
| Purpose | Text proposal for 802.16j Baseline Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Burst-based Forwarding in RS Group

Kanchei (Ken) Loa, Hua-Chiang Yin, Yung-Ting Lee, Yi-Hsueh Tsai, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In p802.16j/D2, the burst-based forwarding scheme has been defined to accommodate frequent changes of the forwarding path due to the MS movement or the CDMA contention-based ranging within the RS group with the transparent RS. This contribution further describes benefits of the burst-based forwarding scheme in a RS group in comparison with the CID-based forwarding scheme.

In the CID-based forwarding scheme, whenever a new path is established between the MR-BS and the MS, a CID-to-Path-binding is required to update the routing table at RSs along the relay path. However, in the burst-based forwarding scheme, no CID-to-path-binding updates are required whenever the access station is changed since forwarding rules are encoded in the Burst IE sent from the MR-BS to the RS. So, it is obvious that the burst-based forwarding scheme is superior to the CID-based forwarding scheme when the relay path of an MS changes frequently. In an RS group, the access station of the MS may be re-selected by the MR-BS whenever an MS sends a CDMA code or whenever MS moves between RSs. That is, the relay path of the MS could be changed very often. As a result, the burst-based forwarding scheme is more suitable for the RS group in comparison with the CID-based forwarding scheme.

In order to elaborate benefits of the burst-based forwarding in a RS group, two examples are given as follows.

Example 1: MS sending BR CDMA code in RS group

Figure 1 illustrates the flow chart of the CID-based forwarding in a RS group, where the access station of MS is changed from RS1 to RS2 after the MS sends a BR CDMA code to request UL Bandwidth. It is obvious that the MR-BS needs to incur overheads to update the CID-to-path-binding when the access station of the MS is changed from RS1 to RS2 owing to the CDMA BR, which could happen frequently.

Figure 1(a) MR-BS re-selects RS2 as the access station of MS after receiving BR

Figure 1(b) MR-BS updates the routing table of MS by DSx messages

Figure 2 illustrates the flow chart of the burst-based forwarding scheme in RS group. Instead of using DSx messages to update the CID-to-path-binding, Burst IEs defined by the burst-based forwarding scheme are used to enable RS2 to forward data between the MR-BS and the MS.

Figure 2(a) MR-BS re-selects RS2 as the access station of MS after BR (The same as Figure 1(a)).

Figure 2(b) MR-BS encodes the data forwarding rules in Burst IE

Example 2: MS movement in RS group

Figure 3 illustrate the flow chart of a MS movement in an RS group, where the access station of the MS is changed from RS1 to RS2. In this example, since RS1 and RS2 broadcast the same preamble/FCH/MAP, the MS is not aware of the change of the access station but the MR-BS recognizes the changes by receiving RS_MOB_MEAS-REP from RS1 and RS2. In order to update the forwarding rules, MR_Member_List_Update message can be used in a unicast fashion as defined in P802.16j/D2. However, the period to complete the member list update could be long due to ACK failure. As a result, the burst-based forwarding can be used for forwarding temporary data before the member list update is completed.

Figure 3(a) MR-BS recognize MS moving into the proximity of RS2

Figure 3(b) Burst-based forwarding is used for data forwarding before CID-to-path binding update is complete.

Summary

- The burst-based forwarding scheme is the most efficient forwarding scheme for the RS group (transparent RS or non-transparent RS in a RS group).

- The burst-based forwarding scheme for the transparent RS has been accepted in D2.

- The burst-based forwarding scheme for an RS group with non-transparent RS is provided by comment #2216.