| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction on Sec 1.6 Relay Overview |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Rakesh Taori SAIT | Voice: Fax: rakesh.taori@samsung.com |
| Re: | IEEE 802.16j-07/059: “IEEE 802.16 Working Group Letter Ballot Recirc #28a: Announcement” |  |
| Abstract | This contribution proposes modifications on section 1.6 |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction on Sec1.6 Relay Overview

Rakesh Taori 
SAIT

Introduction

This is a proposal for resolving comments #2007 through #2026 of the commentary database 802.16-08/002

Specification Changes

[Change Section 1.6 as following]

Multihop Relay (MR) is an optional deployment that may be used to provide additional coverage or performance advantage in an access network. In MR networks, the BS may be replaced by a Multihop Relay BS (MR-BS) and one or more Relay Stations (RS).

Traffic and signaling between the SS and MR-BS are relayed by the RS thereby extending the coverage and performance of the system in areas where RSs are deployed. Each RS is under the supervision of a parent MR-BS. In a more than two hop system, traffic and signaling between an access RS and MR-BS may also be relayed through intermediate RSs. The RS may be fixed in location (i.e. attached to a building) or, in the case of an access RS, it may be mobile (i.e. traveling with a transportation vehicle). The SS may also communicate directly with the MR-BS.

The various MR features defined throughout this standard permit a multihop relay system to be configured in several modes

The protocols (including the mobility features) on the access link remain unchanged. New functionality has been specified on the relay link to support the MR features. ..

Two different modes, viz. centralized and distributed scheduling modes, are specified for controlling the allocation of resources to an SS. In centralized scheduling mode the resource allocation for an RS's subordinate stations is determined at the MR-BS; conversely in distributed scheduling mode the resource allocation of an RS's subordinate stations is determined by the RS, in cooperation with the MR-BS.

The MAC CPS includes extensions to signaling to support functions such as network entry (of RS, and of MS through RS), bandwidth request, forwarding of PDUs, connection management and handover.

For the data plane, two different security models are defined (see clause 7). The first one, referred to as centralized security model, is based on key management between MR-BS and SS. The second security model, referred to as distributed security model, incorporates key management between MR-BS and access-RS and access-RS and MS. For the control plane, the key management is hop-by-hop in case of distributed scheduling mode.

The PHY includes extensions to the OFDMA-PHY layer (see 8.4) for transmission of PHY PDUs across the relay link between the MR-BS and the RS. An RS can only support one segment.

An RS may be configured to operate either in normal CID allocation mode, where primary management and basic CIDs are allocated by the MR-BS or in local CID allocation mode where the primary management and basic CID are allocated by the RS.

Two different modes of physical layer operation are defined at the RS, namely transparent and non-transparent. A non-transparent RS can operate in both centralized and distributed scheduling mode, while a transparent RS can only operate in centralized scheduling mode.