| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Zone Configuration IE |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Senarath Gamini, Israfil Bahceci, Peiying Zhu, Wen Tong, Derek Yu, Sun Sheng, Mark Naden Nortel Networks 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | E-mail: gamini@nortel.com |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution clarifies Zone configuration IE in RCD message. |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on Zone Configuration IE
Gamini Senarath, et. al.

Nortel

### Introduction

As discussed during the meeting, configuration of 16j frame in D2 is confusing since some configuration parameters are missing from Zone Configuration IE. We update the IE with the required parameters.

### Proposed Text Changes

| Syntax | size | note |
| --- | --- | --- |
| Zone Configuration IE _Format() { | - | - |
| Zone Configuration bitmap | 8 bits | b0=1, permutation based included b1=1, range of subchannels included b2=1, STC mode b3=1, AMC mode b4=1, The zone is for Safety/PAR reduction b5-b7: reserved |
| if (b0 of Zone Configuration bitmap ==1) { |  |  |
| Permutation based | 6 bits | DL_PermBase or UL_PermBase to be used in this zone |
| Permutation | 2 bits | 0b00: PUSC permutation 0b01: FUSC permutation 0b10: Optional FUSC permutation 0b11: Adjacent subcarrier permutation |
| PRBS_ID | 2 bits | Values: 0..2. Refer to 8.4.9.4.1 |
| Reserved | 6 bits |  |
| } | - | - |
| if (b1 of Zone Configuration bitmap ==1) { | - | - |
| if (The IE is in DL-subframe configuration) { |  |  |
| Used subchannel bitmap | 6 bits | bit #0, Subchannel group 0 bit #1, Subchannel group 1 bit #2, Subchannel group 2 bit #3, Subchannel group 3 bit #4, Subchannel group 4 bit #5, Subchannel group 5 |
| reserved | 2 bits | Shall be zero |
| } |  |  |
| if (The IE is in UL-subframe configuration) { |  |  |
| Min Subchannel index | 8 bits | The index of subchannel from which the allocation starts. |
| Max Subchannel index | 8 bits | The index of subchannel at which the allocation ends. |
| } |  |  |
| } | - | - |
| if (b2 of Zone Configuration bitmap ==1) { | - | - |
| STC mode | 2 bits | 0b00: Reserved 0b01: STC using 2/3 antennas 0b10: STC using 4 antennas 0b11: FHDC using 2 antennas |
| Matrix Indicator | 2 bits | STC matrix (see 8.4.8.1.4) if (STC == 0b01 or STC == 0b10) { 0b00 = Matrix A 0b01 = Matrix B 0b10 = Matrix C 0b11 = Reserved } else if (STC == 0b11) { 0b00 = Matrix A 0b01 = Matrix B 0b10–11 = Reserved } |
| Midamble presence | 1 bit | 0: Not present 1: MIMO midamble present at the first symbol in STC zone |
| Midamble boosting | 1 bit | 0: No boost 1: Boosting (3 dB) |
| 2/3 antennas select | 1 bit | 0: STC using 2 antennas 1: STC using 3 antennas Selects 2/3 antennas when STC = 0b01 |
| Antenna Assignment | 4 bits | Bit#0: Antenna #0 Bit#1: Antenna #1 Bit#2: Antenna #2 Bit#3: Antenna #3 |
| Reserved | 5 bits |  |
| } | - | - |
| if (b3 of Zone Configuration bitmap ==1) { | - | - |
| AMC Type | 2 bits | Indicates the AMC type in case permutation type = 0b11, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved Note that only 2x3 band AMC subchannel type (AMC Type = 0b01) is supported by MS |
| Reserved | 6 bits |  |
| } | - | - |
| } | - | - |

Antenna Assignment

If Antenna Assignment is non-zero, then the RS shall implement the STC zone taking the role of antennas indicated in this field. For example, for RS with 4 antennas,and STC mode = 10, if the Bit#0-Bit#3 is 0b0000, then the RS transmits according to STC mode with 4 transmit antennas, while if the Bit#0-Bit#3 is 0b1001, then the RS take role of Antenna #0 and Anetnna #3 and transmits pilots based on the antenna assignment and permutations indicated.