| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Comment #2194 in 80216-08_002.cmt |  |
| Date Submitted | 2008-01-22 |  |
| Source(s) | Senarath Gamini, Israfil Bahceci, Hang Zhang,Peiying Zhu, Wen Tong, Derek Yu, Sun Sheng, Mark Naden Nortel Networks 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | E-mail: gamini@nortel.com |
| Re: | IEEE 802.16j-07/043: “IEEE 802.16 Working Group Working Group Letter Ballot #28” |  |
| Abstract | This contribution clarifies data forwarding in systems with RS groups. |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on Comment #2194
Gamini Senarath, et. al.

Nortel

### Introduction

The data forwarding in the presence of RS groups is not clear in 80216j/D2. We revise text amendments suggested in Comment #2194 to clarify the available data forwarding methods for this case.

### Proposed Text Changes

[Change Definitions 3.94 and 3.95 on Page 4 as follows]

3.94 tunnel CID (T-CID): An identifier taken from the connection identifier (CID) space that uniquely

identifies a transport tunnel connection.

3.95 management tunnel CID (MT-CID): An identifier taken from the connection identifier (CID) space

managed by an MR-BS that uniquely identifies a management tunnel connection

.

[Change the text on line 15-16 of page 7 as follows]

An additional type of connection called a tunnel connection may be established between the MR-BS and an

access RS, or between the MR-BS and super-ordinate of an RS group (see 6.3.33).

[Insert the following text starting from Line 24 of Page 79]

In MR networks with RS groups, tunnel-based, CID based or burst-based forwarding can be applied. In tunnel-based forwarding, the tunnel connections are established between the MR-BS and the superordinate station of the RS group, i.e., the super-ordinate station is the end-of-tunnel in DL and begining-of-tunnel in UL. In this case, the data forwarding between the super-ordinate station and the MS/SS may be either CID based or burst-based. The burst based forwarding shall be employed only if selective forwarding is not used, i.e., when all RS group membes are involved in forwarding data to a specific MS/SS. More specific information for data forwarding within RS group is provided in Subclause 6.3.33.

[Change text on line 34 of page 79]

One or more tunnels may be established between the MR-BS and the access RS after the Network entry is

performed. If the MS/SS is served by an RS group, the tunnel connections shall be established between the MR-BS and the super-ordinate station of the RS group.

[Change the text on 37 of page 130 as follows]

When a single tunnel is used between MR-BS and an RS, the relay MAC header may contain

[Change the text on 21-22 of page 161 as follows]

A routing table that contains the mapping between a CID and one or more given paths needs to be updated

when a new tunnel (identified by a Tunnel CID) is generated between the MR-BS and an RS,..

[Change the text on line 22 of page 172 as follows]

Data forwarindg in MR networks with RS group is described in Subclause 6.3.3.8. The following describes data forwarding within RS group in detail: For DL, the members of an RS group may be configured to forward traffic

data for only specific subordinate terminal stations.