| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Fast Ranging Support Considering Non-Transparent RSs in Distributed Scheduling Mode |  |
| Date Submitted: | 2008-03-19 |  |
| Source(s) | Ting Li, Yunbao Zeng, Liangliang Zhang, Shulan Feng Hisilicon Technologies Harbour Building, No.8, Dongbeiwang West Road, HaiDian District, Beijing, China Zheng Shang, Yan Peng Huawei Technologies No.98, Lane91, Eshan Road, Shanghai, China | Voice: 86-10-82829015 Fax: 86-10-82829075 mailto: liting@hisilicon.com; liting_bj@huawei.com Voice: 86-21-50993165 mailto: shang_zheng@huawei.com Voice: 86-21-50993162 mailto: peng.yan@huawei.com |
| Re: | This document is a response to “IEEE 802.16 Working Group Letter Ballot #28b: announcement”. |  |
| Abstract | This contribution proposes a method for supporting fast ranging when considering non-transparent RSs operating in Distributed Scheduling Mode. |  |
| Purpose | Text proposal for 802.16j Draft Document 3.0. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Fast Ranging Support Considering Non-Transparent RSs in Distributed Scheduling Mode

Ting Li, Yunbao Zeng, Liangliang Zhang, Shulan Feng

Hisilicon Technologies Co., LTD

Zheng Shang, Yan Peng

Huawei Technologies Co., LTD

### Introduction

When an MS’s HO target station is a non-transparent RS, the RS may support the MS’s fast ranging. When the RS has unique BSID and is operating in distributed scheduling mode, however, it is not defined in Draft 3.0 how the RS supports the MS’s fast ranging.

### Proposal

When the MR-BS is prenotified of the upcoming HO MS and the MS’s HO target is a non-transparent RS served by the MR-BS and operating in distributed scheduling mode, the MR-BS may send a MOB_INF-IND message to the RS, notifying the RS of MS MAC Address and Action Time. When the RS receives the MOB_INF-IND message, it may provide Fast Ranging allocation for the MS based on Action Time by inserting Fast Ranging IE in UL-MAP at the appropriate frame.

Afterward, the MR-BS may notify the RS of canceling providing Fast Ranging allocation for the MS by sending another MOB_INF-IND message indicating canceling to the RS, if necessary, e.g. the MS rejects or cancels HO to the RS. When the RS receives the MOB_INF-IND message indicating cancellation, it shall not provide Fast Ranging allocation for the MS.

### Proposed Text

[Modify the third paragraph in subclause 6.3.10.3.3:]

Alternatively, if the BS/MR-BS is prenotified for of the upcoming HO MS, it may provide bandwidth for the MS to send a RNG-REQ by using Fast Ranging IE. If the MR-BS is prenotified of the upcoming HO MS and the MS's superordinate station is a non-transparent RS with unique BSID and scheduling is centralized, the MR-BS shall insert this IE in the UL-MAP that it sends to the RS to broadcast on the access link. The MR-BS shall also provide bandwidth along the relay path on which to forward the RNG-REQ to the MR-BS. And the MR-BS shall send a MOB_INF-IND message with Action_Indicator=1 to the RS indicating the RS to provide bandwidth for the MS to send an RNG-REQ by using a Fast_Ranging IE and indicating when the RS provides the Fast_Ranging IE. When the RS receives the MOB_INF-IND message, it shall provide Fast Ranging IE for the MS based on the Action Time in the MOB_INF-IND message. Afterward, if the MR-BS is notified that the MS has rejected or cancelled the HO to the RS, it may send a MOB_INF-IND message with Action_Indicator=0 to the RS. When the RS receives the message, it shall cancel providing bandwidth for the MS to send an RNG-REQ. If the MR-BS is prenotified of the upcoming HO MS and the MS's superordinate station is a transparent RS, the MR-BS shall insert the Fast-Ranging_IE in the UL-MAP, and if management messages are relayed, the MR-BS shall precede the Fast Ranging IE with an UL_Burst_Receive_IE assigned to the RS basic CID. When a transparent RS finds its RS basic CID in an UL_Burst_Receive_IE, it shall listen for the RNG-REQ on the burst specified by the Fast Ranging IE that follows the UL_Burst_Receive_IE and relay the RNG-REQ to the MR-BS on the RS basic CID.

[Insert new subclause 6.3.2.3.XX:]

6.3.2.3.XX MOB_INF-IND message

This message may be used for the MR-BS to notify the non-transparent RS to provide or cancel providing bandwidth allocation for the MS to send an RNG-REQ message during HO. It is applicable to the distributed scheduling mode.

Table X-MOB_INF-IND message format

| Syntax | Size | Note |
| --- | --- | --- |
| MOB_INF-IND_Message_Format() { |  |  |
| Management Message Type = TBD | 8bits |  |
| MS MAC Address | 48bits |  |
| Action_Indicator | 1bit | 0: Cancel; 1:Allocate |
| Reserved | 7bits |  |
| If (Action_Indicator == 1) { |  |  |
| Action Time | 8bits | Acting time when the RS provides Fast_Ranging IE for the MS. |
| } |  |  |
| TLV encoded information | Variable | HMAC/CMAC Tuple |