| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fragment Discard at Access RS for Relay Link Fragmentation |  |
| Date Submitted | 2008-03-15 |  |
| Source(s) | Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze Mitsubishi Electric Corp Wei-Peng Chen Fujitsu Laboratories of America Inc. Masato Okuda Fujitsu Laboratories Ltd Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden Nortel | Voice: 1-617-621-7557 E-mail: tao@merl.com Voice: 81-467-41-2085 E-mail: kuze.toshiyuki@ah.MitsubishiElectric.co.jp Voice: +1 408 530 4622 E-mail: wei-peng.chen@us.fujitsu.com Voice: +81 44 754 2811 E-mail: okuda@jp.fujitsu.com Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | Response to the IEEE 802.16 Working Group Letter Ballot Recirc #28b (IEEE 802.16-08/007). |  |
| Abstract | This contribution proposes to clarify the fragment discard operation at access RS for relay link fragmentation. |  |
| Purpose | To adopt the proposed text into the IEEE 802.16j/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fragment Discard at Access RS for Relay Link Fragmentation

Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze

Mitsubishi Electric Corp

Wei-Peng Chen

Fujitsu Laboratories of America Inc.

Masato Okuda

Fujitsu Laboratories Ltd

Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, G.Q. Wang, Derek Yu, Israfil Bahceci, Robert Sun and Mark Naden
Nortel

# Introduction

In IEEE 802.16 [2], fragmentation is performed between transmitter and receiver directly.  The standard has clearly specified the re-assembly process upon loss.  More specifically:

“... The sequence number assigned to each fragment allows the receiver to recreate the original payload and to detect the loss of any intermediate fragments. A connection may be in only one fragmentation state at any given time.  Upon loss, the receiver shall discard all MAC PDUs on the connection until a new first fragment is detected or a nonfragmented MAC PDU is detected.”

Fragmentation on relay link has been recently introduced in IEEE 802.16j draft D3 [1].  For fragmentation on relay link, the processing at intermediate RS and that at access RS has to be distinguished.  The intermediate RS can either send a fragment of a TDU without waiting until the entire TDU is received, or it can re-assemble the fragments before forwarding them.  However, the egress RS has to reassemble the fragments with the same TDU sequence number and then retrieves individual MAC PDUs from the assembled TDU.

Unfortunately, the current IEEE 802.16j draft D3 [1] has not explicitly defined the operation at egress station, when transmission error or loss occurs.  To address this issue, we propose the following text.

# Proposed Text Changes

6. MAC Common Part Sublayer

6.3.3.8.3 Fragmentation and packing of relay MAC PDU

[Insert text at the end of this paragraph as indicated:]

The egress station reassembles the fragments with the same TDU sequence number and then retrieves individual MAC PDUs from the assembled TDU.

When an egress station receives a fragment with a new TDU sequence number, it shall start a timer Txx.  The egress station shall reset the associated timer Txx, if it has received all the fragments of the TDU before the expiration of this timer Txx.

For distributed security mode, the egress station shall discard all the fragments of this TDU at the expiration of the timer Txx, if the egress station is unable to reassemble the full TDU.

For centralized security mode, the egress station can start to retrieve and forward MAC PDUs from the received fragments of the TDU, without waiting until the entire TDU is reassembled.  At the expiration of the timer Txx, the egress station shall discard any fragments of the TDU, if no MAC PDUs can be retrieved from these fragments.

10. Parameters and constants

10.1 Global values

[Insert a new timer definition at the end of the table 583 as indicated:]

Table 583—Parameters and constants

| System | Name | Time Reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MR-BS, RS | Txx | Time the egress station of a tunnel waits for subsequent fragments of a TDU before discarding all the received fragments of the TDU. | TBD | TBD | TBD |

# Reference

- “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Multihop Relay Specification”, IEEE P802.16j/D3, March 2007

- “IEEE Standard for Local and Metropolitan Area Networks − Part 16: Air Interface for Fixed Broadband Wireless Access Systems, Amendment 2: Physical and Medium Access Control Layers for Combined Fixed and Mobile Operation in Licensed Bands,” IEEE Computer Society and the IEEE Microwave Theory and Techniques Society, February 2006.