| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifying the relay zone indication for RS and MS |  |
| Date Submitted | 2008-03-15 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Changyoon Oh, Jung Je Son Samsung Electronics Rakesh Taori Samsung Advanced Institute of Technology | E-mail: yb.chang@samsung.com Voice: +82 31 279 5519 E-mail: rakesh.taori@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | In response to the call for comments on the IEEE 802.16j/D3 |  |
| Abstract | This contribution is to clarifying the relay zone indication for RS as well as for MS |  |
| Purpose | Discussion and Adoption in the IEEE 802.16j/D3 document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifying the relay zone indication for RS and MS

Youngbin Chang, HyunJeong Kang, Changyoon Oh, Jung Je Son, Rakesh Taori

# 1. Introduction

This contribution is to clarifying the relay zone indication for RS as well as for MS in P802.16j/D3 draft.

1. Relay zone indication for MS

In the last meeting (#53), comments #2556 of the 802.16-08_002 was accepted-modified. MS does not process the signal transmitted in the downlink relay zone indicated by DL_MAP IE with DIUC=13 or STC_DL_Zone_IE with dedicated pilots bit set to 1.

As same as downlink case, MS shall not process the signal in the uplink relay zone. For this purpose, UIUC= 13 is defined in the P802.16j/D3, but not defined UL_zone_IE. If BS uses UL_zone_IE and does not allocate data for MS, MS do not process for sending UL data after UL_zone_IE.

2. Relay zone indication for RS

The RS can obtain relay zone information from DL/UL subframe configuration TLV in the RCD message (11.24.6) during registration process. (6.3.9.9.3. Obtain R-link parameters). Therefore, DIUC/UIUC = 13 in the DL/UL MAP IE, relay zone indicator is redundant to indicate relay zone for RS.

3. Two clarifications are proposed:

- In uplink case, for legacy MS operation, UIUC = 13 or UL_zone_IE could be used for MS not to process to send UL data.

- Relay zone indicator in DIUC is redundant.

# 2. Proposed Text Changes

[Change the text from line 9 to 11 on page 192, in 8.4.5.3:]

MR-BS or RS may transmit DIUC=13 in the DL-MAP in the access zone for MS not to process the signal transmitted in the downlink relay zone to indicate the location of a DL relay zone in the same frame.  The boosting field in DIUC=13 is used to indicate to the receiving RSs that the IE defines a relay zone.

[Delete table 380:][Note: If relay indicator bit in the DIUC=13 is deleted in the DL-MAP_ IE, the table is same as 16e]

[Delete section 8.4.5.4.2 and delete text from line 29 to 31 on page 199:]

MR-BS or RS may transmit UIUC=13 in the UL-MAP in the access zone for MS not to process in the uplink relay zone. a PAPR reduction/safety zone/sounding zone allocation IE (UIUC=13) in the

UL-MAP in the access zone to indicate the location of a UL relay zone.

[Add the following text at line 41 on page 194, in 8.4.5.4.3:]

Insert the following after first paragraph in the 8.4.5.3.4:

MR-BS or RS may transmit STC_DL_Zone_IE with dedicated pilot bit set to 1 in the access zone for MS not to process the signal transmitted in the downlink relay zone.

[Add the following text at line 34 on page 201, after 8.4.5.4.4.2:]

8.4.5.4.7. UL Zone Switch IE format

Insert the following after first paragraph in the 8.4.5.4.7:

When relay zone exits in UL sub-frame, MR-BS or RS may transmit UL_Zone_IE in the access zone and shall not allocate burst for MS not to process in the uplink relay zone.