| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on transparent frame structure |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Youn-Tai Lee Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc #28c: Announcement” |  |
| Abstract | Comment on transparent frame structure |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Transparent Frame Structure

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In P802.16j/D4, it divides the uplink subframe into two time zones, access zone and relay zone, thus an MS has to transmit on more subchannels in access zone, so less transmitting power per subchannel is available. Therefore, there is a loss in the coverage area for the MSs that are not attached to an RS. In order to resolve the problem, this contribution proposes to modify the frame structure as shown in Figure 270a. The radio resource allocations in this figure are described by the logical subchannel allocation rather than physical subchannel allocation.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D4 are listed below.

Spec changes

[Modified the following text as indicated:]

6.3.7.3 DL-MAP

The DL-MAP message defines the usage of the downlink intervals on the access links for a burst mode PHY and may also be used for relay links in the case of transparent RSs.

6.3.7.4 UL-MAP

The UL-MAP message defines the uplink usage on the access link in terms of the offset of the burst relative to the Allocation Start Time (units PHY-specific) and may also be used for relay links in the case of transparent RSs.

[Insert Figure 270xxx as following indicated:]

Figure 270xxx—Example of configuration for a transparent relay frame structure

8.4.5.4.2 PAPR Reduction/Safety Zone/Sounding Zone Allocation IE

[Modified the following table as indicated:]

Table 372—PAPR Reduction/Safety Zone/Sounding Zone Allocation IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Reserved Relay Zone indcator | 1 | Shall be zero 0b0: Normal Gap/PAPR/safety zone 0b1: Relay zone indicator |