| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SBC-REQ/RSP message for using Direct Relay Zone |  |
| Date Submitted | 2008-05-09 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Sunggeun Jin, Hyunjae Kim, Junhong Hui ETRI | E-mail:schae@etri.re.kr |
|  | Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales | E-mail:kyuha.lee@samsung.com |
|  | Aik Chindapol, Yousuf Saifullah, Shashikantt Maheshwari NSN | E-mail:Aik.chindapol@siemens.com E-mail:shashi.maheshwari@nsn.com E-mail:Yousuf@nsn.com |
|  | Eugene Visotsky Motorola | E-mail:eugenev@motorola.com |
|  | Jai H. Eu Jcast Networks Korea, Inc | E-mail:jeu@jcastnet.com |
|  | Hyung-joon Jeon POSDATA | E-mail:hjjun@posdata.co.kr |
|  | Jun Bae Ahn SOLiD technologies | E-mail:jbahn@st.co.kr |
|  | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee, Institute for Information Industry 8F, No. 218, Sec. 2, Dunhua S. Rd., Taipei City 106, Taiwan | loa@iii.org.tw |
| Re: | LB#28c |  |
| Abstract | SBC-REQ/RSP message for using Direct Relay Zone in C802 16j-07_526r3 was missed in D4. |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SBC-REQ/RSP for Using Direct Relay Zone

# 1. Introduction

In the last #51’s meeting, our contributions on Direct relay Zone had already passed as C802.16j-07_526r3. However, we found that SBC-REQ/RSP messages in our contribution documents of C802.16j-07_526r3 did not exist in Draft4.

# 2. Remedy

It is necessary to indicate the capability of Direct Relaying Zone so that MR-BS can negotiate with RS on using “Direct relay Zone” as below.

| Type (1byte) | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | 1 | When Bit#0=1, RS is capable of using the direct relay Zone. Bit#1~bit#7: Reserved. | SBC-REQ SBC-RSP |

# 3. Text changes

[Modify sub-clause 8.4.4.7.5:as follows]

8.4.4.7.5 Direct Relay Zone(Optional)

A Direct Relay Zone may be optionally assigned by the MR-BS to a transparent RS. Only end-to-end HARQ mode shall be used in the direct relay zone and an RS with a direct relay zone shall not be used for more than two hops. In the direct relay zone, an RS may relay data within a single frame, in which an intermediate RS demodulates and de-interleaves, but not decode, received bursts and forwards by re-interleaving and re-modulating the bursts. The direct receiving and direct transmitting zones are configured using the RCD message. The RS shall send SBC-REQ message with the “direct relay zone support” bit is set to one (11.8.3.7.22). If MR system supports direct relay zone, the MR-BS shall SBC-RSP message with the “direct relay zone support” bit is set to one (11.8.3.7.22).   The size of FEC blocks of the busts along the relay path shall be the same as the size of FEC blocks of the bursts in the access link.

11.8.3.7.22 MR PHY feature support

This TLV indicates the MR PHY features supported by the RS and the MR-BS.

[Modify the following table]

| Type (1byte) | Length | Value | Scope |
| --- | --- | --- | --- |
| 206 | 12 | Bit #0: access zone preamble transmission support Bit #1: MBS Data Synchronization with pre-defined relative transmission time (6.3.23.3) Bit #2: MBS data synchronization with target transmission time (6.3.23.3) Bit #3: cooperative relay support Bit #4: support of a second carrier frequency at RS (see 8.4.4.7.2.2) Bit #5: support dual radio RS operation (see 8.4.4.7.2.3) Bit #6: direct relay zone support. Bits #4-7-10: Maximum number of HARQ channels supported in UL_DCH Bit #11-15: Reserved | SBC-REQ SBC-RSP |

# References

C802 16j-07_526r3, “Direct Relay using M&F”