| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RS network entry |  |
| Date Submitted | 2008-05-08 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc #28c: Announcement” |  |
| Abstract | This contribution proposes to modify on RS network entry |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on RS network entry

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Shiann-Tsong Sheu, Youn-Tai Lee

Institute for Information Industry (III)

Introduction

In P802.16j/D4, descriptions of the RS network entry procedure and the associated timers are inconsistent. In addition, flow charts and state machines for the RS network entry procedure are missing. This contribution proposed to modify those descriptions and add flow charts and state machines into P802.16j/D4. Timers for RS network entry procedure are summary in Table 1. The associations between message sequence charts and timer are given in Figure 1 and Figure 2.

Table 1 Timers for RS network entry procedure

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MR-BS | T59 | Wait for RS_NBR-MEAS-REP after sending REG-RSP to RS | TBD | TBD | TBD |
| RS | T62 as performing measurement report | Wait for RS_AccessRS-REQ or RS_Config-CMD after sending RS_NBR-MEAS-REP to MR-BS | TBD | TBD | TBD |
| RS | T62 as skipping measurement report | Wait for RS_Config-CMD after receiving REG-RSP from MR-BS when the RS skips neighbor measurement report | TBD | TBD | TBD |
| MR-BS | T63 | Wait for ACK after sending RS_Config-CMD to RS | TBD | TBD | TBD |
| MR-BS | T65 | Wait for RNG-REQ message after sending RS_AccessRS-REQ message | TBD | TBD | TBD |
| MR-BS | T70 | Wait for ACK after sending RS_AccessRS-REQ to RS | TBD | TBD | TBD |

(a) Access station unchanged					(b) Access station changed

Figure 1 RS network entry procedure when performing neighbor station measurement report

Figure 2 RS network entry procedure when skipping neighbor station measurement report

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D4 are listed below.

Spec changes

6.3.9 Network entry and initialization

[Modified the text as following indicated:]

Implementation of phase b1), l), m) are optional. The MR-BS may instruct the RS to omit phases b1), e), f), g), l), m), n) by the RS network entry optimization TLV in the RNG-RSP message.

6.3.9.9.2 MR-BS and RS behavior during registration

[Replace the following figure as indicated:]

Figure 94a—Handling REG-RSP first reception at an RS

6.3.9.16 RS neighbor station measurement report (optional)

[Modified the text as following indicated:]

The MR-BS may request the RS to do neighbor measurements during network entry or re-entry using the RS network entry optimization TLV in the RNG-RSP message.

The neighbor station measurement report can include: (1) the signal strength and preamble index of neighbor non-transparent stations with unique BSIDs, and (2) signal strength and R-amble indexes of neighbor transparent stations or non-transparent stations with shared BSIDs.

If the measurement is not required, after registration RS shall directly go to the RS configuration phase skipping neighbor station measurement report phase and second stage access selection phases.

The RS obtains the neighbor monitoring scheme parameters from the RCD message. If the RS is a mobile RS, and the RCD message provides the TLV “preamble indexes reserved for mobile RSs”, the mobile RS shall report the signal strengths and preamble indices of these preambles.

The RS shall use the available neighborhood measurement mechanisms defined in Subclauses 6.3.28, 6.3.29 and 8.4.6.1.1.4. The RS shall then send the measurements to the MR-BS using the RS_NBR-MEAS-REP message (6.3.2.3.70) and start T62 timer. This measurement report is used by the MR-BS during the second stage access station selection stage to decide a new access station (or to continue with the current access station). After receiving RS_NBR-MEAS-REP message, the MR-BS shall start T59 timer. The MR-BS may assign the RS a preamble index based on the report from the RS.

[Insert the following figure as indicated:]

Figure 102a—Perform neighbor measurement report at a RS

[Insert the following figure as indicated:]

Figure 102b—Handling RS_NBR-MEAS-REP first reception at an MR-BS

6.3.9.17 Second stage access station selection

[Modified the text as following indicated:]

The second stage access station selection procedure performed in phase m) is an optional procedure as indicated by the RS network entry optimization TLV in the RNG-RSP message. This operation happens after the neighbor station measurement report and before the RS operation parameter configuration. During this operation, the MR-BS shall determine the best path (i.e. access station) for this RS based on the RS’s neighbor station measurements and other information such as path loading. If the current access station is not changed, RS shall directly go to the RS configuration phase skipping second stage access selection phase. If the current access station is changed and is located in the same MR-cell, the MR-BS shall start T65 T70 timer and send the RS_AccessRS-REQ message to the RS to indicate the preamble index of the selected access station. The RS shall respond with the MR_Generic-ACK message, the MR-BS shall stop T70 timer and then start T65 timer. If the access station indicated in the RS_AccessRS-REQ message is not the access station the RS is currently attached to Afterward, the MR-BS and the RS shall perform network re-entry as described in 6.3.9. Once T65 expires, the MR-BS shall release resources allocated to the corresponding RS.

[Insert the following figure as indicated:]

Figure 102c—Handling RS_AccessRS-REQ at an RS

[Insert the following figure as indicated:]

Figure 102d—Perform access station selection at an MR-BS

Figure 102e—Handling RS_NBR-MEAS-REP retransmission at an MR-BS

6.3.9.18 RS operation parameters configuration

6.3.9.18.1 Parameter configuration

[Modified the text as following indicated:]

A transparent RS receives the R-MAP and RCD messages from the access station in the access zone in order to obtain the R-link parameters (see Figures 94g 102i and 94h 102j)

[Insert the following figure as indicated:]

Figure 102f—Handling RS_Config-CMD first reception at an RS

Figure 102g—Handling RS_Config-CMD retransmission at an RS

[Insert the following figure as indicated:]

Figure 102h—Configure operation parameters to an RS at an MR-BS

[Move Figure 94f to here as following indicated:]

Figure 94f 102i—Obtaining and maintaining synchronization with R-MAP at an RS

[Move Figure 94g to here as following indicated:]

Figure 94g 102j—Obtaining RCD message at an RS

[Insert the following subclause as indicated:]

6.3.9.19 Network entry state machine for the MR system

Figure 102k describes the network entry process on the MR-BS side. Figure 102l describe the network entry process on the RS side.

Figure 102k—Network entry state machine REG-REQ at a MR-BS

Figure 102l—Network entry state machine at an RS

10.1 Global values

[Modified the following Table 583 as indicated:]

Table 583—Parameters and constants

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| MR-BS | T59 | Wait for RS_NBR-MEAS-REP after sending REG-RSP to RS | TBD | TBD | TBD |
| RS | T62 as performing measurement report | Wait for RS_AccessRS-REQ RNG-RSP (abort) or RS_Config-CMD after sending RS_NBR-MEAS-REP to MR-BS | TBD | TBD | TBD |
| RS | T62 as skipping measurement report | Wait for RS_Config-CMD after receiving REG-RSP from MR-BS and RS should skip neighbor measurement step | TBD | TBD | TBD |
| MR-BS | T63 | Wait for ACK after sending RS_Config-CMD to RS | TBD | TBD | TBD |
| MR-BS | T65 | Wait for RNG-REQ message with RS basic CID after sending RNG-RSP RS_AccessRS-REQ message which contains abort status and Preamble Indexes TLV | TBD | TBD | TBD |
| MR-BS | T70 | Wait for ACK after sending RS_AccessRS-REQ to RS | TBD | TBD | TBD |