| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | RRM Context Relay between the MR-BS and the non-transparent RS operating in distributed scheduling mode |  |
| Date Submitted | 2008-05-08 |  |
| Source(s) | Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Chun-Yen Hsu Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan [add other co-authors here] | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc #28c: Announcement” |  |
| Abstract | This contribution proposes to reedit the paragraphs of subclause 6.3.10.3.1.1 |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RRM Context Relay between the MR-BS and the non-transparent RS operating in distributed scheduling mode

Kanchei (Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Introduction

RRM is a function performed by the ASNs in a WiMAX Network, aiming at increasing the radio resource usage efficiency. RRM introduces a concept of Radio Resource Agent (RRA) and Radio Resource Controller (RRC) functional elements and signaling between RRA and RRC and between RRC and RRC. WiMAX Forum NWG specifies R6 messages to transfer the RRM context between the BS and the ASN-GW (and another BS). In MR system operating in distributed scheduling mode, however, the MR-BS did not hold all radio resource management information of its subordinate RS such as PHY Quality Info. and spare capacity, which can only be provided by the scheduler. In order to gather RRM context from the distributed RS, a new message is required to relay the information to the MR-BS.

Table1 – RRM Procedures, Messages, Mapping to Reference Points

| RRM Primitives | Communication Peers | Profile A | Profile C | Inter-ASN (profile independent) |
| --- | --- | --- | --- | --- |
| R4/R6 Per-BS Spare_Capacity_Req and Spare_Capacity_Rpt | RRC – RRC | R4 | R4 and R6 | R4 |
|  | RRC – RRA | R6 | None (BS internal) | None (RRC-RRA is ASN internal) |
| R6 Neighbor_BS_Resource_Status_Update | RRC – RRA | R6 | None (BS internal) | None (RRC-RRA is ASN internal) |
| R6 Per-MS PHY_Parameters_Req and PHY_Parameters_Report | RRC – RRA | R6 | None (BS internal) | None (RRC-RRA is ASN internal) |
| R4/R6 Per-BS Radio_Config_Update_Req and Radio_Config_Update_Rpt and Radio_Config_Update_Ack | RRC – RRC | R4 | R4 and R6 | R4 |
|  | RRC – RRA | R6 | None (BS internal) | None (RRC-RRA is ASN internal) |

This contribution proposes a new message relay R6 context on relay link since P802.16j/D4 does not specify messages to transfer RRM context between RS and the MR-BS (and backbone).

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D4 are listed below.

Spec changes

[Insert the following new rows at the end of table 38 in 6.3.2.3]

Table 38 — MAC management messages

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| TBA | RRM_Context-RLY | RRM Context Relay | RS Basic |

[Insert the following new subclause at the end of 6.3.2.3:]

6.3.2.3.xx RRM_ Context-RLY message

This message is used to relay RRM context between an MR-BS and a non-transparent RS operating in distributed scheduling mod. This message is transmitted by MR-BS with using the RS’s basic CID.

Table xxx RRM_Context-REQ message Format

| Syntax | Size | Note |
| --- | --- | --- |
| RRM_ Context-Request Information Format(){ |  |  |
| Management Message Type = xx | 8 bits | TBA |
| NWG RRM TLV Encoded Information | variable | NWG RRM TLV Specific |
| TLV Encoded Information | variable | TLV Specific |
| } |  |  |

The RRM_ Context-RLY message shall include the following parameter encoded as TLV tuples:

HMAC/CMAC Tuple (See 11.1.2.)