| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A more efficient DL-MAP IE for MAPs |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Kanchei (Ken) Loa, Tsung-Yu Tsai, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin Institute for Information Industry 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan [add other co-authors here] | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16j-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc #28c: Announcement” |  |
| Abstract | This contribution proposes the compressed DL-MAP IE |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A more efficient DL-MAP IE for MAPs

Kanchei (Ken) Loa, Tsung-Yu Tsai, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin

Institute for Information Industry (III)

Introduction

In the current definition of the allocation of downlink data in WirelessMAN-OFDMA, each allocation (i.e. a burst) is described as a two-dimensional rectangle located in a table with time axis (OFDMA symbol) and frequency axis (sub-channel). In order to specify the burst with a 2-D allocation scheme, the existing DL-MAP IE, defined in P802.16Rev2/D4, utilizes four parameters, that are, a “OFDMA symbol offset” and a “OFDMA sub-channel offset” to locate the coordinate of the left-top point of the allocated rectangular, plus, “number of OFDMA symbols” and “number of OFDMA sub-channels” to indicate its length and width, respectively. This approach is intuitive, but carries 27-bit overhead.

For reducing the MAP overhead, this contribution proposes a compressed DL-MAP IE that just uses two parameters, which are “number of OFDMA symbols” and “number of sub-channels”, to perform the allocation description with 13-bit overhead. In addition, the mathematic proof is provided in the annex, which demonstrates that with the above two parameters the 2-D allocations of a DL zone can be uniquely determined as long as the sequence of Compressed DL-MAP IEs are ordered along the time axis in a frequency first manner.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D4 are listed below.

Spec changes

8.4.5.3 DL-MAP IE format

[Insert the following table at the end of 8.4.5.3 DL-MAP IE format:]

MR-BS and RS may transmit Compressed DL-MAP IE to describe the data burst on the DL relay link. The sequence of Compressed DL-MAP IEs in the MAP, if present, are ordered along the OFDMA symbol number (in a ascending order) in a subchannel logical number (in a ascending order) first manner.

Table 380a Compressed DL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Compressed_DL-MAP_IE() { | - | - |
| DIUC | 4 bits |  |
| if (DIUC == 14 { | - | - |
| Extended-2 DIUC dependent IE | variable |  |
| } else if (DIUC == 15) { | - | - |
| Extended DIUC dependent IE | variable | See subclauses following 8.4.5.3.1 |
| } else { | - |  |
| if (INC_CID == 1) { | - | The DL-MAP starts with INC_CID =0. INC_CID is toggled between 0 and 1 by the CID-SWITCH_IE() (8.4.5.3.7) |
| N_CID | 8 bits | Number of CIDs assigned for this IE |
| for (n=0; n< N_CID; n++) { | - | - |
| RCID_IE() | variable | For R-MAP, RS_Access-MAP and RS_Relay-MAP, reduced CID format is used |
| } | - | - |
| } | - | - |
| Boosting | 3 bits | 000: normal (not boosted); 001: +6dB; 010: -6dB; 011: +9dB; 100: +3dB; 101: -3dB; 110: -9dB; 111: -12dB; |
| if (Permutation = 0b11 and (AMC type is 2x3 or 1x6)) { | - | - |
| No. OFDMA triple symbol | 5 bits | Number of OFDMA symbols is given in multiples of 3 symbols |
| } else { | - | - |
| No. OFDMA Symbols | 7 bits |  |
| } | - | - |
| No. Subchannels | 6 bits |  |
| Repetition Coding Indication | 2 bits | 0b00 – No repetition coding 0b01 – Repetition coding of 2 used 0b10 – Repetition coding of 4 used 0b11 – Repetition coding of 6 used |
| } | - | - |
| Padding | variable | Shall be set to 0 |
| } | - | - |

[Modify line 13 of page 74 in Table 183c as indicated:]

6.3.2.3.87 RS access MAP (RS_Access-MAP) message

Table 183c—RS Access MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| Indicator | 8 bits | Bit 0: 0: Parameters of DL_Frame_Prefix remain same with the latest Configuration. 1: The parameters of DL_Frame_Prefix are updated. Bit 1: 0: RS shall use Normal map format, 1: RS shall use Compressed map format Bit 2: 0: DL-MAP not included 1: DL-MAP included Bit 3: 0: UL-MAP not included 1: UL-MAP included Bit 4: 0: SUB-DL-UL-MAP not included 1: SUB-DL-UL-MAP included Bit 5: 0: HARQ-MAP not included 1: HARQ-MAP included Bit 6: 0: Normal DL-MAP IE 1: Compressed DL-MAP IE Bit 6-7: reserved |

[Modify line 45 of page 74 in Table 183c as indicated:]

| DL_IE count | 8 bits | Number of DL_IE in the burst |
| --- | --- | --- |
| If(bit #6 of indicator == 0) { |  |  |
| for(i=0;i<DL_IE count; i++){ |  |  |
| If(RS DL fixed forwarding delay is not set and DIUC<=12){ |  |  |
| DL_Allocation_Reference_IE() | variable |  |
| } |  |  |
| DL-MAP_IE() | variable | “DL-MAP” in DL-MAP or Compressed DL-MAP |
| } |  |  |
| } else { |  |  |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID 11 0b10 = RCID 7 0b11 = RCID 3 |
| for(i=0;i<DL_IE count; i++){ |  |  |
| Compressed_DL-MAP_IE() | variable |  |
| } |  |  |
| } |  |  |

[Modify line 16 of page 212 in Table 496a as indicated:]

8.4.5.10 R-MAP message

Table 496a—R-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID 11 0b10 = RCID 7 0b11 = RCID 3 |
| DL-MAP IE format | 1 bit | 0b0 = Normal DL-MAP IE 0b1 = Compressed DL-MAP IE |
| Length | 11 bits | Length of R-MAP in bytes |
| DL_IE count | 6 bits | Number of DL_IE in the burst |
| UL_IE count | 6 bits | Number of UL_IE in the burst |
| for(i=0;i<DL_IE count; i++){ |  |  |
| If(DL-MAP IE format == 0b0) { |  |  |
| DL_-MAP_IE() | variable |  |
| } else { |  |  |
| Compressed_DL-MAP_IE() | variable |  |
| } |  |  |
| } |  |  |

--- End of Spec Change --

Annex

The mathematical proof is provided here to assure that the 2-D allocations is unique in a rectangular zone by using Compressed DL-MAP IEs.

Figure 1 Rectangular zone

In Figure 1, let the rectangular zone and the k-th burst denote as R and Bk, and the left-top corner of the R and Bk are pR  <cR, sR> and pk  <ck, sk>, where CR and SR are number of the sub-channel and the OFDMA symbol of the zone, respectively; Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; CR, SR, Ck, Sk  Positive Integer; cR, sR, ck, sk  Non-negative Integer.

- Define f(p) = a(p  pR) = (c  cR) + CR(s  sR), where a  (1, CR) and p  (c, s) represent the location of the point at the c-th sub-channel and the s-th OFDMA symbol. The following axioms shall be satisfied

- f(pR) = 0
proof:	f(pR) = a(pR  pR) = 0

- CRSR  1  f(p)  0 for every p  R
proof:	CR  1  c  cR  0 and SR  1  s  sR  0 for every p  R
	 f(p) = (c  cR) + CR(s  sR)  0 for every p  R
	 f(p) = (c  cR) + CR(s  sR)  (CR  1) + CR(SR  1) = CRSR  1 for every p  R
	proven

- f(pi) = f(pj) if and only if pi = pj, for every pi, pj  R

- f(pi)  f(pj) if and only if pi  pj, for every pi, pj  R
proof: () if ci = cj and si = sj
	 f(pi)  f(pi) = (ci  cj) + CR(si  sj) = 0
	 f(pi) = f(pj)
	() if ci  cj and si = sj
	 f(pi)  f(pi)| = (ci  cj) + CR(si  sj) = (ci  cj)  0
	 f(pi)  f(pj)
	() if si  sj
	 CR  1  |ci  cj|  0 and |si  sj|  1
	 |f(pi)  f(pj)| = |(ci  cj) + CR(si  sj)|  CR|(si  sj)|  |(ci  cj)|  CR  (CR  1) = 1
	 f(pi)  f(pj)
	()proven

- pk  Bk and f(pk)  f(p) for every p  Bk if and only if pk is the left-top corner of Bk
proof: () if pk  Bk and pk is the left-top corner of Bk
	 c  ck and s  sk for every p  Bk
	 f(p) = (c  cR) + CR(s  sR)  (ck  cR) + CR(sk  sR) = f(pk) for every p  Bk
	() let q  Bk and f(q)  f(p) for every p  Bk and pk is the left-top corner of Bk
	 f(pk)  f(q)  f(pk)
	 f(q) = f(pk)
	 from axiom (3), q = pk
	() proven

- Assume the whole zone is filled up non-overlapped bursts, i.e., R=i=1NBi and Bi  Bj =  for i  j. The following axioms shall be satisfied

- Let p  R, then we can find only one Bk  R such that p  Bk
proof:	assume p  Bk for every Bk  R
	 p∉∪i=1NBi=R
	 conflict, we can find at least Bl  R such that p  Bl
	assume we can find Bi, Bj  R such that p  Bi and p  Bj
	 p  Bi  Bj = 
	 conflict, we can find only one Bl  R such that (c, s)  Bl

- Let Qk=i=kNBi and q  Qk such that f(q)  f(p) for every p  Qk, then we can find only one Bl  Qk such that q is the left-top corner of Bl
proof:	assume q  Bl for every Bl  Qk
	 q∉∪i=kNBi=Qk
	 conflict, we can find at least Bl  Qk such that q  Bl
	f(q)  f(p) for every p  Qk
	 f(q)  f(p) for every p  Bl  Qk
	 from axiom (5), q is the left-top corner of Bl

- Assume the k-th burst IE, denotes as IEk, contains non-zero parameter (Ck, Sk) and appears in the MAP with following sequence IE1, …, IEk , …, IEN, where i < j if f(pi) < f(pj) and pk denotes the left-top corner of the k-th burst (Bk); Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; Ck, Sk  Positive Integer; ck, sk  Non-negative Integer. The following axioms shall be satisfied

- The sequence of IE: IE1, …, IEk , …, IEN is unique
proof:	Bi  Bj =  for i  j
	 pi  pj, where pk denotes the left-top corner of the k-th burst
	 from axiom (4), f(pi)  f(pj)
	 The sequence of IE: IE1, …, IEk , …, IEN is unique

- Let pR  <cR, sR> denotes the left-top corner of the zone (R), pk  <ck, sk> denotes the left-top corner of the k-th burst (Bk), Dk = {p| p  <c, s>; Ck > c  0 and Sk > s  0 } and Bk = Dk + pk = {p + pk| p  Dk}, then the estimated left-top corner of the k-th burst is qk can be determined as follows:
(a) The left-top corner of the 1st IE is q1 = pR.
(b) Let Q2 = R  C1 = {p| p  R and p  C1}, where C1 = D1 + q1 = {p + q1| p  D1}
(c) The left-top corner of the kth IE is qk such that f(qk)  f(p) for every p  Qk
(d) Let Qk+1≡Qk−Ck=p|p∈R and p∉∪i=1kCi, where Ck = Dk + qk = {p + qk| p  Dk}
The following axioms shall be satisfied

- Based on (Ck, Sk) and sequence IE1, …, IEk , …, IEN, qk = pk for i = 1 to N
proof: from axiom (5) and (6)
	 we can find only one Bk  R such that pR  Bk and pR is the left-top corner of Bk
	 Bk = B1 since f(pR) < f(pk) for every pk  R and pk  pR
	assume qk = pk for k = 1 …, n
	 Cn = Dn + qn = Dn + pn = Bn
		and Qn+1≡Qn−Bn=p|p∈R and p∉∪i=1nBi=p|p∈∪i=n+1NBi
	 let qn+1 is the point such that f(qn+1)  f(p) for every p  Qn+1
	 from (7), we can find only one Bl  Qn+1 such that qn+1 is the left-top corner of Bl
	 qn+1=pl∈pk|k=n+1,...,N
	 qn+1 = pl = pn+1 since f(qn+1)  f(pn+1)  f(pk) for every pk  Qn+1

- Let Bm is a known rectangular zone with (Cm, Sk) and the left-top corner pm and let R'=R−Bm=i=1NBi. Assume the k-th burst IE, denotes as IEk, contains non-zero parameter (Ck, Sk) and appears in the MAP with following sequence IE1, …, IEm-1, IEm+1, …, IEN, where i < j if f(pi) < f(pj) and pk denotes the left-top corner of the k-th burst (Bk); Ck and Sk are numbers of the sub-channel and the OFDMA symbol of the k-th burst, respectively; Ck, Sk  Positive Integer; ck, sk  Non-negative Integer.
Let pR  <cR, sR> denotes the left-top corner of the zone (R), pk  <ck, sk> denotes the left-top corner of the k-th burst (Bk), Dk = {p| p  <c, s>; Ck > c  0 and Sk > s  0 } and Bk = Dk + pk = {p + pk| p  Dk}, then the estimated left-top corner of the k-th burst is qk can be determined as follows:
(a) The left-top corner of the 1st IE is q1 = pR.
(b) Let Q2 = R  C1 = {p| p  R and p  C1}, where C1 = D1 + q1 = {p + q1| p  D1}
(c) The left-top corner of the kth IE is qk such that f(qk)  f(p) for every p  Qk
(d) Let Qk+1≡Qk−Ck=p|p∈R and p∉∪i=1kCi, where Ck = Dk + qk = {p + qk| p  Dk 
The following axioms shall be satisfied

- The sequence of IE: IE1, …, IEm-1, IEm+1, …, IEN is unique
proof:	Bi  Bj =  for i  j
	 pi  pj, where pk denotes the left-top corner of the k-th burst
	 from axiom (4), f(pi)  f(pj)
	 The sequence of IE: IE1, …, IEm-1, IEm+1, …, IEN is unique

- Based on (Ck, Sk) and sequence IE1, …, IEm-1, IEm+1, …, IEN, qk = pk for i = 1 to N
proof: from axiom (9), we can find qk = pk for i = 1 to m  1
	since Bm is a known rectangular zone thus qm = pm
	then from axiom (9), we can find qk = pk for i = m + 1 to N