| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on ARQ operation |  |
| Date Submitted | 2008-05-08 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Jungje Son Samsung Electronics | E-mail: yb.chang@samsung.com |
|  | Rakesh Taori Samsung Advanced Institute of Technology, Korea | E-mail: rakesh.taori@samsung.com |
| Re: | Call for comments P802.16j/D4 |  |
| Abstract | Clarification of ARQ operation in tunnel packet mode |  |
| Purpose | Adopt the proposed text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on ARQ operation

Youngbin Chang, Youngbin Chang, Hyunjeong Kang, Jungje Son

Samsung Electronics

Rakesh Taroi

Samsung Advanced Institute of Technology

# Introduction

# This contribution includes ARQ feedback IE clarification in tunnel packet mode.

# In the two-link ARQ mode, the access RS sends TSN and FO within ARQ feedback IE.  However, when the access RS sends the ARQ feedback IE with FO, MR-BS does not have FO information which Intermediate RS have fragmented. Therefore, in two-link ARQ mode, TSN information is only valid to send ACK/NAK. In this case, all ARQ feedback type is possible.

# In the hop-by-hop ARQ mode, the access RS can send TSN and FO with ARQ feedback IE because FO information is available in the intermidate RS. When FO is used for ARQ feedback, the only cumulative ARQ feedback is possible.

# Specification changes

[Change the first paragraph in section 6.4.4.2.2 on page 85 as following:]

6.3.4.2.2 ARQ Feedback IE for Tunnel packet

Table 167a represents the ARQ feedback IE for tunnel packet in a hop-by-hop ARQ mode, table 167b represents the ARQ feedback IE for tunnel packet in a two-link ARQ mode These two IEs are used between MR-BS and access RS by the receiver to signal positive or negative acknowledgments. A set of IEs of this format may be transported either as a packed payload ("piggybacked") within a packed MAC PDU or as a payload of a standalone MAC PDU.

[Modify the title in the table 167a on page 86 as following:]

Table 167a – ARQ Feedback IE format for tunnel packet in hop-by-hop ARQ

[Add a new table after table 167a:] [‘CID’ and ‘BSN’ is changed into ‘T-CID’ and ‘TSN’ in current 16e specification]

Table 167b – ARQ Feedback IE format for tunnel packet in two-link ARQ

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | Variable |  |
| T-CID | 16 | The ID of the tunnel connection |
| LAST | 1 | 0 = More ARQ feedback IEs in the list 1 = Last ARQ feedback IE in the list |
| ACK Type | 2 | 0x0 = Selective ACK entry 0x1 = Cumulative ACK entry 0x2 = Cumulative with Selective ACK entry 0x3 = Cumulative ACK with Block Sequence ACK entry |
| TSN | 7 | TDU sequence number |
| Number of ACK Maps | 2 | If ACK Type == 01, the field is reserved and set to 00. Otherwise the field indicates the number of ACK maps: 0x0 = 1, 0x1 = 2, 0x2 = 3, 0x3= 4 |
| If (ACK Type != 01) { |  |  |
| for (i = 0; i< Number of ACK Maps + 1; ++i) { |  |  |
| if (ACK Type != 3) { |  |  |
| Selective ACK Map | 16 |  |
| } |  |  |
| Else { |  | Start of Block Sequence ACK Map definition (16 bits) |
| Sequence Format | 1 | Number of block sequences associated with descriptor 0: 2 block sequences 1: 3 block sequences |
| if (Sequence Format = 0) { |  |  |
| Sequence ACK Map | 2 |  |
| Sequence 1 Length | 6 |  |
| Sequence 2 Length | 6 |  |
| Reserved | 1 |  |
| } |  |  |
| else{ |  |  |
| Sequence ACK Map | 3 |  |
| Sequence 1 Length | 4 |  |
| Sequence 2 Length | 4 |  |
| Sequence 3 Length | 4 |  |
| } |  |  |
| } |  | End of Block Sequence ACK Map definition |
| } |  |  |
| } |  |  |
| Reserved | 4 |  |
| } |  |  |