| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Full Duplex Relay |  |
| Date Submitted | 2008-05-09 |  |
| Source(s) | Takki Yu, Seung Hee Han, Sungyoon Jung, Jungje Son, Youngbin Chang, Hyunjeong Kang Samsung Electronics Co., Ltd. Korea Rakesh Taori Samsung Advanced Institute of Technology, Korea | Voice: +82 31 279 4978 E-mail: tk.yu@samsung.com sh0506.han@samsung.com jungje.son@samsung.com E-mail: rakesh.taori@samsung.com |
| Re: | LB#28c |  |
| Abstract | This contribution proposes to include the full duplex relaying as one of the possible options in 802.16j standard. |  |
| Purpose | Accept proposed changes into IEEE P802.16j/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Full Duplex Relay

Takki Yu, Seung Hee Han, Sungyoon Jung, Jungje Son, Youngbin Chang, Hyunjeong Kang

Samsung Electronics

,Rakesh Taori

Samsung Advanced Institute of Technology

# Introduction

Full duplex relay station can transmit and receive simultaneously in a single frequency band leading to the following advantages:

- Potential reduction in spectral efficiency loss : re-utilizes BS-RS link resources as RS-MS link resources

- Reduced overhead : no need for R-TTG and R-RTG, relay amble, and duplicated broadcasting information

- No loss in uplink coverage : an MS can send data over the whole UL frame to increase uplink coverage

However, full duplex relaying is not defined as relaying mechanism in the current draft. A full duplex relay station can be readily used in some of the usage models of 802.16j as shown in Figure 1. In the usage models such as underground and coverage hole where sufficient level of isolation between relay transmit and receive antennas is likely to be present, we can exploit the benefits of full duplex relaying.

This contribution proposes that full duplex relaying be included as one of the possible options in the 802.16j standard. Details of the operation and the corresponding text for inclusion in the draft is provided here.

Figure 1. Usage model of full duplex

# Overall description on “Full duplex relaying”

We suggest that the following definitions be included :

* Full duplex relaying (FDR) : a capability of communicating with the superordinate station and subordinate station(s) simultaneously, on the same carrier frequency.

* Full duplex RS: a non-transparent relay station capable of full duplex relaying.

Communication with a superordinate station and subordinate station(s) is performed using two different antenna sets where one of the antenna set is used for communicating with the superordinate station while at the same time the other antenna set is used for communicating with the subordinate station(s). FDR operation may cause interference from relay transmit antenna set to relay receive antenna set because it receives and transmits simultaneously using the same time/frequency resources. Hence, in order to achieve benefits from full duplex relay, a certain degree of isolation might be needed between the relay transmit and receive antenna sets. In some usage models, because of the nature of the usage scenario, sufficient levels of isolation may be available. For example, FDR may be an efficient relay solution to the underground usage model by locating the antenna communicating with BS outside and antenna communicating with MSs inside.

Figure 2 shows the high-level frame structure of full duplex relay for a two-hop case. The approach to the frame structure is same as that in [2], which preserves the consistency with the current frame structure except simultaneous operation of transmitting and receiving on the same carrier.

Figure 2. Tx/Rx configuration of FDR in case of two hop frame structure

# Full duplex RS frame structure

The frame structure of full duplex RS is shown in Figure 3. The first and second antenna sets operate on the same carrier frequency.

Figure 3. An example of frame structure for full duplex RS.

# Network entry – Capability negotiation

An RS having a capability of full duplex relaying can inform the MR-BS during network entry using SBC negotiation.

# Reference:

[1] IEEE P802.16j/D4 April 2008

[2] IEEE C802.16j-08/079r1 “Out-of-band relay clarification” March 2008

# Proposed changes to the draft

Insert new definition 3.125:

3.125 full duplex relaying (FDR) : a capability of communicating with the superordinate station and subordinate station(s) simultaneously, on the same carrier frequency.

3.126 full duplex RS: a non-transparent relay station capable of full duplex relaying.

Insert the following text in a new section after section 8.4.4.7.2.3

8.4.4.7.2.x. Full duplex relay frame structure

A full duplex RS is capable of communicating with subordinate station(s) and a superordinate station simultaneously by using two different antenna sets using the same carrier frequency. One of the antenna set is used for communicating with the superordinate station while at the same time the other antenna set is used for communicating with the subordinate station.

An example of a full duplex RS frame structure is shown in Figure xxx, where the carrier frequency of the first  and second antenna set is the same. If the RS is not supporting subordinate RSs, then the ‘DL relay zone’ and the ‘UL relay zone’ shown in Figure xxx will not be present.

Figure xxx. Example of minimum configuration for a full duplex relay frame structure

Change 11.8.3.7.22 as indicated:

11.8.3.7.22 MR PHY feature support

This TLV indicates the MR PHY features supported by the RS and the MR-BS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 206 | 1 | Bit #0: Access zone preamble transmission support Bit #1: MBS Data Synchronization with pre-defined relative transmission time (6.3.23.3) Bit #2: MBS data synchronization with target transmission time (6.3.23.3) Bit #3: cooperative relay support Bit #4: support of a second carrier frequency at RS (see 8.4.4.7.2.2) Bit #5: support dual radio RS operation (see 8.4.4.7.2.3) Bit #6: support FDR operation (see 8.4.4.7.2.x) Bits #6-7: Reserved | SBC-REQ SBC-RSP |

Bit #4 can only be set to 1 if bit #0 is set to 1.  Bit #5 can only be set to 1 if both bits#0 and #4 are set to 1.