| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on R-FCH |  |
| Date Submitted | 2008-05-15 |  |
| Source(s) | Gamini Senarath, Israfil Bahceci, Hang Zhang,Peiying Zhu, Wen Tong, Derek Yu, Sun Sheng, Mark Naden Nortel Networks 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | E-mail: gamini@nortel.com |
| Re: | IEEE 802.16-08/020: “IEEE 802.16 Working Group Letter Ballot Recirc. #28b: Announcement” |  |
| Abstract | This contribution proposes remedies to the dynamic frame configuration in IEEE802.16j/D4. |  |
| Purpose | Discuss and adopt proposed text in TG16j |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on R-FCH

Senarath Gamini, Israfil Bahceci, Hang Zhang, Peiying Zhu, Wen Tong, Derek Yu, Sun Sheng, Mark Naden

Nortel

### 1 Overview

The frame configuration in 16j networks can be done either in a static manner using MAC messages, or in a dynamic manner using R-FCH.  In fact, both methods should imply the same frame structure; however, due to propagation delay for frame structure information along the paths, MAC message based frame configuration may cause inefficient scheduling and inefficient use of resources due to propagation delay to forward the MAC messages from the MR-BS station to the destination station.

The R-zone Location field in R-FCH enables subordinate RSs to locate the next transmission of R-FCH, thereby allowing subordinate RSs to determine the resource allocation on a per frame basis.

In case of any modifications in frame configuration, there are two mechanisms to determine the frame configuration: (i) by the RCD message and (ii) by R-FCH/R-MAP message. On the one hand, one could follow R-FCH per frame to locate the next R-FCH/RMAP transmission from the serving station. On the other hand, it is possible to operate based on RCD to determine frame configuration. Therefore, it is necessary to specify which method shall be followed at the RSs.

In this contribution, for two hop systems R-FCH/R-MAP method is proposed when there are only access zone and RS xone in the frame configuration.

### 2 Problems with R-FCH and Remedies

In current P802.16j/D4 document, R-Zone location indicates the offset (relative to the frame start preamble) for the first OFDM symbol of R-FCH/R-MAP allocation. Whenever the frame configuration is updated, the subordinate RSs has to be provided sufficient time to update their scheduling at which the new configuration is effective.

Topology with 2-hops { option 1}

The R-FCH operation described in P802.16j//D4 may fail at various situations. For example, as shown in Figure 1, if R-FCH/R-MAP occurs at very frame, and that the R-zone location needs to be updated at some frame-k, an update on R-FCH location my conflict with RS’s current UL scheduling since the UL MAP that the RS already transmitted at frame-k provides allocation information at frame-(k+1). The RS has no time to update its UL allocation. In addition, any UL MAP IE in R-MAP at frame-k is affective at frame k+ Relay UL Allocation Start Time, where Relay UL Allocation Start Time is obtained from RCD message. Therefore, any updated R-FCH should be affective no earlier than max(2, Relay UL Allocation Start Time+1) frames in which an R-FCHs are transmitted.

### 3 Required Text Changes

[Insert the following text to Line 31 of Page 50 in Subclause 6.3.2.3.69]

The RS Config CMD message may include the following TLV to indicate the mechanism to determine the frame configuration of the superordinate station.

Frame_Config_Flag (See 11.25.7)

6.3.9.18.1 Parameter configuration

[Insert the following paragraph after Line 59 of Page 106 in Subclause 6.3.9.18.1]

Frame configuration during operational mode may be done either in a quasi-static manner using RCD and RS Config CMD MAC messages (See 6.3.2.3.65 and 6.3.2.3.69), or in a dynamic manner using R-FCH (See 8.4.4.7.4). The Frame Config Flag TLV may be transmitted in RS Config CMD message to indicate which mechanism to be followed for determining the superordinate station’s frame structure in the upcoming frame(s).

[Change Subclause 8.4.4.7.3 as indicated]

8.4.4.7.3 R-FCH channel

If a DL relay zone of a non-transparent RS contains an R-FCH, the R-FCH shall be transmitted using the

same approach as that described for the FCH in 8.4.4.2. The R-FCH contains the R-Zone_Prefix as

described in 8.4.4.7.4. For two-hop system, an RS receiving an updated R-FCH at frame-k shall apply the changes at frame k + Relay UL Allocation Start Time+1. The Relay UL Allocation Start Time is provided from RCD. If an R-FCH is lost, the RS shall attempt to resynchronized again using the location specified by the previous R-FCH. If the synchronization fails, it may try to obtain R-FCH using the last received RCD information. If this fails, the RS shall stop transmitting the frame start preamble, listen to access zone and obtain relay zone information from Gap IE or STC zone switch IE and synchronize to the R-FCH/R-MAP.

[Change Subclause 8.4.4.7.4 as indicated]

8.4.4.7.4 R-Zone prefix

The R-Zone_Prefix is a data structure transmitted on R-FCH of a DL relay zone. The R-Zone_Prefix includes information regarding the location of the first transmit relay zone in the next frame that contains a transmit relay zone and the information required for decoding R-MAP. In the case of two hop systems, the R-Zone configuration can be done using R-FCH when the relay frame structure has no idle zones and one relay zone.

For all the other cases, if the case that RS_config-CMD message contains the Frame_Config_Flag set to 1, the frame configuration TLVs in the RCD message shall be followed to locate the relay zones. Otherwise, the R-FCH/R-MAP shall be used to determine the frame configuration parameters. Table 377a defines the format of the R-Zone_Prefix for FFT sizes other than 128 and Table 377b defined the format for a FFT size of 128.

[Insert the following text to Subclause 11.24]

11.24.9 Frame Config_Flag

| Frame Config Flag | TBD | Bit 0: set to 1, it indicates that the RS shall use the RCD message to obtain frame configuration. If it is set to 0, R-FCH/R-MAP messages shall be used for determining the frame structure. Bit1-7: reserved |
| --- | --- | --- |

[Change 3rd row of Table 377on Page 197 as indicated]

| R-Zone_Location | 8 | The field indicates the location of the first transmit relay zone in the next frame that contains a relay zone. For a two hop case, when the Frame_Config_Flag is set to 0 , if the RS receives an updated value of this parameter, the frame configuration changes shall be applied at frame max(2, Relay UL Allocation Start Time+1). |
| --- | --- | --- |

[Change 3rd row of Table 377b on Page 198 as indicated]

| R-Zone_Location | 8 | The field (unsigned integer) indicates the OFDM symbol index referenced to the beginning of next frame that contains a transmit relay zone in unit of m OFDM symbols where: m=floor (L/11); L= Number of OFDM Symbols in one frame; and OFDM Symbol Index = 3 + R-Zone_Location * m For a two hop case, when the Frame_Config_Flag is set to 0 , if the RS receives an updated value of this parameter, the frame configuration changes shall be applied at frame max(2, Relay UL Allocation Start Time+1). |
| --- | --- | --- |

This field provides an offset in number of OFDM symbols relative to the preamble of the next frame which contains an R-FCH (according to 16j/D4).