| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on data forwarding schemes for the transparent RS |  |
| Date Submitted | 2008-09-10 |  |
| Source(s) | Kanchei (Ken) Loa, Yung-Ting Lee, Yi-Hsueh Tsai, Hua-Chiang Yin, Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) 7F, No. 133, Sec. 4, Minsheng E. Rd., Taipei City 105, Taiwan Shiann-Tsong Sheu National Central University (NCU) Yang-Han Lee, Yih-Guang Jan Tamkang University (TKU) [add other co-authors here] | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw stshue@ce.ncu.edu.tw yhlee@ee.tku.edu.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | Comment on data forwarding schemes for the transparent RS |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on data forwarding schemes for the transparent RS

Kanchei (Ken) Loa, Yung-Ting Lee, Yi-Hsueh Tsai, Hua-Chiang Yin, Youn-Tai Lee, Chun-Yen Hsu 
Institute for Information Industry (III)

Shiann-Tsong Sheu
National Central University (NCU)

Yang-Han Lee, Yih-Guang Jan
Tamkang University (TKU)

Problem Statement

There are three basic data forwarding schemes defined in IEEE P802.16j/D6a (1) CID based forwarding for DL/UL, (2) CID based forwarding assisted by inserting DL Allocation Reference IEs in the MAPs for RS with centralized scheduling and variable forwarding delay (3) burst-based forwarding for UL during IR, BR and HR.

The CID based forwarding schemes require CID forwarding rules for DL/UL maintains at each RS, which have the benefit of zero signaling overhead to execute the forwarding operation. However, in a dynamic environment where caused by either the MS mobility or the CDMA ranging (PR and BR), the CID forwarding rules require frequent changes. Every time the MS sends a CDMA code for PR or BR, the access station (transparent RS or member of the RS group) may be re-selected by the MR-BS as shown in Fig. 1.

In order to meet the QoS requirements for the MS, the “Changing Data Forwarding Rules” procedure in Fig.1 shall be accomplished as fast as possible. In IEEE P802.16j/D6a, there are two approaches defined for the “Data Forwarding Rules Changed”:

- Approach 1: the MR-BS may send the DSx messages to update the CID list of the transparent RSs. The approach is robust since it is a three-way handshake process. However, updating CID tables via DSx messages incurs overheads and causes disruption in the service. The service outage increases forwarding latency that is proportional to the frame duration, the RS hop count and the message lost probability.

- Approach 2: the MR-BS may utilize the RS_Member_List_Update (see 6.3.2.3.83) instead of DSx messages to update the CID list of the transparent RSs for CID based forwarding in both DL and UL. The RS_Member_List_Update require two-way handshake messages exchange only. However, there exist potential risks of inconsistent data forwarding rules at involved RSs when either RS_Member_List_Update or associated ACK cannot be correctly received. One way to resolve the inconsistent data forwarding rules is to set a larger “Frame Number Action” in RS_Member_List_Update. Before the “Frame Number Action” expires, the MR-BS could retransmit RS_Member_List_Update to the RSs who did not ACK. It, however, wastes the radio resources and increases the possibility of violating QoS requirements.

Figure 1 Example of MR-BS re-selecting RS2 as the access station of MS after receiving BR ranging code

Proposed Solution

In order to resolve the problems mentioned above, the burst-based forwarding scheme is proposed to complement the CID based forwarding schemes to achieve zero service disruption.

- First, when MR-BS reselects the access station for the MS, the MR-BS performs the “Changing Data Forwarding Rules” procedure by sending RS_Member_List_Update to the involved RSs and waits for associated ACKs to finish the procedure.

- Before the “Changing Data Forwarding Rules” to be finished, the burst-based forwarding scheme may be used to set temporal forwarding rules at the involved RSs for forwarding data to the MS such that there is no disruption in the services. An example is given in Figure. 2.

Figure 2 Example of MR-BS using burst-based forwarding scheme to serve MS through RS2.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D6a are listed below.

Spec changes

6.3.3.8.2 Transmission using station CID

[Change the following text in line 3 of page 86 as indicated]

There are two schemes for RS to forward received data without using tunnels. The first one is called CID based forwarding and the other is burst-based forwarding. In CID based forwarding scheme, the forwarding of MAC PDUs by each RS is performed based on the CID contained in the MAC PDU header. When forwarding using this scheme, the inclusion of CID in the DLMAP is optional. When operating in centralized scheduling mode, if the RS DL fixed forwarding delay TLV or the RS UL fixed forwarding delay TLV is included in RCD message, the RS shall forward each MAC PDU at the frame determined by the receiving frame of the MAC PDU plus the fixed delay specified in the TLV. If the RS DL fixed forwarding delay TLV is not included in RCD, the MR-BS shall include DL Allocation Reference IEs in RS_Relay-MAP and RS_Access-MAP messages.

For transparent RSs, the MR-BS shall send the RS_Member_List_Update (see 6.3.2.3.78) to update the CID list of the transparent RSs for CID based forwarding in both DL and UL. If the MR-BS does not receive MR_Generic-ACK message from all RSs designated in the RS_Member_List_Update message after the Frame Action Number, the burst-based forwarding scheme may be used to replace its original forwarding rules to forward the data to the MS.

In burst-based forwarding scheme, the forwarding of the bursts is performed by encoding the forwarding rules in MAP IEs. In DL, the data burst that are scheduled to be relayed by the RS is described by the DL-MAP IE with the RS primary management CID. The MR-BS shall send the forwarding rules to the RS via R-MAP including the DL_Burst_Transmit_IE (see 8.4.5.10.1.xxx). The RS shall forward the data in allocations defined by those DL MAP IEs indicated by the DL_Burst_Transmit_IE. In UL, the data burst that are scheduled to be relayed by the RS(s) is described by the UL-MAP IE with UL_Burst_Receive_IE (see 8.4.5.4.32). The RS shall receive the data in allocations defined by those UL MAP IEs following the UL_Burst_Receive_IE and forward to its superordinate station in the next available allocation, defined by the UL-MAP IE, in UL relay zone.

[Modified Table 458a in Line 32, Page 223 as follows]

| R-link specific IE | variable | For non-transparent mode RSs only |
| --- | --- | --- |

[Modified Table 458c in Line 49, Page 224 as follows]

| 0D | DL Burst Transmit IE |
| --- | --- |
| 0C0E-1F | Reserved |

[Add the following subclause 8.4.5.10.1.12 in line 51 of page 244 as indicated]

8.4.5.10.1.14 DL Burst Transmit IE

An MR-BS may send R-MAP including DL_Burst_Transmit IE to the subordinate RSs to indicate the bursts to be forwarded by Ns and Nr in the IE. The Lk included in DL_Burst_Transmit IE refers to number of bytes in the burst which is forwarded by the RS.

Table 458q—DL Burst Transmit IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| DL_Burst_Transmit_IE{ | - | - |
| Type | 4 bits | DL_ Burst_Transmit_IE = 0x0D |
| Length | 8 bits |  |
| RCID_IE() | variable | Reduced RS basic CID |
| Ns | 8 bits | The first DL-MAP IE number in DL-MAP the transparent RS shall forward |
| Nr | 8 bits | Indicate the number of MAP IEs in DL-MAP the transparent RS shall forward |
| for(k=1;k<=Nr;k++){ | - | - |
| Lk | 16 bits | Burst length in bytes to be forwarded |
| } |  |  |
| Padding | variable | Shall be set to 0 |
| } |  |  |

11.7.28 MR-BS and RS MAC feature support

[Change the following text in line 26 of page 261 as indicated]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 49 | 3 | Bit #19: Burst-based forwarding support Bit #1920-23 : Reserved | REG-REQ REG-RSP |

11.25 RS_Config-CMD message TLV encoding

Insert new subclause 11.25.1:

11.25.1 Generic configuration

[Change the following text in line 4 of page 289 as indicated]

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| RS operational mode | 1 | 2 | Bit #11: Burst-based forwarding Bit #1112-15: Reserved | RS_Config_CMD |