| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamically relay zone configuration for two-hop relay |  |
| Date Submitted | 2008-07-16 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Youn-Tai Lee Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Yang-Han Lee, Yih-Guang Jan Tamkang University (TKU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw stsheu@ce.ncu.edu.tw yhlee@ee.tku.edu.tw |
| Re: | IEEE 802.16-08/028: “IEEE 802.16 Working Group Letter Ballot Recirc #28d: Announcement” |  |
| Abstract | Dynamically relay zone configuration |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamically relay zone configuration for two-hop relay

Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Hua-Chiang Yin, Youn-Tai Lee

Institute for Information Industry (III)

Shiann-Tsong Sheu

National Central University (NCU)

Yang-Han Lee, Yih-Guang Jan

Tamkang University (TKU)

Introduction

In current 16j/D5 standard, for the MR system using non-transparent RSs, the start point of the first DL relay zone is described by the R-Zone_Location in the previous R-FCH, thus the MR-BS can dynamically configure the downlink relay zone for 2-hop relay. However, the uplink relay zone can only be configured by the RCD message, which incurs high overhead when the configuration of uplink relay zone is changed frequently. (see Figure 1)

Figure 1 Relay zone configured by RCD

In order to support efficient dynamically relay zone configuration for the 2-hop non-transparent RS operated in distributed scheduling, we propose adding a new R-link specific IE, namely UL relay zone configuration IE, containing the four parameters to describe the UL relay zone for R-MAP.

In addition, any UL allocation in R-MAP is relative to UL Allocation Start time for RS, which can be greater than or equal to zero. Meanwhile, any UL allocation in the access zone refers to next frame. So, when RS sends its UL access zone's scheduling info to its MS at frame-k, it refers to frame k+1, e.g., frame k+1 already scheduled. So, any dynamic update on frame configuration should refer to at least 2 frames later than the reception time of R-MAP if an IE in the R-MAP will be used to update configuration. Assume UL Allocation start time for RS is 0 or 1. In these cases, RS will not be able to update its UL access zone scheduling. Even if UL Allocation start time for RS is 2, it will have a very short time to update/re-compute its UL scheduling, e.g.., the interval between R-MAP (it receives) and the following UL MAP (it will transmit) can be as low as 2 ms. Therefore, a separate timer “UL relay zone configuration action frame” for this specific IE is proposed.

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D5 are listed below.

Spec changes

[Modify the following text in line 24 of page 229 as indicated:]

8.4.5.10.1 R-link specific IE

Table 496c—R-link specific IE types

| Type (hexadecimal) | Usage |
| --- | --- |
| 0C | UL relay zone configuration |
| 0C0D-1F | Reserved |

8.4.5.10.1.11 UL relay zone configuration IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| UL relay zone configuration IE_Format() { | - | - |
| Type | 5 bits | Relay zone configuration IE = 0x0C |
| Length | 4 bits | Length = 2 or 4 |
| OFDMA symbol offset | 8 bits | This value indicates start symbol offset of all subsequent UL allocations in this R-MAP message. The reference point of this offset is the start of UL sub-frame. |
| No. of OFDMA symbol | 7 bits |  |
| Indicator | 1 bit | 0x0b0: All subchannels in UL are used 0x0b1: Partial subchannels in UL are used |
| If(Indicator == 0b1) { | - |  |
| Subchannel offset | 7 bits | This value indicates start subchannel offset of all subsequent UL data burst allocations in this message (R-MAP). |
| No. of subchannel | 7 bits |  |
| reserved | 2 bits | Shall be zero |
| } | - | - |
| } | - | - |

11.24.1 Generic channel description

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Relay UL allocation start time for operational RS | 7 |  | Indicates the effective start time of the uplink allocation defined by the R-MAP on R-link | RCD |
| UL relay zone configuration action frame | 8 | 1 | Indicates the effective action frame of the UL relay zone configuration by the R-MAP on R-link | RCD |

Relay UL allocation start time

UL allocation start time indicates the effective start time of the uplink allocation defined by the R-MAP on R-link. If the effective start time is defined as 0, the uplink allocation defined by the R-MAP is effective in the current frame; if the value is set to N, the uplink allocation defined by the R-MAP in frame i is effective in frame i + N.

UL relay zone configuration action frame

UL relay zone configuration action frame indicates the effective action time of the UL relay zone configuration defined by the R-MAP on R-link. If the effective action time is defined as 0, the UL relay configuration defined by the R-MAP is effective in the current frame; if the value is set to N, the configuration defined by the R-MAP in frame i is effective in frame i + N.