| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on RCD multicast |  |
| Date Submitted | 2008-07-07 |  |
| Source(s) | Yang Liu, Hongyun Qu, Yuqin Chen ZTE Corporation 706/4, Liantang Pengji Industrial Park, Shenzhen, P.R.China 518004 | E-mail: [liu.yang8@zte.com.cn] |
| Re: | This is in response for call for comments P802.16j/D5 |  |
| Abstract | This contribution proposes a method for RCD multicast |  |
| Purpose | To review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on the RCD multicast

Yang Liu, Hongyun Qu, Yuqin Chen

ZTE Corporation

# Introduction

Currently, the 16j D5 defines RCD message may be multicast as indicated below. However, the case of how RCD is used in a group of RSs is not well defined.

6.3.2.3.65 R-link channel descriptor (RCD) message

The RCD message consists of TLVs that provide configuration parameters that may be applicable to more

than one RS. It shall be transmitted by an MR-BS to all RSs by using the fragmentable broadcast CID or to

an RS on the RS’s primary management CID.

For instance, for two RSs (RS1 and RS2 in Fig.1) with distributed scheduling which is distant to each other and therefore can share a same piece of resource block, RCD may be used to configure them in a multicast manner.

Fig.1 Topology of two RSs with distributed scheduling sharing a same piece of resource block

# Proposed Changes

[Modify the description of the first paragraph of 6.3.2.3.65 on Page 39 as indicated:]

6.3.2.3.65 R-link channel descriptor (RCD) message

The RCD message consists of TLVs that provide configuration parameters that may be applicable to more

than one RS. It shall be transmitted by an MR-BS to all RSs by using the fragmentable broadcast CID or to

an RS on the RS’s primary management CID or several RSs sharing a same multicast CID. The message can be sent over the relay zone in operation mode or in the access zone (unicast) during the neighborhood measurement phase (if neighbor measurement is required by the MR-BS) and during the configuration phase for providing relay link usage and configuration information in the network entry procedure. It is also used to inform subordinate RSs of any changes to the configuration of the relay link that may occur after the RS has entered the network. In addition, this message is used to configure several distant RSs with distributed scheduling simultaneously on one multicast CID.