| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ feedback correction for tunnel packet |  |
| Date Submitted | 2008-07-08 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics, Suwon, Korea | E-mail: yb.chang@samsung.com |
| Re: | IEEE802.16-08/028”IEEE 802.16 Working Group Letter Ballot Recirc #28d: Announcement” |  |
| Abstract | This contribution proposes ARQ feedback IE correction on Table 167a. |  |
| Purpose | Discuss and adopt the proposed text in the TG16j draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ feedback correction for tunnel packet

Youngbin Chang, Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung Electronics

# 1. Introduction

The current ARQ feedback IE for tunnel packet in [1] has inefficiency because only cumulative ACK is possible. When the first tunnel packet or fragmented tunnel packet is failed, the proceeding successful tunnel packets will not be reported to BS or superordinate RS before first tunnel packet is recovered.

To solve this problem, the modified ARQ feedback IE is proposed. This modified IE contain cumulative ACK concept, but TSN indicates the highest TSN value of TDU received successfully in the receiver. If some of error TDUs or TDU fragments are exist in lesser TSN value, ARQ feedback IE contain only these error TDUs or TDU fragments information. This ARQ feedback IE can be applied to both two-link ARQ and hop-by-hop ARQ.

# 2. Proposed text change

[Modify the table 167a as follows:]

Table 167a—ARQ Feedback IE format for Tunnel packet

| Syntax | Size | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST) { | variable |  |
| T-CID | 16 | The ID of the tunnel connection |
| LAST | 1 | 0 = More ARQ feedback IEs in the list 1 = Last ARQ feedback IE in the list |
| TSN | 7 | TDU sequence number |
| FI | 1 | 0x0 = Unfragmented 0x1 = Fragmented |
| If(FI = 1) { |  |  |
| S_FO | 12 | Start fragment offset in byte |
| E_FO | 12 | End fragment offset in byte |
| } |  |  |
| N_TDU | 7 | Number of error TDUs or TDU fragments |
| For(i=0; i<N_TDU; i++) { |  |  |
| TSN | 7 |  |
| FI | 1 | 0x0 = Unfragmented 0x1 = Fragmented |
| If(FI = 1) { |  |  |
| S_FO | 12 | Start fragment offset in byte |
| E_FO | 12 | End fragment offset in byte |
| } |  |  |
| } |  |  |
| Padding |  |  |
| } |  |  |

[Replace the text in line 23-32 as follows:]

TSN

TSN value indicates the highest TSN value of TDU successfully received in the receiver.

FI

FI(Fragment Indicator) represent whether TSN value of TDU is fragmented or not

S_FO

S_FO value indicates start point of TDU fragment in byte corresponding to TSN.

E_FO

E_FO value indicates end point of TDU fragment in byte corresponding to TSN.

N_TDU

Number of error TDUs or TDU fragments. When N_TDU =0, all TDUs with equal and lesser TSN values have been successfully received in the receiver.

Reference:

[1] IEEE P802.16j/D5 May 2008