| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Replacement of single and dual radio terminology |  |
| Date Submitted | 2008-07-15 |  |
| Source(s) | Mike Hart UK Broadband Ltd. 6 The Square, Stockley Park East Uxbridge, UB11 1FW, UK | E-mail: mike.hart@ukbroadband.com |
| Re: | LB#28d |  |
| Abstract | This contribution proposes to replace the terms single and dual radio with time separated transmit receive (TTR) and simultaneous transmit receiver (STR) |  |
| Purpose | Accept proposed changes |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Replacement of single and dual radio terminology

Mike Hart

UK Broadband Ltd.

# Introduction

It is generally appreciated that the terminology “single radio” and “dual radio” relay and reference to radio hardware causes some ambiguity in the current version of the draft. This contribution proposes updates to the draft to address this issue by using alternative terminology.

# Proposed changes to the draft

Change part of the subclause 1.6 (line 5-13, page 3) as indicated:

1.6 Multihop relay

A transparent RS communicates with the superordinate station and subordinate station(s) using the same carrier frequency. A non-transparent RS may communicate with the superordinate station and subordinate station(s) using the same or different carrier frequencies. The former case is useful if a single frequency network is being deployed. The latter is useful if multiple carrier frequencies are being used in a network. In the case of separate carrier frequencies being used, the non-transparent RS can either be a single radio RS, where time division and carrier frequency switching is used to enable communication with the superordinate station and subordinate station(s), or it can be a dual radio RS, where simultaneous communication with the superordinate station and subordinate station(s) is possible.

Insert new definition 3.125, 3.126, 3.127, 3.128:

3.125 Time-division transmit and receive (TTR) relay: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) is separated in time.

3.126 TTR RS: a non-transparent relay station which performs TTR relaying.

3.127 Simultaneous transmit and receive (STR) relay: a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously.

3.128 STR RS: a non-transparent relay station capable of performing STR relaying.

Change part of the subclause 6.3.9.18.1 (line 47-50, page 106) as indicated:

6.3.9.18.1 Parameter configuration

A non-transparent RS should maintain its synchronization by listening to the R-amble transmitted by its superordinate station (see 6.3.2.3.65). A non-transparent STR dual radio RS may alternatively listen to the frame start preamble of its superordinate station to maintain synchronization. A transparent RS should maintain its synchronization by listening to the preamble transmission from its superordinate station.

Change part of the subclause 8.4.4.7.2 (line 21-25, page 189) as indicated:

8.4.4.7.2 Frame structure for non-transparent mode

Two approaches for supporting relaying are specified for TTRsingle radio RSs. A TTR single radio RS shall be capable of being configured to support either one of the operations, but shall not be required to support both operations simultaneously. The DL subframe shall include at least one access zone and the UL subframe may include one or more UL access zones and one or more relay zones.

Change the title of Figure 270c and 270d as indicated:

Figure 270c—Example of minimum configuration for a single radio non-transparent TTR relay frame structure2

Figure 270d –Example of minimum configuration for a dual radio non-transparent STR relay frame structure; MR-BS and RS have partitioned the UL subframe in the frequency domain

Change part of the subclause 8.4.4.7.2.2 (line 40-48, page 190) as indicated:

8.4.4.7.2.2 TTRSingle radio relay frame structure

A TTR single radio relay station can either perform relaying on the same carrier frequency or on a separate carrier frequency, depending on its negotiated and configured capabilities, by time dividing communication with the superordinate station and subordinate station(s).

An example of a TTR single radio RS frame structure is shown in Figure 270b for the case of relaying on the same carrier. For the case of relaying on a separate carrier, the frame structure is the same as illustrated in Figure 270b, except that the communication with the subordinate station(s) is performed on a different carrier frequency to that used by the superordinate station.

Change the subclause 8.4.4.7.2.3 as indicated:

8.4.4.7.2.3 STRDual radio relay frame structure

An STR dual radio relay station supports simultaneous communication with subordinate station(s) and the superordinate station by using different radio hardware to communicate with the subordinate stations from that used to communicate with the superordinate station.

An example of an STR dual radio RS frame structure is shown in Figure 270c. This frame structure can support more than two hop relaying by including a transmit DL and a receive UL relay zone on the second carrier, as shown in Figure 270c. If the RS is not supporting subordinate RSs then the DL and UL relay zones shown in Figure 270c will not be present.

The arrangement of signaling shall be the same as that described in 8.4.4.7.2.2 except that it is possible that the RS frame be configured such that the RS is both transmitting and receiving at the same time but on separate carriers.

Figure 270e –Example of minimum configuration for a dual radio non-transparent STR relay frame structure

Change the subcluase 11.8.3.7.22 as indicated:

11.8.3.7.22 MR PHY feature support

This TLV indicates the MR PHY features supported by the RS and the MR-BS.

[NOTE TO EDITOR: Just change the description of bit #5 as indicated]

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 206 | 1 | Bit #5: support STR dual radio RS operation (see 8.4.4.7.2.3) | SBC-REQ SBC-RSP |

If the RS supports transmission and reception to and from subordinate station(s) on a different carrier frequency to that used for transmission and reception to and from the superordinate station, bit #4 shall be set to 1, otherwise it shall be set to 0. Bit #4 can only be set to 1 if bit #0 is set to 1.

If RS supports STR and TTR, bit #5 shall be set to 1. If RS only supports TTR, bit #5 shall be set to 0.

Bit #5 can only be set to 1 if both bits#0 and #4 are set to 1.

Change the subcluase 11.25.1 as indicated:

11.25.1 Generic configuration

This field may be used by an MR-BS to configure one or a group of RSs.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| RS operational mode | 1 | 2 | Bit #0: RS scheduling mode, Bit#0=0, centralized scheduling, Bit #0=1, distributed scheduling Bit #1: RS security mode, Bit #1=0, centralized security, Bit #1=1, distributed security Bit #2:0 = shared BSID with other access stations, 1 = unique BSID Bit #3:Embedded path management Bit #4:Explicit path management Bit #5:Burst-based forwarding Bit #6:Tunnel mode Bit #7:Local CID allocation mode Bit #8: Superordinate RS of an RS group Bit #9: Use a different carrier frequency for subordinate station communication at the RS Bit #10: Operate in STR relaying dual radio mode Bit #11~15: reserved | RS_Config-CMD |
| Preamble index | 2 | 1 | The preamble index assigned to the RS. | RS_Config-CMD |

When Bit#9 is set to 0, the RS shall use the same carrier frequency for transmission and reception to and from both the superordinate and subordinate station(s) and Bit#10 shall be set to 0 to indicate it shall operate in TTR mode.

When Bit#9 is set to 1, the RS shall use a different carrier frequency for transmission and reception to and from the subordinate station(s) to that used to transmit and receive to and from the superordinate station and Bit#10 can be set to either 0 or 1 to indicate that the RS shall operate in either TTR or STR mode respectively.