| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on non-transparent STR relay |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin Institute for Information Industry Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | Modifications required for the STR relay |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Comments on non-transparent STR relay

Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin
Institute for Information Industry

Shiann-Tsong Sheu
National Central University (NCU)

Yih-Guang Jan, Yang-Han Lee
Tamkang University (TKU)

- Introduction

According to definition 3.125, a STR (Simultaneous transmit and receive) relay is a relay mechanism where transmission to subordinate station(s) and reception from the superordinate station, or transmission to the superordinate station and reception from subordinate station(s) are performed simultaneously. However, in D6 the MAC operations of the relay were designed for the TTR relay only. Therefore, In order to clarify the STR relay, this contribution proposes modifications in the frame structure and RS network entry. Additionally, the relay zone is not required for this type of relay except when STR relay and TTR relay coexist in the same MR cell. So, we propose alternative STR relay frame structure to optimize the spectrum efficiency. (see Figure 1)

In order to facilitate the incorporation of this proposal into IEEE 802.16j standard, specific changes to the draft standard P802.16j/D6a are listed below.

Figure 1 Example of minimum configuration for a non-transparent STR relay frame structure

- Spec changes

[Modified the following paragraph in line 5 of page 3 as indicated:]

3.97 DL access zone: A portion of the DL sub-frame in the MR-BS/RS frame used for MR-BS/RS to MS or RS (except transparent TTR RS) transmission. The DL access zone may consist of the entire downlink subframe, depending on the method used to separate the transmissions on the access and relay links.

3.98 UL access zone: A portion of the UL sub-frame in the MR-BS/RS frame used for MS or RS (except TTR RS) to MR-BS/RS transmission. A frame may have no UL access zone, or the UL access zone may consist of the entire uplink subframe, depending on the method used to separate the transmissions on the access and relay links.

[Modified the following paragraph in line 20 of page 200 as indicated:]

- 8.4.4.7.2 Frame structure for non-transparent TTR mode

8.4.4.7.2.1 MR-BS frame structure

[Modified the following paragraph in line 36 of page 202 as indicated:]

8.4.4.7.2.2 TTR rRelay frame structure

[Modified the following paragraph in line 58 of page 202 as indicated:]

A relay zone may be utilized for either transmission, or reception, or idle but the RS shall not be required to support both modes of operation within the same zone.

- 8.4.4.7.2.3 Frame structure for non-transparent STR mode relay frame structure

A STR relay station supports simultaneous communication with subordinate station(s) and the superordinate station. Two approaches for supporting relaying are specified for STR RSs.

The first approach uses the BS frame structure defined in 8.3.4.2 for the MR-BS frame and the RS frame. The MR-BS frame structure is the same as the frame structure defined in 8.3.4.2. The RS frame structure is the same as the frame structure defined in 8.3.4.2 on the second carrier of the RS. The RS shall receive data in the DL subframe and transmit data in the UL subframe on the first carrier as a SS.

The second approach uses frame structure defined in 8.3.4.7.2 for the MR-BS frame and the RS frame, which allows coexisting with TTR RS. An example of a STR RS frame structure which allows coexisting with TTR RS is shown in Figure 237e. This frame structure can support more than two hop relaying by including a transmit DL and a receive UL relay zone on the second carrier, as shown in Figure 237e. If the RS is not supporting subordinate RSs then the DL and UL relay zones shown in Figure 237e will not be present.

[Move Figure 237e to 8.4.4.7.3 as following indicated:]

Figure 237e—Example of minimum configuration for a non-transparent STR relay frame structure coexisting with TTR RS in TDD mode

The arrangement of signaling shall be the same as that described in 8.4.4.2 and 8.4.4.7.2.2 except that it is possible that the RS frame be configured such that the RS is both transmitting and receiving at the same time but on separate carriers.

6.3.9.18 RS operation parameters configuration

6.3.9.18.1 Parameter configuration

[Modified the following paragraph in line 26 of page 112 as indicated:]

The remaining steps prior to changing to the RS operational state are indicated below.

- For a non-transparent TTR RS, the MR-BS shall provides complete R-Link parameter information using the RCD message which is used to locate the relay zone (see Figure 90j). Then, the RS shall decode the R-FCH and R-MAP messages within the relay zone. Once the RS has received at least one R-FCH and R-MAP message and is able to decode a burst in the R-link successfully, the RS will achieve R-link MAC synchronization. An RS MAC remains in synchronization as long as it continues to successfully receive the R-MAP. If the RS does not receive a valid R-MAP message in a period equal to the Lost R-MAP Interval it shall try to establish synchronization with another access station. The processes of acquiring synchronization and maintaining synchronization are illustrated in Figure 90i. Once the RS has re-established synchronization with the original access station or a new access station, the RS shall search for the RCD message broadcasted by the access station (see Figure 90j).

- A transparent RS or STR RS shall receives the R-MAP and or RCD messages from the access station in the access zone, or shall receive in the relay zone if coexist with the TTR RS in order to obtain the R-link parameters (see Figures 90i and 90j).

- A non-transparent TTR RS should maintain its synchronization by listening to the R-amble transmitted by its superordinate station (see 6.3.2.3.60).

- A non-transparent STR RS may alternatively maintain its synchronization by listening to the frame start preamble or the R-amble of its superordinate station to maintain synchronization.

- A transparent RS should maintain its synchronization by listening to the preamble transmission from its superordinate station.

[Modified the following paragraph in line 53 of page 112 as indicated:]

The frame number used by the non-transparent RS with unique BSID shall take into account the RS Frame Offset TLV. If an RS Frame Offset is not provided, the RS shall use the same frame number as its superordinate station, i.e. the RS shall consider the RS Frame Offset is zero.

[Replace “Non-transparent mode” by “Non-transparent TTR mode” in Figure 9088f as following indicated:]

- Figure 9088f—Handling RS_Config-CMD first reception at an RS

[Modified the following paragraph in line 1 of page 51 as indicated:]

- 6.3.2.3.65 RS neighbor station measurement report (RS_NBR-MEAS-REP) message

This message may be transmitted by an RS to the MR-BS or superordinate RS on its primary management CID.

- Table 165f—RS_NBR-MEAS-REP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS_NBR-MEAS-REP_Message_Format(){ | - | - |
| Management Message Type = 75 | - | - |
| N_Frequency | 8 bits | Number of frequency reported on |
| for (n=0, n< N_Frequency){ | - | - |
| Carrier centre frequency | 24 bits | RS should measure and report the signal in the specified carrier centre frequency (kHz) |
| N_Amble_Index | 8 bits | Number of amble reported on |
| Begin PHY Specific Section { | - | - |
| for (i=0, i< N_Amble_Index, i++){ | - | - |
| Amble Type | 1 bit | 0: indicates the preamble index 1: indicates the R-amble index |
| Amble Index | 7 bits | The 7 LSB of the preamble or R-amble index corresponds to the RSSI values in the neighbor list. |
| Report Response TLVs | variable | TLV specific |
| } | - | - |
| } |  |  |
| } | - | - |
| TLV Encoded Information | variable | TLV specific |
| } | - | - |

The Report Response TLV shall include physical CINR or RSSI of the preamble index.

- N_Frequency

- Number of frequency reported on

- N_Amble_Index

Number of amble reported on. The amble is either a preamble or an R-amble.

The Report Response TLV shall include physical CINR or RSSI of the preamble index.

[Modified the following paragraph in line 12 of page 36 as indicated:]

6.3.2.3.24 SBC-RSP (SS and RS basic capability response) message

- Station Information TLV (see 11.8.21)

- MR-cell carrier configurations (see 11.8.3.7.28)

[Insert the following subclause in line 28 of page 267 as indicate:]

11.8.3.7.28 MR-cell carrier configurations

This TLV is used for a STR RS to perform neighbor station measurement report during network entry.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| XXX | variable | Number of carrier (8 bits) for (i=0;i<Number of bands; i++) { Carrier centre frequency in kHz (24 bits) Channel bandwidth in kHz (16 bits) FFT size (2 bits) reserved (6 bits) } | SBC-RSP |

[Modified the following paragraph in line 22 of page 216 as indicated:]

8.4.5.3.10 HARQ, and Sub-MAP and R-MAP Pointer IE

This IE shall only be used by a BS supporting HARQ ofor SUB-DL-MAP for MSs supporting HARQ, or transmitting R-MAP to transparent RS in DL access zone. There shall be at most four HARQ MAP Pointer IEs in the DLMAP. There shall be at most 3 SUB-DL-UL-MAP pointer IEs per frame, as specified in Section 6.3.2.3.55.Table 334 shows the format for the HARQ, and Sub-MAP and R-MAP Pointer IE.

[Modified the following paragraph in line 23 of page 222 as indicated:]

8.4.5.10 R-MAP message

This message shall be used in non-transparent frame structure and may be used in transparent frame structure to signal the resource assignments and other control information. For the non-transparent case, this message shall be sent in the first transmitted DL relay zone. This message If the R-MAP message is presented in the downlink relay zone, it shall immediately follow the R-FCH and shall not be preceded by a MAC header and message type field. The modulation and coding rate for the R-MAP message is indicated in the R-FCH. If the R-MAP message is presented in the downlink access zone for the transparent mode, the placement of R-MAP message within a frame shall be the same as SUB-DL-ULMAP shown in Figure 42. The INC_CID flag in the DL-MAP_IE(s) shall start with INC_CID=0. The message format is shown in Table 458a