| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Frame Configuration in 802.16j Networks |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Peiying Zhu, Gamini Senarath, Hang Zhang, Israfil Bahceci, Derek Yu, Mark Naden Nortel Networks | Voice: +1 613 7658089 pyzhu@nortel.com |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | We clarify the usage of frame configuration TLVs in the RCD message. |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clarifications on Frame Configuration in 802.16j Networks

Peiying Zhu, Gamini Senarath, Hang Zhang, Israfil Bahceci, Derek Yu, Mark Naden

Nortel Networks

- Introduction

In the 16j/D6a draft, the frame configuration TLV is sent with the RCD message to inform the usage of frame structure to the RSs.  In 11.24.5 and 11.24.6, it is mentioned that the frame configuration TLV informs the usage of frame structure to the subordinate relay stations.

However, it is not stated whether this TLV informs an RS’s super-ordinate station frame configuration so that RS can automatically decide its own configuration or using this TLV MR-BS configure./order the RS to have a given frame configuration. Only one place(section 6.3.9.18.1)  in the document this issues is clarified. All the other places it just mentions that this message informs the usage of the frame structure. But does not indicate how.  Section 6.3.9.18.1 states "The Frame Configuration mode TLV may be transmitted in RS_Config-CMD message to indicate which mechanism to be followed for determining the super ordinate station's frame structure in the upcoming frame(s)."

This indicates that the RCD provides an RS the frame configuration of the super-ordinate station of the RS. This may be good for certain usage scenarios such as range extension or wireless-backhauled BS scenarios where RS coverage area is isolated from other coverage areas. However, for overlapping coverage areas interference control would be an issue as shown below.

Since both usage scenarios are useful in practice, we propose to have one bit to indicate whether:

- the RCD is used by an MR-BS to inform/configure the RS’s frame configuration or

- it is used to inform the frame configuration of the super-ordinate station of an RS.

By adhering to one of above interpretations, frame configuration TLVs can be properly interpreted at the RS when RS receives the RCD message.

In addition, for the RS group operation the superordiante station of the RS group should be aware of whether members are transparent RSs or the non-transparent RSs. For this purpose we introduced  Bit#11. This would help the superordinate RS when Map is prepared for its group members to match the frame configuration provided by the MR-BS..

- Spec changes

[Modify the text on Line 1-3, Page 113 as]

The Frame Configuration mode TLV may be transmitted in RS_Config-CMD message to indicate which mechanism to be followed for determining the superordinate station's frame structure in the upcoming frame(s).

[Modify text from Line 3 of Page 283 as follows:]

This field is used by an MR-BS to inform the usage of the frame structure to all RSs (see also Bit #12 of RS operational mode TLV in Subclause 11.25.1).

[Modify text from Line 3 of Page 287 as follows:]

This field is used by an MR-BS to inform the usage of the direct relay zone to all RSs (see also Bit #12 of RS operational mode TLV in Subclause 11.25.1).

[Modify the second row of the table in Subclause 11.25.1]

| RS operational mode | 1 | 2 | Bit #0: RS scheduling mode, Bit#0=0, cen- tralized scheduling, Bit #0=1, distributed scheduling Bit #1: RS security mode, Bit #1=0, central- ized security, Bit #1=1, distributed security Bit #2:0 = shared BSID with other access stations, 1 = unique BSID Bit #3:Embedded path management Bit #4:Explicit path management Bit #5:Reserved Bit #6:Tunnel mode Bit #7:Local CID allocation mode Bit #8: Superordinate RS of an RS group Bit #9: Use a different carrier frequency for subordinate station communication at the RS Bit #10: Operate in STR relaying mode Bit #11: The superordinate station of an RS group serves non-transparent RS members (Bit#11 = 0) or transparent RS members (Bit #11=1). Bit #12: 0 = the MR-BS configures the frame configuration of the RS using the Frame configuration TLV in RCD, 1 = the MR-BS or the super-ordinate station provides the frame configuration of the superordiante station using Frame configuration TLV in RCD Bit #131-15: Reserved | RS_Config-CMD |
| --- | --- | --- | --- | --- |