| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Carification on FDD and H-FDD frame structure for IEEE 802.16j Multihop Relay Networks |  |
| Date Submitted | 2008-09-17 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin Institute for Information Industry Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | Modifications required for the STR relay |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Comments on non-transparent STR relay

Kanchei(Ken) Loa, Yi-Hsueh Tsai, Yung-Ting Lee, Youn-Tai Lee, Hua-Chiang Yin

Institute for Information Industry

Shiann-Tsong Sheu

National Central University (NCU)

Yih-Guang Jan, Yang-Han Lee

Tamkang University (TKU)

- Introduction

The current spec only cover the TDD operation. The FDD operations of the relay could be designed by utilizing the 16j TDD frame structure or the Rev2 FDD frame structure. The disadvantage of applying 16j TDD frame structure to the FDD relay is that the zone-switch gaps consume many resources. This contribution proposes a FDD non-transparent frame structure, which utilizes the Rev2 FDD frame structure, requires zero extra zone-switch gaps. The only limitation is that this type of FDD non-transparent relay supports H-FDD operations on the MS only.

Figure 1—Example of configuration for a non-transparent relay frame structure (case 1)

Figure 2—Example of configuration for a non-transparent relay frame structure (case 2)

- Spec changes

3.97 DL access zone: A portion of the DL sub-frame in the MR-BS/RS frame used for MR-BS/RS to MS or RS (except non-transparent TTR RS in TDD mode) transmission. The DL access zone may consist of the entire downlink subframe, depending on the method used to separate the transmissions on the access and relay links.

3.98 UL access zone: A portion of the UL sub-frame in the MR-BS/RS frame used for MS or RS (except non-transparent TTR RS in TDD mode) to MR-BS/RS transmission. A frame may have no UL access zone, or the UL access zone may consist of the entire uplink subframe, depending on the method used to separate the transmissions on the 8.4.4.7 Frame structure of MR-BS and RS

8.4.4.1 FDD frame structure

[This is a place holder]

8.4.4.1.5 Frame Structure for transparent mode

[Insert the text as following indicated:]

8.4.4.1.6 Frame Structure for non-transparent TTR mode

The MR-BS frame structure is the same as the frame structure defined in 8.4.4.1. The R-amble may appear in the DL Subframe 2 for synchronization. The non-transparent TTR RS frame structure is the same as the frame structure defined in 8.4.4.1 for the DL subframe 1 and uplink subframe UL1. It’s subordinate SSs shall listen to RS in the DL Subframe 1 and transmit to the RS in the uplink subframe UL1. The RS shall listen to RS in the DL Subframe 2 and transmit to from the RS in the uplink subframe UL2 as a SS.

8.4.4.2 TDD frame structure

[Modified the subclause titles as following indicated:]

8.4.4.72.1 Frame Structure for transparent mode

8.4.4.72.1.1 MR-BS frame structure

8.4.4.72.1.2 Relay frame structure

8.4.4.72.2 Frame structure for non-transparent STR mode

8.4.4.72.2.1 MR-BS frame structure

8.4.4.72.2.2 Relay frame structure

8.4.4.72.3 Frame structure for non-transparent STR mode