| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | FDD and H-FDD frame structure for TTR RS and MR-BS |  |
| Date Submitted | 2008-11-11 |  |
| Source(s) | Gamini Senarath, Hang Zhang, Israfil Bahceci, Peiying Zhu, Derek Yu, Mark Naden Nortel Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee, Institute for Information Industry (III) Yung-Ting Lee, Hua-Chiang Yin Coiler Shiann-Tsong Sheu National Central University (NCU) Yih-Guang Jan, Yang-Han Lee Tamkang University (TKU) | Voice: +613 763 5287 bahcecis@nortel.com Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE-SA Sponsor Ballot: P802.16j/D6a |  |
| Abstract | Modifications required for the FDD mode |  |
| Purpose | Text proposal for 802.16j Draft Document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- FDD and H-FDD frame structure for TTR RS and MR-BS

Gamini Senarath, Israfil Bahceci, Hang Zhang, Peiying Zhu, Derek Yu, Naden Mark
Kanchei(Ken) Loa, Yi-Hsueh Tsai, Youn-Tai Lee
Yung-Ting Lee, Hua-Chiang Yin
Shiann-Tsong Sheu, Yih-Guang Jan, Yang-Han Lee

Nortel, III, Coiler, NCU, TKU

### Introduction

IEEE 802.16j/D7 supports only TDD frame structure, and therefore, only the TDD mode of operation. However, the IEEE 802.16Rev2 supports FDD operation, which includes half-duplex FDD (H-FDD) and full duplex FDD capable mobiles. The FDD and H-FDD operation has also been included as additional features in WiMAX Profile Release 1.5. Therefore, relay support for FDD operation is very desirable such that the coverage and other performance enhancement of relay systems can be extended to the FDD based 16e systems as well.

We propose FDD and H-FDD mode of operation for TTR RS.

### TTR RS and MR-BS Operation in FDD and H-FDD mode

For FDD operation, DL and UL frequencies are different from each other. An RS can also be designed to operate in FDD mode by using the same carrier for its DL (UL) transmission as its superordinate station does for its DL (UL). Figure 1 presents the DL/UL transmission in FDD mode.

f1

f2

f1

f2

UL

DL

BS

RS

MS

Figure 1 DL and UL transmission in FDD mode with half-duplex FDD relay.
Each hop employs the same carrier pairs in FDD.

The H-FDD frame structure in 802.16Rev2 can be extended to IEEE802.16j by reusing 16e FDD frame structure.

- The RS may obtain the UL carrier from the UCD during network entry. The same carrier frequencies used at the first hop can be used at other hops. In FDD mode, a TLV to configure the UL carrier for RS to use in communicating with its subordinate stations can be signaled.

- The RSs can operate in full duplex mode, e.g., may be allocated resources at all H-FDD available resources using HDD Paired Allocation IE.

Alternatively, the H-FDD frame structure in 802.16Rev2 can be extended to IEEE802.16j with minor modifications by reusing 16j TDD frame structure.

- Frame configuration: R-link channel description (RCD) MAC message can be employed to signal the access and relay zones. We have the following options:

  - The access zones are contiguous and fall within one of the subframes of H-FDD frame, while the relay zones are contiguous and fall within the other subframe. The FCH/MAP and R-FCH/R-MAP can be transmitted in the first tx access zone and relay zone, respectively. In this case, all the half-duplex MSs/SSs are set as Group 1 users, and all the RSs are set as Group 2 users, (or vice versa) in FDD mode.

  - Each subframe may contain at least one access zone and zero or more relay zones.

- The RS may obtain the UL carrier from the UCD during network entry. In FDD mode, an additional TLV to configure the UL carrier for RS to use in communicating with its subordinate stations can be signaled.

- The RSs can operate in full duplex mode, e.g., may be allocated resources at all H-FDD available resources using HDD Paired Allocation IE.

### Examples for TTR RS and MR-BS using 16e (REV2) FDD frame structure

Figure 1—Example of configuration for a non-transparent relay frame structure

- Spec changes

[Insert the subclause in line 7 of page 209 as following indicated:]

8.4.4.10 FDD frame structure of MR-BS and RS

8.4.4.10.2 Frame Structure for non-transparent TTR

The TTR RS may communicate with MR-BS in either H-FDD or FDD mode.

If the MR-BS and TTR RS frame structures in Group-1 and Group-2, if present, is the same as the BS frame structure in Group-1 and Group-2 defined in 8.4.4.1, the relay link between the RS and its superordinate station shall operate in Group-2 H-FDD only, where the RS shall listen to DL Subframe 2 and transmit in uplink subframe UL2. The access link between the RS and its subordinate SSs shall operate in Group-1 H-FDD mode only. The common SYNC symbol defined in 8.4.6.1.1.2 may appear in the Group-2 DL Subframe of the superordinate station for the RS synchronization.

If the MR-BS and TTR RS frame structures in both Group-1 and Group-2, if present, are the same as the MR-BS and TTR RS frame structures defined in 8.4.4.7.2 respectively, the relay link between the RS and its superordinate station shall operate in either H-FDD or FDD. Group switching of the RS shall be inhibited. The access link between the RS and its subordinate SSs shall operate in either H-FDD or FDD. The Group-2 DL Subframe of the superordinate station shall exclude the preamble.